| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F359 |
| --- | --- |

| Title: | Deblocking filter modification for constrained intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee, Sung-Chang Lim, Hui Yong Kim, Jin Soo Choi. Realistic Media Research Team, ETRI 161 Gajeong-dong, Yuseong-gu Daejeon, South Korea | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr sclim@etri.re.kr hykim5@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

This contribution presents deblocking filter modification for constrained intra prediction. Constrained intra control that intra blocks in one frame are not affected by errors of another frame. To this end, constrained intra only allows neighboring samples from intra blocks for intra prediction. However, if an inter block is not correctly reconstructed, the errors of the inter block would be propagated to neighboring intra blocks by HM3.0 deblocking process. The proposal prevents the problem by disabling deblocking filtering on pixels inside intra block when its neighboring block is inter block. It is reported that the BD-rate increase due to the proposed method is negligible: 0.1% and 0.1% in RA-HE and RA-LC, respectively, and 0.0% and 0.0% in LD-HE and LD-LC, respectively.

# Introduction

Constrained Intra Prediction (CIP) is a tool to control that intra blocks in one frame are not affected by a corruption or errors of another frame. As a method to solve the problem, CIP only allows reconstructed pixels from intra blocks in the same slice for prediction of the current block. However, deblocking process of HM3.0 [1] performs filtering on intra or inter blocks. This brings error propagation problem into the intra blocks.

# Deblocking filter modification for constrained intra prediction

The deblocking process in HM3.0 does not consider constrained intra prediction. This could raise error propagation from inter block to intra block as illustrated in Figure 1 (a). So, we propose modification of deblocking filter for CIP by disabling deblocking filter on pixels of intra block when its neighboring block is inter block as illustrated in Figure 1 (b).

- HM3                      (b) Proposal

Figure 1 : An issue in HM3.0 and a proposal

Deblocking filtering could have four cases according to the prediction mode of current block and its neighboring block as represented in Figure 2. We propose (b) and (c) in Figure 2 to prevent error propagation when constrained_intra_pred_flag is equal to 1.

Figure 2 : Deblocking filter modification for the proposal

# Harmonization with PCM deblocking

HM3.2 has pcm_loop_filter_disable_flag for disabling deblocking filter on pixels of I_PCM blocks. Therefore we have harmonized CIP deblocking process with PCM deblocking process as described in Figure 3.

If constrained_intra_pred_flag is equal to 1, deblocking filter on pixels of intra block is disabled when a block is intra and the other block is inter. When both 'P' and 'Q' blocks are all intra or inter, decision of disabling is affected by pcm_loop_filter_disable_flag. In details, if pcm_loop_filter_disable_flag is equal to 1, deblocking filter on pixels of I_PCM block is disabled.

If constrained_intra_pred_flag is equal to 0 and pcm_loop_filter_disable_flag is equal to 1, deblocking filter on pixels of the I_PCM block is disabled.

Figure 3 : A flowchart of decision for disabling deblocking filter

# Experimental results

The proposed method was implemented based on HM3.2rc1. The experiments are conducted on the platform of Intel Core i7 CPU 990X @ 3.46GHz with 24GB RAM.

Table 1 represents the performance of the proposal when constrained_intra_pred_flag is equal to 1 for the both anchor and proposal. The proposal is cross-checked by Panasonic (JCTVC-F651).

Table 1 : Performance of the proposal

# Conclusion

The proposed modification of deblocking filter for constrained intra prediction protects intra blocks from error propagation. The BD-rate increase due to the proposed method is negligible, and the impacts on subjective quality is unnoticeable. We suggest the proposal to be adopted into the HM.

# Reference

- JCT-VC, “High Efficiency Video Coding (HEVC) text specification Working Draft 3”, JCTVC-E603, JCT-VC Meeting, Geneva, Mar 2011.

# Patent rights declaration(s)

ETRI (Electronics and Telecommunications Research Institute) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex - Decoding process (Based on WD3_d8)

- 8.6.1.4.5		Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	a variable dSam containing a decision,

–	a variable tC.

Output of this process is:

–	number of filtered samples nD,

–	filtered sample values, pi’ and qi’ with i = 0..nD – 1,

Depending on dSam, the following applies:

–	When the variable dSam is equal to 1, the following strong filtering applies while nD is set equal to 3:

–  If constrained_intra_pred_flag is equal to 1, the sample, pi with i = 0..nD-1, is in a coding unit coded with intra prediction mode, and the sample, qi with i = 0..nD-1, is not in a coding unit coded with intra prediction mode, the following applies:

q0’ = Clip1Y( ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-x)

q1’ = Clip1Y( ( p0 + q0 + q1 + q2 + 2 ) >> 2 )		(8-x)

q2’ = Clip1Y( ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )		(8-x)

–  Otherwise, if constrained_intra_pred_flag is equal to 1, the sample, pi with i = 0..nD-1, is not in a coding unit coded with intra prediction mode, and the sample, qi with i = 0..nD-1, is in a coding unit coded with intra prediction mode, the following applies:

p0’ = Clip1Y( ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-x)

p1’ = Clip1Y( ( p2 + p1 + p0 + q0 + 2 ) >> 2 )		(8-x)

p2’ = Clip1Y( ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )		(8-x)

–  Otherwise, the following applies:

p0’ = Clip1Y( ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-1)

p1’ = Clip1Y( ( p2 + p1 + p0 + q0 + 2 ) >> 2 )		(8-2)

p2’ = Clip1Y( ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )		(8-3)

q0’ = Clip1Y( ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-4)

q1’ = Clip1Y( ( p0 + q0 + q1 + q2 + 2 ) >> 2 )		(8-5)

q2’ = Clip1Y( ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )		(8-6)

–  Otherwise, the following weak filtering applies while nD is set equal to 2:

 = Clip3( -tC, tC, ( 13*( q0 – p0 ) + 4*( q1 – p1 ) - 5*( q2 – p0 ) + 16 ) >> 5 )	(8-7)

–  if constrained_intra_pred_flag is equal to 1, the sample, pi with i = 0..nD-1, is in a coding unit coded with intra prediction mode, and the sample, qi with i = 0..nD-1, is not in a coding unit coded with intra prediction mode, the following applies:

q0’ = Clip1Y( q0 -  )		(8-x)

q1’ = Clip1Y( q1 – /2 )		(8-x)

–  Otherwise, if constrained_intra_pred_flag is equal to 1, the sample, pi with i = 0..nD-1, is not in a coding unit coded with intra prediction mode, and the sample, qi with i = 0..nD-1, is in a coding unit coded with intra prediction mode, the following applies:

p0’ = Clip1Y( p0 +  )		(8-x)

p1’ = Clip1Y( p1 + /2 )		(8-x)

–  Otherwise, the following applies:

p0’ = Clip1Y( p0 +  )		(8-8)

q0’ = Clip1Y( q0 -  )		(8-9)

p1’ = Clip1Y( p1 + /2 )		(8-10)

q1’ = Clip1Y( q1 – /2 )		(8-11)

Each of the filtered sample values, pi’ with i = 0..nD-1, is substituted by the corresponding input sample value pi if all of the following conditions are true.

–	pi is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similary, each of the filtered sample values, qi’ with i = 0..nD-1, is substituted by the corresponding input sample value qi if all of the following conditions are true.

–	qi is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

[Ed. (WJ): for PCM case, deblocking filter applies first and the filtered pixels are restored. Rather than this, it’s better to skip the filtering itself for PCM samples since first filtering is actually not needed.]

Table 8-1 – Derivation of threshold variables β and tC from input Q

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |  |
| β | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | 64 | 64 | 64 | 64 |  |
| tC | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 |  |

- 8.6.1.4.6		Filtering process for a chroma sample

[Ed: (WJ) no filtering when bS is equal or less than 2]

Inputs of this process are:

–	sample values, pi and qi with i = 0..1,

–	a variable bS specifying the boundary filtering strength.

–	a variable tC.

Output of this process is:

–	The filtered sample values, p0’ and q0’.

When the variable bS is greater than 2, the filtered sample values p0’ and q0’ are derived by

 = Clip3( -tC, tC, ( ( ( ( q0 – p0 ) << 2 ) + p1 – q1 + 4 ) >> 3 ) )	(8-12)

–  if constrained_intra_pred_flag is equal to 1, the sample, p0, is in a coding unit coded with intra prediction mode, and the sample, q0, is not in a coding unit coded with intra prediction mode, the following applies:

q0’ = Clip1C( q0 -  )		(8-x)

–  Otherwise, if constrained_intra_pred_flag is equal to 1, the sample, p0, is not in a coding unit coded with intra prediction mode, and the sample, q0, is in a coding unit coded with intra prediction mode, the following applies:

p0’ = Clip1C( p0 +  )		(8-x)

–  Otherwise, the following applies:

p0’ = Clip1C( p0 +  )		(8-13)

q0’ = Clip1C( q0 -  )		(8-14)

The filtered sample value, p0’ is substituted by the corresponding input sample value p0 if all of the following conditions are true.

–	p0 is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similary, the filtered sample value, q0’ is substituted by the corresponding input sample value q0 if all of the following conditions are true.

–	q0 is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

[Ed. (WJ): for PCM case, deblocking filter applies first and the filtered pixels are restored. Rather than this, it’s better to skip the filtering itself for PCM samples since first filtering is actually not needed.]