| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F363-r1 |
| --- | --- |

| Title: | CE10: FastVDO-Samsung Core Transform Proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Wei Dai, Madhu Krihnan, Pankaj Topiwala Ilkoo Kim, Samsung 5840 Banneker Rd., #230 Columbia, MD 21044 | Tel: Email: | +1-410-730-6922 pankajtva@yahoo.com |
| Source: | FastVDO, Samsung |  |  |

_____________________________

# Abstract

This submission is an update of E253, a joint proposal from FastVDO and Samsung, for the core transforms for the HeVC standard. This is the third iteration of this core transform proposal. This version employs the 4-pt and 8-pt transforms from the Cisco/TI design, currently incorporated in the HM 3.0 software, and 16-pt and 32-pt transforms of FastVDO and Samsung design, which are variants of the designs from E253. It is reported that the compression efficiency of this design is comparable to that of the HM 3.0 transforms, while evidence is provided to indicate that the computational complexity of the larger transforms of size 16 and 32, which dominate the complexity burden, is significantly lower than those of the HM 3.0 design. Thus, it is claimed that this transform architecture provides a favorable tradeoff between complexity and performance.

# Introduction

The core transforms are a fundamental part of the draft HeVC design. At the fourth meeting in Daegu, a Core Experiment was set up (CE10) to explore the design of core transforms, study the complexity and performance of candidate architectures, and develop solutions that provide a good tradeoff between complexity and performance.

At the fifth meeting in Geneva, it was decided for purposes of testing to incorporate into the HM 3.0 software architecture the design proposed by Cisco/TI in E243, which provides for excellent coding from an objective, mean-square error point of view. However, from the perspective of computational complexity as indicated by the number of arithmetical operations needed for its computation, that design seems to be of relatively high complexity. This is due to the fact that the transforms of all sizes in that design are defined by matrix multiplication; while such a design can be partially factorized in a flow-graph (“partial butterfly”), it still has the complexity burden of matrix multiply (e.g., O(N^2) for a 1-D transform). This is in contrast to fully factorized designs, which are modern descendents of the fast Fourier transform design; such algorithms generically have O(NLogN) complexity.  These considerations in fact apply to a number of candidate architectures, which approximate the discrete cosine transform (DCT), including the series proposed these past meetings by FastVDO and Samsung.

With the four different sizes of transforms: 4, 8, 16, and 32, it is obvious that the larger transforms dominate the complexity burden, and in particular, it is sufficient to compare and contrast just the 32-pt transform. Furthermore, among the arithmetical operations of multiplies, adds, and shifts, it is generally accepted that multiplies are a higher burden for generic architectures than adds and shifts. Since transforms the size as large as 32 points pose a significant overall complexity burden in the codec, reducing the complexity burden of computing such transforms is an important design goal. Notionally, a 32-pt matrix multiply based transform would need approximately 65K multiplications for a 2-D implementation; a partial butterfly factorization can bring that number down to approximately 23K mults for every 32-size block, which is still a very large number [E243]. It was shown at the last meeting [E253] that the 32-pt transform of our design required no multiplies at all, and only adds/shifts, and even these were quite sparse. More specific details are provided herein and in the AhG report.

This arithmetical complexity metric is also corroborated by evidence recently obtained in hardware integration simulations by producing Run-Time Libraries (RTL), as performed by Samsung, which measure the number of gates needed to implement various proposed designs in Field Programmable Gate Arrays (FPGAs). A metric based on estimating gate count is a widely accepted measure of hardware complexity. That is because a design, once implemented in an FPGA, can then be automatically mapped to an Application-Specific Integrated Circuit (ASIC), so an FPGA mapping is a critical step towards hardware realization. An automated optimization approach was utilized for the FPGA mapping by Samsung, providing impartial results.  The net result of this analysis is that fully factorized designs are significantly lower in complexity than matrix multiply designs (this work will be reported independently by Samsung at the Torino meeting). It is therefore desirable to develop a transform design that can both achieve high compression efficiency approaching that of the HM 3.0 transform design, but which entails a lower complexity burden. This proposal develops such a design.

First, as the lower-order 4 and 8pt are of relatively modest complexity, it was elected to reuse the HM 3.0 transforms, which already demonstrated excellent performance. However, our vision was that the larger transforms of size 16 and 32, which carry the weight of the complexity burden, needed to be of a different, more factorizable design. It was elected to continue to use a Loeffler-type factorization for both sizes, just as we have in the previous two meetings. The 16-pt design was led by FastVDO, while the 32-pt design was led by Samsung.  Details of the 16-pt and 32-pt transforms are provided in an accompanying slide deck.

# RD Performance

The RD performance (Normal Qp) is provided in an accompanying excel spreadsheet. A summary sheet from that spreadsheet is provided below.  The Low and High Qp data will be provided when available.

It can be seen that this transform design is comparable to the HM 3.0 design in compression efficiency. (Several tests with the Nebuta sequence, Class A, were not finished as of this writing, and will be updated when available.  Also, as our software was written in a pedantic, highly unoptimized manner for debugging purposes, it resulted in high encoder times which are not indicative of what an optimized architecture could produce.)

# Arithmetic Complexity Analysis

The 4-pt and 8-pt transforms are reused from the HM 3.0, and their complexity is as reported in E243. The 16-pt transform has identical transform to the 16-pt as presented in E253.  (These will be quoted in the next revision.) In fact, it is sufficient to focus attention on the 32-pt transform, as it dominates the complexity.  Our 32-pt transform is borrowed from F251, so we quote data from Table 2 from F251. That table provides operations count for a 1D transform. For a full 2D transform, one must multiply by each value by 2N, for a size N transform.

In particular, for a full 2D 32-pt transform, we have

Table 2. Operations for full 2D 32-pt transforms.

| Size | Proposed scheme |  |  | Partial butterfly |  |
| --- | --- | --- | --- | --- | --- |
|  | “*” | “+” | “<<” | “*” | “+” |
| 32 | 5568 | 14144 | 512 | 22016 | 23808 |

# Patent rights declaration(s)

FastVDO and Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).