| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F368 |
| --- | --- |

| Title: | CE4 Subtest 3.3.1.a: Cross-check MediaTek proposal JCTVC-E091 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross check report |  |  |
| Author(s) or Contact(s): | Xiaolin Shen, Xingguo Zhu, Binbin Yu, (ZJU) | Email: babylion@zju.edu.cn 05xxgcsgx@zju.edu.cn zjuybb@zju.edu.cn |  |
| Source: | Zhejiang University, China |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the performance of JCTVC-E091, Adaptive De-Quantization Offset (AQO) proposed by Media Tek [1]. It is the test subset 3.3.1. a in CE4 [2]. The verification task has been done successfully, and the results match those provided by Media Tek.

# Crosscheck task description

As described in [2], three scenarios are verified:

- Adaptive de-quantization offset (AQO) only

- De-quantization offset based lambda refinement only

- Combination of AQO and lambda refinement

Media Tek provided the source code which is based on HM3.0 + bugfix rev 850. The verification is carried by setting

- --AQO=1 --AQOLambda=0 for AQO only

- --AQO=1 --AQOLambda=2 for lambda refinement only

- --AQO=1 --AQOLambda=1 for AQO + lambda

# Verification Results

The BD-rates from our experimental results match those provided by Media Tek exactly. For details, refer to the excel sheets. The timing results we get are not reliable, since cross-checking is performed on different workstations and by multiple processes.

Table 1 Average BDPSNR results of 3 tasks

|  | Aver-y | Aver-U | aver-v |
| --- | --- | --- | --- |
| AQO only | -0.6 | -0.9 | -1.3 |
| Lambda only | -0.2 | -1.1 | -1.3 |
| AQO+Lambda | -1.1 | -2.0 | -2.4 |

# Conclusion

The results presented by Media Tek are confirmed.

# Reference

[1] Xiang Li, Xun Guo, Shawmin Lei, “Adaptive De-Quantization Offset”, JCTVC-E091, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March, 2011.

[2] K.Sato, etc. “Description of Core Experiment 4: Quantization”, JCTVC-E704, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March, 2011.