| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F372 |
| --- | --- |

| Title: | Bi-derivative merge candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hideki Takehara Shigeru Fukushima Hiroya Nakamura JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-5729 takehara.hideki@jk-holdings.com fukushima.shigeru@jk-holdings.com nakamura.hiroya@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

In merge mode, motion information used for motion compensation is indicated only by signaling the index of merge candidates. However, the current merge candidates in the HM3.0 are insufficient especially in case that some candidates are not available.

In this proposal, additional merge candidates are created by using motion information of current merge candidates without additional access to other blocks.

The simulation results report that the proposed technique provides 0.2% BD-rate gain for random access settings and 0.7% gain for low delay settings with 3% encoder runtime and 1% decoder runtime increase.

# Introduction

Merge is one of the techniques to reduce motion information. In merge mode, motion information used for motion compensation is indicated only by signaling the index of merge candidates.

However, the current merge candidates in the HM3.0 [1] are insufficient especially in case that some candidates are not available.

This proposal aims to improve coding efficiency by creating additional merge candidates using motion information of current merge candidates.

# Algorithm Description

## Proposed Algorithm

An additional merge candidate that is named bi-derivative (BD) merge candidate is introduced. The BD merge candidate is created if at least one of the current merge candidates is not available. The index of the BD merge candidate is assigned to the last position in the merge candidates list.

Figure 1 illustrates the flowchart of merge candidate derivation process in this proposed algorithm.

Figure 1 Flowchart of merge candidate derivation process

If the number of current merge candidates is not the maximum number of merge candidates (5 in HM3.0), the derivation process of BD merge candidate is invoked.

The BD merge candidate is derived from independently using L0 and L1 motion information of current merge candidates. The LJ motion information for BD merge candidate is derived from motion information of one merge candidate in the LJ (where J is 0 or 1). The LK motion information for BD merge candidate is derived from motion information of one merge candidate in the LK (where K is 0 or 1) excluding the merge candidate that is selected for LJ.

BD merge candidate can be created without additional access to other blocks.

Figure 2 illustrates the derivation process of BD merge candidate.

Figure 2 derivation of BD merge candidate

## Implementation

### Software

This proposed technique is implemented into HM3.0 revision828 software (CE1/CE9 anchor).

### Syntax structure

The syntax of this proposal is not changed from HM3.0.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 and Table 3 present the simulation results for random access and low delay, respectively.

Table 2 Simulation results of proposed algorithm for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2 | -0.3 | -0.1 | -0.3 | -0.2 | -0.4 |
| Class B | -0.2 | -0.2 | -0.3 | -0.2 | -0.2 | -0.2 |
| Class C | -0.2 | -0.2 | -0.2 | -0.2 | -0.3 | -0.3 |
| Class D | -0.2 | -0.4 | -0.2 | -0.2 | -0.4 | -0.4 |
| Class E |  |  |  |  |  |  |
| Overall | -0.2 | -0.3 | -0.2 | -0.2 | -0.2 | -0.3 |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 3 Simulation results of proposed algorithm for low delay

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | -1.0 | -1.3 | -1.1 | -1.2 | -1.0 |
| Class C | -0.6 | -0.8 | -1.0 | -0.7 | -0.8 | -1.0 |
| Class D | -0.5 | -0.7 | -0.2 | -0.5 | -0.5 | -0.2 |
| Class E | -0.5 | -0.4 | -0.7 | -1.1 | -1.2 | -1.1 |
| Overall | -0.5 | -0.8 | -0.8 | -0.8 | -0.9 | -0.8 |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

The simulation results report that the proposed technique provides 0.2% BD-rate [3] gain for random access settings and 0.7% gain for low delay settings with 3% encoder runtime and 1% decoder runtime increase. (See the attached excel sheet)

# Conclusions

This contribution reports the proposed technique can bring substantial coding gain.

Therefore, we suggest that the proposed technique be further investigated in CE.

# Future Work

The derivation process of BD merge candidate is introduced in this proposal.

There are some future works.

- Improvement of the derivation process for creating more efficient BD merge candidate.

- Simplification of the derivation process for BD merge candidate.

- Application of multiple BD merge candidates.

- Evaluate of BD merge candidates in robustness condition.

# References

[1] “WD3: Working Draft 3 of High-Efficiency Video Coding”,

JCTVC-E603, Geneva, March 2011

[2] Frank Bossen “Common test conditions and software reference configurations”,

JCTVC-E700, Geneva, March 2011

[3] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).