| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F373 |
| --- | --- |

| Title: | Merge based mvd transmission |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Motoharu Ueda Kazumi Arakage Satoru Sakazume JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@jk-holdings.com ueda.motoharu@jk-holdings.com someya.kazumi@jk-holdings.com sakazume.satoru@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

In the HM3.0, merge based skip mode is adopted since skip mode has same characteristic with merge that motion vector difference (mvd) is not transmitted. In the case of transmitting mvd, AMVP (Advanced Motion Vector Prediction) is used to reduce motion information.

In this proposal, it is described to improve coding efficiency by extending merge motion information derivation process based on merge mode to mvd transmission case.

The simulation results report that the proposed technique provides 0.6% BD-rate gain for random access settings and 1.3% gain for low delay settings with 14% encoder runtime and 1% decoder runtime increase.

# Introduction

Merge is one of the techniques to reduce motion information. In the HM3.0 [1], merge based skip mode is adopted since skip mode has same characteristic with merge that motion vector difference (mvd) is not transmitted [2]. In the case of transmitting mvd, AMVP (Advanced Motion Vector Prediction) is used to reduce motion information.

However, motion information derivation process based on merge mode is useful for mvd transmission case since redundant motion information is not transmitted.

This proposal aims to improve coding efficiency by extending merge motion information derivation process based on merge mode to mvd transmission case.

# Algorithm Description

## Proposed Algorithm

### Normative algorithm

Motion information derived from merge mode is used not only for no mvd transmission case but also for mvd transmission case. For the merge based mvd transmission, a new mode that is named merge mvd mode is introduced.

Table 1 presents transmitted motion information by each mode in the proposed algorithm.

Table 1 Transmitted motion information

|  | Inter Pred | Merge Index | Ref Index | AMVP Index | Mvd |
| --- | --- | --- | --- | --- | --- |
| Merge mode | - | ○ (1) | - | - | - |
| MergeMvd mode | ○ | ○ (1) | - | - | ○ (1) |
| AMVP mode | ○ | - | ○ (1 or 2) | ○ (1 or 2) | ○ (1 or 2) |

The AMVP mode means mvd transmission mode using AMVP as Pred_LC mode or Pred_BI mode in current HEVC. The merge mode and the AMVP mode are not changed from HM3.0. The merge mvd mode is newly introduced. In the merge mvd mode, one mvd is transmitted in addition to the merge index.

Table 2 presents inferred motion information in the merge mvd mode for each merge candidate type.

Table 2 Inferred motion information in the merge mvd mode

| Merge candidate type | Ref Pic | Mvd Ref Pic |
| --- | --- | --- |
| Spatial Uni-pred | Same as Merge mode / Ref0 | Not merged Pic |
| Spatial Bi-pred | Same as Merge mode | Distance based |
| Temporal Bi-pred | Same as Merge mode | Distance based |

Figure 1 illustrates the selection of the reference picture to add mvd for each merge candidate type.

Figure 1 Selection of the reference picture to add mvd

If merge candidate type which is indicated by merge index is uni-prediction, the reference index of the other list which is not selected by the merge index is set to 0 for bi-prediction. The reference picture on which transmitted mvd is added to mvp is the picture indicated by the reference index in the other list.

Otherwise (merge candidate type is bi-prediction), same reference picture as for the merge mode is used for the merge mvd mode. The reference picture on which transmitted mvd is added to mvp is selected based on the distance to the current picture.

The decoding process for the merge mvd mode is shared with the merge mode. The additional process for the merge mvd mode is as follows.

- Select the reference picture to add mvd

- (In case that merge candidate type is uni-prediction,) calculate mvp of the other list for bi-prediction

- Calculate motion vector by adding the mvd to mvp for the selected reference picture

### Non-normative algorithm

For the merge mvd mode, additional motion estimation is executed.

Table 3 presents the specification of the motion estimation for the merge mvd mode.

Table 3 Specification of the motion estimation for merge mvd mode

| Initial search position | mv calculated for AMVP mode and mvd_ls = (0,0) |
| --- | --- |
| Full pixel search range | ±2 |
| Full pixel search algorithm | Full search |
| Sub pixel search algorithm | Same as for AMVP mode |
| Evaluation method | Bi-pred ME (mv in the other list is fixed) |

For no additional motion estimation case, the motion vector calculated for AMVP mode is applied as the only candidate for transmitting mvd in merge mvd mode.

## Implementation

### Software

This proposed technique is implemented into HM3.0 revision828 software (CE1/CE9 anchor).

### Syntax structure

The prediction unit syntax of this proposal is presented in Table 4.

Table 4 Prediction unit syntax of the proposed technique

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag ) |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else if( inter_pred_flag[ x0 ][ y0 ] = = Pred_L0 ) {/* Pred_L0*/ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_BI*/ |  |
| merge_mvd_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_mvd_flag[ x0 ][ y0 ] ){ |  |
| if( NumMergeCand > 1) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_ls[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_ls[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| } |  |
| else |  |
| { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

### Semantics description

Semantics description for new syntax element is as follows.

merge_mvd_flag[ x0 ][ y0 ] has the same semantics as merge_flag.

When merge_mvd_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mvd_ls[ x0 ][ y0 ][ compIdx ], has the same semantics as mvd_l0, with l0 and list 0 replaced by ls and list selection, respectively. The list s is selected by motion information derivation process of merge mvd mode.

# Experiments

## Test conditions

The common test conditions [3] are used.

## Experimental environment

The experimental environment used is presented in Table 5 and Table 6.

Table 5 Experimental environment for encoding

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Table 6 Experimental environment for decoding

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 7 and Table 8 present the simulation results with additional motion estimation for random access and low delay, respectively.

Table 9 and Table 10 present the simulation results without additional motion estimation for random access and low delay, respectively.

Table 7 Simulation results (with additional ME) for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | -0.5 | -0.5 | -0.8 | 0.2 | -0.3 |
| Class B | -0.7 | -0.6 | -0.7 | -0.6 | -0.6 | -0.6 |
| Class C | -0.6 | -0.5 | -0.5 | -0.4 | -0.5 | -0.4 |
| Class D | -0.7 | -0.5 | -0.5 | -0.5 | -0.6 | -0.5 |
| Class E |  |  |  |  |  |  |
| Overall | -0.6 | -0.5 | -0.6 | -0.6 | -0.4 | -0.5 |
| Enc Time[%] | 114% |  |  | 117% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 8 Simulation results (with additional ME) for low delay

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.1 | -1.7 | -1.7 | -1.7 | -1.9 | -1.7 |
| Class C | -1.2 | -1.2 | -1.3 | -1.2 | -1.0 | -1.1 |
| Class D | -1.1 | -1.0 | -0.5 | -0.9 | -0.8 | -1.1 |
| Class E | -1.3 | -1.5 | -2.2 | -1.9 | -3.0 | -2.0 |
| Overall | -1.2 | -1.4 | -1.4 | -1.4 | -1.6 | -1.5 |
| Enc Time[%] | 112% |  |  | 114% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 9 Simulation results (without additional ME) for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2 | -0.2 | -0.2 | -0.2 | -0.1 | -0.3 |
| Class B | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 | -0.1 |
| Class C | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 | 0.0 |
| Class D | -0.2 | -0.2 | -0.2 | -0.1 | -0.2 | -0.2 |
| Class E |  |  |  |  |  |  |
| Overall | -0.2 | -0.2 | -0.2 | -0.1 | -0.2 | -0.2 |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 10 Simulation results (without additional ME) for low delay

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.6 | -1.0 | -0.6 | -0.6 | -0.6 |
| Class C | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
| Class D | -0.4 | -0.5 | 0.1 | -0.3 | -0.3 | -0.4 |
| Class E | -0.4 | -0.3 | -0.6 | -0.9 | -0.6 | -0.4 |
| Overall | -0.4 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

The simulation results report that the proposed technique with additional motion estimation provides 0.6% BD-rate [4] gain for random access settings and 1.3% gain for low delay settings with 14% encoder runtime and 1% decoder runtime increase. (See the attached excel sheet)

The simulation results report that the proposed technique without additional motion estimation provides 0.1-0.2% BD-rate gain for random access settings and 0.4-0.5% gain for low delay settings with 3-4% encoder runtime and no decoder runtime increase. (See the attached excel sheet)

# Conclusions

This contribution reports the proposed technique can bring substantial coding gain.

Therefore, we suggest that the proposed technique be further investigated in CE.

# Future Work

In this proposal, the merge mvd mode is newly introduced by switching with AMVP mode.

However, there are some future works.

- Non-normative algorithm for low complexity encoder (e.g. no motion estimation for the merge mvd mode)

- How to add mvd to mvp (e.g. the number of transmitted mv)

- Syntax position of the merge mvd mode (e.g. simply replace the merge mode with the merge mvd mode)

# References

[1] “WD3: Working Draft 3 of High-Efficiency Video Coding”,

JCTVC-E603, Geneva, March 2011

[2] “BOG report of CE9: MV Coding and Skip/Merge operations”,

JCTVC-E481, Geneva, March 2011

[3] Frank Bossen “Common test conditions and software reference configurations”,

JCTVC-E700, Geneva, March 2011

[4] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).