| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F374 |
| --- | --- |

| Title: | Cross-verification report of JCTVC-F099 by JVC KENWOOD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of performance of temporal MVP described in JCTVC-F099 by NEC. It is confirmed that the BD-rate results match those provided by the proponent.

# Introduction

This contribution reports cross-verification results of performance of temporal MVP described in JCTVC-F099 by NEC [1].

# Experiments

## Software

The software used is based on HM3.0.

The modification of macro from HM3.0 is as follows.

#define MRG_NEIGH_COL          0

#define AMVP_NEIGH_COL         0

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table 1 and Table 2.

Table 1 Experimental environment for encoding

| CPU | Intel Xeon 5160 3.00GHz |
| --- | --- |
| Memory | 16GB (FBDIM) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Table 2 Experimental environment for decoding

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 3, Table 4, and Table 5 present the simulation results for random access, low delay B, and low delay P, respectively.

Table 3 Simulation results for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -3.1 | -2.5 | -2.9 | -3.3 | -2.6 | -2.9 |
| Class B | -2.0 | -1.4 | -1.4 | -1.6 | -1.1 | -1.2 |
| Class C | -2.1 | -2.1 | -2.2 | -2.0 | -2.0 | -2.0 |
| Class D | -2.0 | -1.8 | -1.9 | -2.0 | -1.8 | -1.8 |
| Class E |  |  |  |  |  |  |
| Overall | -2.3 | -1.9 | -2.1 | -2.2 | -1.8 | -1.9 |
| Enc Time[%] | 109% |  |  | 107% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  |

Table 4 Simulation results for low delay B

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.0 | -2.0 | -1.8 | -2.2 | -1.5 | -1.4 |
| Class C | -2.4 | -2.4 | -2.5 | -2.5 | -2.3 | -2.2 |
| Class D | -2.2 | -1.9 | -2.4 | -2.2 | -1.3 | -1.2 |
| Class E | -2.8 | -2.4 | -1.8 | -2.5 | -1.3 | -1.2 |
| Overall | -2.3 | -2.1 | -2.1 | -2.3 | -1.6 | -1.5 |
| Enc Time[%] | 108% |  |  | 108% |  |  |
| Dec Time[%] | 108% |  |  | 110% |  |  |

Table 5 Simulation results for low delay P

|  | Low delay P HE |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.9 | -1.6 | -0.9 | -1.8 | -1.3 | -1.2 |
| Class C | -2.2 | -2.3 | -2.3 | -2.3 | -2.3 | -2.3 |
| Class D | -1.9 | -1.5 | -1.4 | -2.0 | -1.5 | -2.0 |
| Class E | -2.6 | -2.4 | -2.3 | -2.2 | -1.5 | -1.5 |
| Overall | -2.1 | -1.9 | -1.6 | -2.0 | -1.6 | -1.7 |
| Enc Time[%] | 106% |  |  | 106% |  |  |
| Dec Time[%] | 106% |  |  | 107% |  |  |

# Conclusions

Cross-verification results of performance of temporal MVP described in JCTVC-F099 by NEC are reported. It is confirmed that the BD-rate [3] results match those provided by the proponent.

# References

[1] “Performance report of temporal motion vector prediction”,

JCTVC-F099, Torino, July 2011

[2] Frank Bossen “Common test conditions and software reference configurations”,

JCTVC-E700, Geneva, March 2011

[3] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).