| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F376 |
| --- | --- |

| Title: | Utilisation of CABAC equal probability mode for intra modes coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Il-Koo Kim | Email: | v.seregin@samsung.com ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

In this document CABAC equal probability coding (bypass) mode utilisation in addition to current context modelling for luma and chroma intra modes coding is studied and tested. In proposed method majority bins of intra mode are coded with bypass mode without context modelling by exception of the first bin of chroma intra mode, and luma intra mode for 8x8, 16x16 and 32x32 prediction units. Experimental results reportedly show 70% bins bypass coded with 0.0%, 0.0% and 0.0% BD-rate gain in high efficiency intra-only, random access and low-delay test conditions respectively.

# Problem Statement

According to current test model, every bin of luma and chroma intra direction is coded with context modelling. Context models are separated for the flag, indicating whether intra direction is equal to the most probable mode and rest of the bins in case intra direction is not most probable. For the last case, one context model is used for luma and one for chroma to code all the remaining bins coming from different prediction units (PU) sizes. Maximum possible luma intra modes number varies for different PU sizes and in current implementation this number is 34 for 8x8, 16x16 and 32x32 PU, 17 for 4x4 and 3 for 64x64. So, statistics of intra mode bins representing PUs with various intra mode numbers can be different and coding all remaining bins with shared context model may be not efficient.

Also, context models require some additional computational complexity and memory storage, since context model state is usually updated after each bin coding and it is necessary to keep context state information. So, to overcome this disadvantage, along with context modelling, so called bypass mode is proposed to use. Bypass mode means that there is no context modelling procedure and every symbol is coded with equal probability state.

# Proposal

Stated above problem with context sharing is addressing in this proposal and additionally complexity reduction is achieved by utilising CABAC bypass mode. After most probable mode flag coding, and if intra direction is not equal to most probable mode, it is proposed to separate rest of luma intra direction bins into two categories by maximum intra modes number: first group with 34 intra directions (8x8, 16x16 and 32x32 PU) and second group with other PUs (4x4 and 64x64).

For the first group, since number of intra modes are same, some common statistical properties may exists and context modelling is kept, but it is applied only for the first bin and rest of bins, if exist, are coded with equal probabilities. And second group is coded with equal probabilities only (Tab.1). Let us remark, that coding with equal probabilities or CABAC bypass mode does not require context modelling at all, so it is less complex since context update are skipped.

In addition, similar procedure is applied for chroma intra mode coding, where the first bin of chroma intra mode of every PU is coded with shared context model as it is in current HM and remained bins are coded with equal probabilities without context modelling (Tab.1).

Tab.1. Configuration of bin coding

| PU |  | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 |
| --- | --- | --- | --- | --- | --- | --- |
| Luma | first bin |  | context | context | context |  |
|  | others |  |  |  |  |  |
| Chroma | first bin | context | context | context | context | context |
|  | others |  |  |  |  |  |

* context modelling bypass is marked by gray colour

Proposed method with context skipping was implemented on top of HM3.1. Amount of bypassed bins are analysed in Tab.2 and results are summarised in Tab.3.

Tab.2. Amount of bypassed intra mode bins

|  | Bypassed bins | Total amount of bins |
| --- | --- | --- |
| HE Intra | 70% | 99.7% |
| HE Random access | 70% | 99.5% |
| HE Low delay | 72% | 99.0% |

Comparing to HM3.1, context model number and intra mode bins are not increased, but amount of bins coded without context modelling is about 70%, so complexity, related to context update of intra mode bins, is significantly reduced.

Tab.3. Average BD-rate saving (against HM3.1. anchor)

| BD-rate | Y | U | V | Enc | Dec |
| --- | --- | --- | --- | --- | --- |
| HE Intra | 0.02 | -0.04 | -0.03 | 100% | 99% |
| HE Random access | -0.03 | -0.06 | -0.03 | 100% | 100% |
| HE Low delay | -0.04 | 0.21 | 0.24 | 100% | 100% |
| Total | -0.02 | 0.04 | 0.06 | 100% | 100% |

# Conclusion

Skipping context modelling for intra mode coding reduces complexity and number of intra mode bins, bypass coded, is about 70% with no performance loss. Based on the results, we suggest including this modification into HM.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).