| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F377 |
| --- | --- |

| Title: | Arithmetic coding in high level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032 Japan | Tel: Email: | +81-3-5435-3308 teruhikos@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

In WD3, some syntax elements, ALF and SAO parameters, are encoded in slice layer by arithmetic coding. However, slice layer is handled by firmware, not hard wire, in many HW implementations. Since the resource of firmware is limited, CABAC arithmetic coding is too high for firmware. Therefore, HW complexity become too high if some syntax elements are encoded by CABAC. This contribution proposes,

- Not to use arithmetic coding in high level syntax (slice and higher layer)

- Move ALF and SAO parameter to the top of the slice data

- Or move SAO parameter to the end of slice header

- And/or move syntax to initialize CABAC (slice_type and slice_qp_delta) to the beginning of slice header

# Problem

According to the CABAC initialization process in 9.3.1, CABAC is initialized when starting the parsing of the slice data and at the first tree block of a slice. However, syntax elements in slice header contradict with this initialization process.

alf_param() is located at the end of slice header and the problem is lower. But sao_param() is located at the middle of slice header and this is more problem than ALF.

sao_param() is encoded just after slice_qp_delta. However, slice_qp_delta is used to initialize CABAC. Therefore decoding maybe suspended by initialization, and further syntax elements in slice header can not be parsed.

Moreover, slice layer is handled by firmware, not hard wire, in many HW implementations. Since the resource of firmware is limited, CABAC arithmetic coding is too high for firmware. Therefore, HW complexity become too high if some syntax elements are encoded by CABAC.

# Proposal

There could be the following solutions.

- Move ALF and SAO syntax elements to the beginning of slice data

- Move SAO syntax elements to the end of slice header

- In addition to the above, move syntax to initialize CABAC (slice_type and slice_qp_delta) to the beginning of slice header

Solution (a):

In this case, syntax elements using CABAC are moved sao_offset_param() and alf_cu_control_param() to the top of slice data. The following is proposed syntax.

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

| sao_param( ) { | C | Descriptor |
| --- | --- | --- |
| sample_adaptive_offset_flag | 2 | u(1) |
| if ( sample_adaptive_offset_flag ) { |  |  |
| sao_split_param( 0, 0, 0 ) |  |  |
| sao_offset_param( 0, 0, 0 ) |  |  |
| } |  |  |
| } |  |  |

| slice_data( ) { | Descriptor |
| --- | --- |
| CurrTbAddr = LCUAddress |  |
| moreDataFlag = 1 |  |
| if ( sample_adaptive_offset_flag ){ |  |
| sao_split_param( 0, 0, 0 ) |  |
| sao_offset_param( 0, 0, 0 ) |  |
| } |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_cu_control_param( ) |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = -1 |  |
| do { |  |
| xCU = HorLumaLocation( CurrTbAddr ) |  |
| yCU = VerLumaLocation( CurrTbAddr ) |  |
| moreDataFlag = coding_tree( xCU, yCU, Log2TbSize ) |  |
| CurrTbAddr = NextTbAddress( CurrTbAddr ) |  |
| } while( moreDataFlag ) |  |
| } |  |

In addition to the above the following changes may be applied too. In this case, all syntax elements in alf_cu_control_param() is encoded by CABAC.

| alf_cu_control_param() { | C | Descriptor |
| --- | --- | --- |
| if( adaptive_loop_filter_flag ) { |  |  |
| alf_cu_control_flag | 2 | u(1) ¦ ae(v) |
| if( alf_cu_control_flag ) { |  |  |
| alf_cu_control_max_depth | 2 | ue(v)¦ ae(v) |
| alf_length_cu_control_info | 2 | se(v)¦ae(v) |
| for( i = 0; i < NumAlfCuFlag; i++ ) |  |  |
| alf_cu_flag[ i ] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

Solution (b):

In this case, sao_param() should be moved to the end of slice header. Slice header could be the following.

Slice header:

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| } |  |
| } |  |
| } |  |

Solution (c):

In this case, for example, in addition to solution (b), syntax elements to initialize CABAC (slice_type and slice_qp_delta) are moved to the beginning of slice header

Slice header:

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| slice_qp_delta | se(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| } |  |
| } |  |

# Conclusion

Slice layer is handled by firmware, not hard wire, in many HW implementations. Since the resource of firmware is limited, CABAC arithmetic coding is too high for firmware. Therefore, HW complexity become too high if some syntax elements are encoded by CABAC. This contribution proposes,

- Not to use arithmetic coding in high level syntax (slice and higher layer)

- Move ALF and SAO parameter to the top of the slice data

- Or move SAO parameter to the end of slice header

- And/or move syntax to initialize CABAC (slice_type and slice_qp_delta) to the beginning of slice header

# Patent rights declaration(s)

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Syntax in WD3

The current related syntax is as follows.

Slice header:

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

SAO parameters:

| sao_param( ) { | C | Descriptor |
| --- | --- | --- |
| sample_adaptive_offset_flag | 2 | u(1) |
| if ( sample_adaptive_offset_flag ) { |  |  |
| sao_split_param( 0, 0, 0 ) |  |  |
| sao_offset_param( 0, 0, 0 ) |  |  |
| } |  |  |
| } |  |  |

| sao_split_param( x0, y0, saoDepth ) { | C | Descriptor |
| --- | --- | --- |
| if( saoDepth < SaoMaxDepth ) |  |  |
| sao_split_flag[ saoDepth ][ x0 ][ y0 ] | 2 | u(1) ¦ ae(v) |
| if( sao_split_flag[ saoDepth ][ x0 ][ y0 ] ) { |  |  |
| sao_split_param( x0 + 0, y0 + 0, saoDepth + 1 ) |  |  |
| sao_split_param( x0 + 1, y0 + 0, saoDepth + 1 ) |  |  |
| sao_split_param( x0 + 0, y0 + 1, saoDepth + 1 ) |  |  |
| sao_split_param( x0 + 1, y0 + 1, saoDepth + 1 ) |  |  |
| } |  |  |
| } |  |  |

| sao_offset_param( x0, y0, saoDepth ) { | C | Descriptor |
| --- | --- | --- |
| if( sao_split_flag[ saoDepth ][ x0 ][ y0 ] ) { |  |  |
| sao_offset_param( x0 + 0, y0 + 0, saoDepth + 1 ) |  |  |
| sao_offset_param( x0 + 1, y0 + 0, saoDepth + 1 ) |  |  |
| sao_offset_param( x0 + 0, y0 + 1, saoDepth + 1 ) |  |  |
| sao_offset_param( x0 + 1, y0 + 1, saoDepth + 1 ) |  |  |
| } else { |  |  |
| sao_type_idx[ saoDepth ][ x0 ][ y0 ] | 2 | ue(v) ¦ ae(v) |
| if( sao_type_idx[ saoDepth ][ x0 ][ y0 ] != 0 ) |  |  |
| for( i = 0; i < NumSaoClass; i++ ) |  |  |
| sao_offset[ saoDepth ][ x0 ][ y0 ][ i ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |

AIF parameters:

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_region_adaptation_flag | 2 | u(1) |
| alf_length_luma_minus_5_div2 | 2 | ue(v) |
| alf_no_filters_minus1 | 2 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |  |
| alf_start_second_filter | 2 | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |  |
| for (i=1; i< 16; i++) |  |  |
| alf_filter_pattern[i] | 2 | u(1) |
| } |  |  |
| if (AlfNumFilters > 1) |  |  |
| alf_pred_method | 2 | u(1) |
| alf_min_kstart_minus1 | 2 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |  |
| alf_coeff_luma[i][j] |  | ge(v) |
| alf_chroma_idc | 2 | ue(v) |
| if ( alf_chroma_idc ) { |  |  |
| alf_length_chroma_minus_5_div2 | 2 | ue(v) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |  |
| alf_coeff_chroma[i] |  | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |

| alf_cu_control_param() { | C | Descriptor |
| --- | --- | --- |
| if( adaptive_loop_filter_flag ) { |  |  |
| alf_cu_control_flag | 2 | u(1) |
| if( alf_cu_control_flag ) { |  |  |
| alf_cu_control_max_depth | 2 | ue(v) |
| alf_length_cu_control_info | 2 | se(v) |
| for( i = 0; i < NumAlfCuFlag; i++ ) |  |  |
| alf_cu_flag[ i ] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

Slice data:

| slice_data( ) { | Descriptor |
| --- | --- |
| CurrTbAddr = LCUAddress |  |
| moreDataFlag = 1 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = -1 |  |
| do { |  |
| xCU = HorLumaLocation( CurrTbAddr ) |  |
| yCU = VerLumaLocation( CurrTbAddr ) |  |
| moreDataFlag = coding_tree( xCU, yCU, Log2TbSize ) |  |
| CurrTbAddr = NextTbAddress( CurrTbAddr ) |  |
| } while( moreDataFlag ) |  |
| } |  |