| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F379 |
| --- | --- |

| Title: | CE2: Test results of asymmetric motion partition (AMP) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim Woo-Jin Han JeongHoon Park (Samsung) Xiaozhen Zheng (HiSilicon Technologies) | Email: | ilkoo.kim@samsung.com whhan.han@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. and HiSilicon Technologies |  |  |

_____________________________

# Abstract

In this document, test results of asymmetric motion partition (AMP) are provided. With encoding speedup, the average gain is 0.6% with 12% encoding and 1% decoding time decrease.

# Introduction

In the previous Geneva meeting, asymmetric motion partition (AMP) with overlapped block motion partition (OBMC) was proposed (JCTVC-E316 [1]). In this document, test result of AMP without OBMC is provided based on HM-3.0 [3].

# Asymmetric motion partition (AMP)

AMP was initially proposed in JCTVC-A124 [2]. It allowed rectangular shape PU splitting of a block for inter prediction. Figure 1 shows an example of the AMP where a 64x64 block is asymmetrically partitioned into 64x16, 64x48, 16x64, or 48x16 blocks, instead of two 64x32 or 32x64 blocks. The motion partitions shown in the figure correspond to 2NxnU, 2NxnD, nLx2N, and nRx2N, respectively.  AMP is used from the size of 64x64 to 16x16 CU. AMP improves the coding efficiency, since irregular image patterns, which otherwise would be constrained to being represented by a symmetric partition, can now be more efficiently represented without requiring further splitting.

Figure 1 Example of AMP for 64x64 CU

# Encoding speed-up of AMP

For encoder speed up, additional conditions are checked before doing motion estimation for AMP. If the certain conditions are met, additional motion estimation for AMP can be skipped. Pseudo code for speed up is as followings. Conditions of mode skipping are based on the two values: the best PU size before AMP is evaluated (rpcBestCU->getPartitionSize(0)) and the best PU size at lower level in CU quad-tree (eParentPartSize), which contains current PU.

// SIZE_2NxnU, SIZE_2NxnD, SIZE_nLx2N, SIZE_nRx2N

if( pcPic->getSlice(0)->getSPS()->getAMPAcc(uiDepth) )

{

Bool bTestAMP_Hor = false, bTestAMP_Ver = false;

Bool bTestMergeAMP_Hor = false, bTestMergeAMP_Ver = false;

if ( rpcBestCU->getPartitionSize(0) == SIZE_2NxN )

{

bTestAMP_Hor = true;

}

else if ( rpcBestCU->getPartitionSize(0) == SIZE_Nx2N )

{

bTestAMP_Ver = true;

}

else if ( rpcBestCU->getPartitionSize(0) == SIZE_2Nx2N && rpcBestCU->getMergeFlag(0) == false && rpcBestCU->isSkipped(0) == false )

{

bTestAMP_Hor = true;

bTestAMP_Ver = true;

}

if ( eParentPartSize >= SIZE_2NxnU && eParentPartSize <= SIZE_nRx2N )

{

bTestMergeAMP_Hor = true;

bTestMergeAMP_Ver = true;

}

if ( rpcBestCU->getPartitionSize(0) == SIZE_2Nx2N && rpcBestCU->isSkipped(0) == false )

{

bTestMergeAMP_Hor = true;

bTestMergeAMP_Ver = true;

}

if ( rpcBestCU->getWidth(0) == 64 )

{

bTestAMP_Hor = false;

bTestAMP_Ver = false;

}

if ( eParentPartSize == SIZE_NONE )

{

if ( rpcBestCU->getPartitionSize(0) == SIZE_2NxN )

{

bTestMergeAMP_Hor = true;

}

else if ( rpcBestCU->getPartitionSize(0) == SIZE_Nx2N )

{

bTestMergeAMP_Ver = true;

}

}

//! Do AMP

if ( bTestAMP_Hor )

{

xCheckRDCostInter( rpcBestCU, rpcTempCU, SIZE_2NxnU );   rpcTempCU->initEstData();

xCheckRDCostInter( rpcBestCU, rpcTempCU, SIZE_2NxnD );   rpcTempCU->initEstData();

}

else if ( bTestMergeAMP_Hor )

{

xCheckRDCostInter( rpcBestCU, rpcTempCU, SIZE_2NxnU, true );   rpcTempCU->initEstData();

xCheckRDCostInter( rpcBestCU, rpcTempCU, SIZE_2NxnD, true );   rpcTempCU->initEstData();

}

if ( bTestAMP_Ver )

{

xCheckRDCostInter( rpcBestCU, rpcTempCU, SIZE_nLx2N );   rpcTempCU->initEstData();

xCheckRDCostInter( rpcBestCU, rpcTempCU, SIZE_nRx2N );   rpcTempCU->initEstData();

}

else if ( bTestMergeAMP_Ver )

{

xCheckRDCostInter( rpcBestCU, rpcTempCU, SIZE_nLx2N, true );   rpcTempCU->initEstData();

xCheckRDCostInter( rpcBestCU, rpcTempCU, SIZE_nRx2N, true );   rpcTempCU->initEstData();

}

}

In the pseudo-code, if the value of bTestAMP_Hor is “true”, SIZE_2NxnU and SIZE_2NxnD are evaluated. If the value of bTestAMP_Ver is “true”, SIZE_nLx2N and SIZE_nRx2N are evaluated. In order to compensate loss from skipping AMP mode, additional PU-merging conditions for second PU of AMP is checked instead of skipping AMP mode totally. If the values of bTestMergeAMP_Hor and bTestMergeAMP_Ver are “true” (even if bTestAMP_Hor and bTestAMP_Ver are false), forced PU-merging for second PU of AMP is evaluated. Otherwise, no AMP modes will be evaluated. The variable eParentPartSize means selected PU size at lower level in CU quad-tree, which contains current PU.

# Experimental results

In this section, two test results are provided: AMP without encoding speed-up and AMP with encoding speed-up. The implementations are based on HM-3.0 reference software [3]. The tests are performed under common test conditions [4]. Table 1 and Table 2 summarize the result of each test. According to Table 1, AMP without encoding speed-up shows 0.9% gain with 47% encoding time and -1% decoding increase. Encoding time is significantly increased because 4 additional PU splitting are added and each of them requires extra processes including motion estimation, motion vector prediction for each PU and residual coding at the encoder side.

Table 1 Experimental results of AMP without encoding speed-up

|  | Random access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9 | -1.2 | -0.9 | -0.7 | -0.7 | -0.5 |
| Class B | -0.9 | -1.0 | -1.0 | -0.7 | -0.7 | -0.6 |
| Class C | -0.9 | -1.0 | -1.1 | -0.7 | -0.9 | -0.9 |
| Class D | -0.8 | -1.0 | -0.9 | -0.5 | -0.7 | -0.6 |
| Class E |  |  |  |  |  |  |
| Overall | -0.9 | -1.0 | -1.0 | -0.7 | -0.7 | -0.7 |
| Enc Time[%] | 144% |  |  | 151% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay (B) HE |  |  | Low delay (B) LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.1 | -1.5 | -1.5 | -0.9 | -0.8 | -0.6 |
| Class C | -1.0 | -1.2 | -1.3 | -0.7 | -0.6 | -0.7 |
| Class D | -1.1 | -1.3 | -1.5 | -0.6 | -0.5 | -0.9 |
| Class E | -2.3 | -2.2 | -2.4 | -1.7 | -1.1 | -1.3 |
| Overall | -1.3 | -1.5 | -1.6 | -0.9 | -0.7 | -0.8 |
| Enc Time[%] | 144% |  |  | 150% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 2 shows the result of AMP with encoding speed-up. AMP with encoding speed-up shows 0.6% gain with 12% encoding time and 1% decoding decrease. Compared to “without encoding speed-up” case, 35% encoding time is reduced and no decoding time is increased. Encoding and decoding time measurement may have 1-2% measurement error since clustered system is utilized to produce these results.

Table 2 Experimental results of AMP with encoding speed-up

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5 | -0.8 | -0.5 | -0.4 | -0.6 | -0.2 |
| Class B | -0.5 | -0.8 | -0.7 | -0.4 | -0.5 | -0.5 |
| Class C | -0.6 | -0.8 | -0.8 | -0.5 | -0.6 | -0.7 |
| Class D | -0.5 | -0.9 | -0.8 | -0.4 | -0.5 | -0.6 |
| Class E |  |  |  |  |  |  |
| Overall | -0.5 | -0.8 | -0.7 | -0.4 | -0.6 | -0.5 |
| Enc Time[%] | 112% |  |  | 112% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | -1.1 | -1.2 | -0.5 | -0.4 | -0.3 |
| Class C | -0.7 | -1.0 | -0.9 | -0.4 | -0.4 | -0.7 |
| Class D | -0.7 | -1.2 | -0.8 | -0.5 | -0.7 | -0.2 |
| Class E | -1.5 | -1.9 | -1.7 | -1.0 | -1.0 | -0.9 |
| Overall | -0.8 | -1.2 | -1.1 | -0.6 | -0.6 | -0.5 |
| Enc Time[%] | 111% |  |  | 111% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The results of Table 2 are cross-verified by HHI (JCTVC-F435).

# Conclusions

In this contribution, AMP without encoding speedup and with encoding speed-up is presented. AMP with reasonable encoding speed-up shows 0.6% gain with 12% encoding time increase and 1% decoding time decrease. By encoding speed-up, 0.6% out of 0.9% gain is maintained with significant encoding time decrease, 47% to 12%. Furthermore, AMP can be easily incorporated with non-square transform quad-tree scheme (NSQT, JCTVC-F412) and this unification brings additional 0.7% gain. Thus, it is recommended to adopt AMP as a part of HEVC standard.

# References

- I.-K. Kim, P. Chen and L. Guo, “CE2: Test results of asymmetric motion partition (AMP) with overlapped block motion compensation (OBMC)”, JCTVC-E316, 5th Meeting: Geneva, CH, 16-23 March, 2011

- K. McCann and W.-J. Han et al, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, JCTVC-A124, 1st Meeting: Dresden, DE, 15-23 April, 2010

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0/

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed draft text

### Coding unit syntax

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| else { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( slice_type == I && log2CUSize == Log2MinCUSize ) |  |
| intra_part_mode | u(1) |
| } else if( slice_type != I ¦ ¦ log2CUSize = = Log2MinCUSize ) |  |
| pred_type | u(v) ¦ ae(v) |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| x2 = x1 - ( ( 1 << log2CUSize ) >> 2 ) |  |
| y2 = y1 - ( ( 1 << log2CUSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CUSize ) >> 2 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 1, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x0, y1, log2CUSize, log2CUSize – 1, 1 , 0 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 1, log2CUSize, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x1, y0, log2CUSize - 1, log2CUSize, 1 , 0 ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 2, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x0, y2, log2CUSize, log2CUSize – 1, 1 , 0 ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 1, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x0, y3, log2CUSize, log2CUSize – 2, 1 , 0 ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 2, log2CUSize, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x2, y0, log2CUSize - 1, log2CUSize, 1 , 0 ) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 1, log2CUSize, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x3, y0, log2CUSize - 2, log2CUSize, 1 , 0 ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, log2CUSize – 1, log2CUSize – 1, 0 , 0 ) |  |
| prediction_unit( x1, y0, log2CUSize – 1, log2CUSize – 1, 1 , 0 ) |  |
| prediction_unit( x0, y1, log2CUSize – 1, log2CUSize – 1, 2 , 0 ) |  |
| prediction_unit( x1, y1, log2CUSize – 1, log2CUSize – 1, 3 , 0 ) |  |
| } |  |
| } |  |
| if( !pcm_flag ) { |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

pred_type specifies prediction mode and partitioning type of the current coding unit. The semantics of pred_type depend on the slice type. The variables PredMode, PartMode and IntraSplitFlag are derived from the value of pred_type as defined in Table 7-3. The value of pred_type shall not be equal to 8 or 9 when slice_type is equal to P or B and log2CUSize is greater than Log2MinCUSize.

When pred_type is not present, the variables PredMode and PartMode are derived as follows.

- If slice_type is equal to I and log2CUSize is greater than Log2MinCUSize,

  - PredMode is inferred to be equal to MODE_INTRA

  - PartMode is inferred to be equal to PART_2Nx2N

  - IntraSplitFlag is inferred to be equal to 0

- Otherwise (slice_type is equal to P or B), if skip_flag[ x0 ][ y0 ] is equal to 1,

  - PredMode is inferred to be equal to MODE_SKIP

  - PartMode is inferred to be equal to PART_2Nx2N

Table 7-3 - Name association to prediction mode and partitioning type

| - slice_type | - pred_type | - IntraSplitFlag | - PredMode | PartMode |
| --- | --- | --- | --- | --- |
| - I | - 0 | - 0 | - MODE_INTRA | - PART_2Nx2N |
|  | - 1 | - 1 | - MODE_INTRA | - PART_NxN |
| - P or B | - 0 | - - | - MODE_INTER | PART_2Nx2N |
|  | - 1 | - - | - MODE_INTER | PART_2NxN |
|  | - 2 | - - | - MODE_INTER | PART_Nx2N |
|  | - 3 | - - | - MODE_INTER | PART_NxN |
|  | - 4 | - - | - MODE_INTER | - PART_2NxnU |
|  | - 5 | - - | - MODE_INTER | - PART_2NxnD |
|  | - 6 | - - | - MODE_INTER | - PART_nLx2N |
| - I | - 7 | - - | - MODE_INTER | - PART_nRx2N |
|  | - 8 | - 0 | - MODE_INTRA | - PART_2Nx2N |
| - I | - 9 | - 1 | - MODE_INTRA | - PART_NxN |
|  | - inferred | - - | - MODE_SKIP | - PART_2Nx2N |

#### Derivation process of prediction unit boundary

Inputs of this process are:

–	a variable log2CUSize specifying the coding unit size,

–	a prediction partition mode PartMode.

Outputs of this process are:

–	two-dimensional arrays of (nS)x(nS), horEdgeFlags and verEdgeFlags.

Depending on PartMode, the following applies:

–	If PartMode is equal to PART_2NxN or PART_NxN, horEdgeFlags[ k ][ 1 << ( log2CUSize – 1 ) ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_Nx2N or PART_NxN, verEdgeFlags[ 1 << ( log2CUSize – 1 ) ][ k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_2NxnU, horEdgeFlags[ k ][ (1 << ( log2CUSize – 1 )) – (1 << ( log2CUSize – 2 )) ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_2NxnD, horEdgeFlags[ k ][ (1 << ( log2CUSize – 1 )) + (1 << ( log2CUSize – 2 )) ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_nLx2N, verEdgeFlags[ (1 << ( log2CUSize – 1 )) – (1 << ( log2CUSize – 2 )) ][ k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_nRx2N, verEdgeFlags[ (1 << ( log2CUSize – 1 )) + (1 << ( log2CUSize – 2 )) ][ k ] is set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

#### Initialisation process for splitPredPartModeTable

Outputs of this process are initial values of the variable array splitPredPartModeTable and associated counters.

The variable array splitPredPartModeTable is initialized as follows:

–	For each cuDepth with a value from 0 to (Log2MaxCUSize – Log2MinCUSize), set splitPredPartModeTable[cuDepth][codeNum] equal to codeNum, with codeNum =  0…10.

For each cuDepth with a value from 0 to (Log2MaxCUSize – Log2MinCUSize), there are four counters used for codeword index adaptive mapping. These counters are initialized according to subclause Error: Reference source not found.

#### Parsing process for cu_split_pred_part_mode

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element cu_split_pred_part_mode in subclause Error: Reference source not found.

Inputs to this process are bits from slice data, splitPredPartModeTable.

Output of this process is the syntax element cu_split_pred_part_mode.

The value of cu_split_pred_part_mode is derived as follows.

- If log2CUSize is greater than Log2MinCUSize, the parsing process described in subclause Error: Reference source not found is invoked with vlcNum of 14 and cMax of 10 as inputs and the variable codeNum as output.

- Otherwise, the parsing process described in subclause Error: Reference source not found is invoked with vlcNum of 14 and cMax of 9 as inputs and the variable codeNum as output.

- The syntax element cu_split_pred_part_mode is derived as

cu_split_pred_part_mode = splitPredPartModeTable[Log2MaxCUSize – log2CUSize][codeNum] 	(9-1)

- If log2CUSize is equal to Log2MinCUSize and cu_split_part_mode has a value of 5, PredMode and PartMode are determined as follows

- Read one bit and assign its value to a variable b

- If b is equal to 1, set PredMode to MODE_INTRA and PartMode to PART_2Nx2N

- Otherwise, read the next one bit and assign its value to the variable b.

    - if b is equal to 1, set PredMode to MODE_INTRA and PartMode to PART_NxN

    - Otherwise, set PredMode to MODE_INTER and PartMode to PART_NxN

- The variable array splitPredPartModeTable[Log2MaxCUSize – log2CUSize] is updated by invoking process Error: Reference source not found with splitPredPartModeTable[Log2MaxCUSize – log2CUSize], codeNum and a value of 4 as inputs.

#### Binarization process for pred_type

Input to this process is a request for a binarization for the syntax element pred_type and a variable cLog2CUSize specifying the current CU size.

Output of this process is the binarization of the syntax element.

The binarization for pred_type is given by Table 9-4 depending on slice type and the size of the coding unit.

Table 9-4 – Binarization for pred_type

| Slice type | Value of pred_type | PredMode | PartMode | Bin string |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | cLog2CUSize > 3 | cLog2CUSize = = 3e |
| I | 0 | MODE_INTRA | PART_2Nx2N | - | 1 |
|  | 1 | MODE_INTRA | PART_NxN | - | 0 |
| P | 0 | MODE_INTER | PART_2Nx2N | 01 | 01 |
|  | 1 | MODE_INTER | PART_2NxN | 0011 | 0011 |
|  | 2 | MODE_INTER | PART_Nx2N | 0001 | 00011 |
|  | 3 | MODE_INTER | PART_NxN | - | 0000 |
|  | 4 | MODE_INTER | PART_2NxnU | 00100 | 00100 |
|  | 5 | MODE_INTER | PART_2NxnD | 00101 | 00101 |
|  | 6 | MODE_INTER | PART_nLx2N | 00000 | 000100 |
|  | 7 | MODE_INTER | PART_nRx2N | 00001 | 000101 |
|  | 4 | MODE_INTRA | PART_2Nx2N | 1 | 11 |
|  | 5 | MODE_INTRA | PART_NxN | - | 10 |
| B | 0 | MODE_INTER | PART_2Nx2N | 1 | 1 |
|  | 1 | MODE_INTER | PART_2NxN | 011 | 011 |
|  | 2 | MODE_INTER | PART_Nx2N | 001 | 001 |
|  | 3 | MODE_INTER | PART_NxN | - | 0001 |
|  | 4 | MODE_INTER | PART_2NxnU | 0100 | 0100 |
|  | 5 | MODE_INTER | PART_2NxnD | 0101 | 0101 |
|  | 6 | MODE_INTER | PART_nLx2N | 0000 | 0000 |
|  | 7 | MODE_INTER | PART_nRx2N | 0001 | 0001 |
|  | 4 | MODE_INTRA | PART_2Nx2N | 000 | 0000 1 |
|  | 5 | MODE_INTRA | PART_NxN | - | 0000 0 |