| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F381 |
| --- | --- |

| Title: | Clean decoding refresh definition and decoding process. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | TK Tan A. Fujibayashi 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +65 97993861 tktan.p@rd.nttdocomo.co.jp +81-468-40-6833 fujibayashi@nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an editorial refinement to the definition of the clean decoder refresh.  It also proposes to explicitly specify the decoding process that is implied by the definition, so as to avoid any ambiguity.

# Introduction

Clean random access is an important feature for the HEVC standard and there should be a normative way of specifying its effects.  Clean decoder refresh (CDR) and instantaneous decoder refresh (IDR) are two tools that WD currently signals as NAL unit types in the NAL unit header.

Both IDR and CDR, enables clean random access by ensuring that reference pictures that are not available when decoding starts in the middle of the bitstream at the IDR or CDR picture, are not used for reference in the decoding process of subsequent pictures.  However, using IDR pictures as the random access point causes the decoded picture buffer (DPB) to be reset immediately and therefore reduces the coding efficiency of the coded bitstream.  Using CDR pictures as random access point improves the coding efficiency as it does not reset the DPB immediately.

This contribution clarifies the definition of the CDR picture and proposes improvements in the description and the decoding process.

Figure 1 shows the random access entry point hereafter referred to as the CDR picture (shown in yellow colour) and the first picture in decode order with an output order greater than that of the CDR picture hereafter referred to as the CDR reset picture (shown in dark blue colour).

The figure shows the position of these pictures relative to other pictures in decode order and output order.  Due to frame reordering, some pictures with output order smaller than the output order of the CDR picture is decoded after the CDR picture but prior to the CDR reset picture in decode order (these are shown in light green colour).

In a random access decoding starting from the CDR picture, the above mentioned pictures cannot be decoded correctly as reference pictures prior to the CDR picture is not available.  They should also be discarded as the output of the random access decoding starts with the CDR picture.  This is illustrated in Figure 2.

Figure 1: Pictures in decode and output order in a normal decoding

Figure 2: Pictures in decode and output order in a random access decoding starting at the CDR picture

This contribution contains two parts.  The first part proposes an editorial improvement to the description of the CDR in the WD3.0 [1].  The second part proposes to specify the decoding process explicitly so as to remove the ambiguity of the current implicit decoding process.

# CDR definition

In a CDR random access process, the reference pictures prior to the CDR reset picture (except for the CDR picture) are not decodable and are discarded.  The same decoding outcome is achieved during a normal decoding if starting from the CDR reset picture in decoding order, these pictures shall no longer be used for inter prediction.  Therefore, the CDR reset picture and the CDR picture are defined as follows.

- CDR reset picture: The first coded picture in decode order with an output order greater than the output order of the CDR picture. The CDR reset picture shall only refer to the CDR picture for inter prediction. After the CDR reset picture is decoded, all reference pictures with decode order or output order smaller than the corresponding value for the CDR picture shall be marked as “unused for reference”.

- clean decoding refresh (CDR) picture: A coded picture containing only slices with I slice types that indicates a starting point for decoding from the middle of a bitstream. The CDR picture shall not be affected by the marking process of the coded pictures prior to the CDR reset picture in decode order.

# CDR decoding process

It is proposed to specify the decoding process explicitly by adding the following text to section 8.2.2 and 8.2.3 of the WD.

Add to section 8.2.2 Decoding process for reference picture lists construction

- If the current picture is the CDR reset picture, then reference pictures with decode order or output order smaller than the corresponding value for the most recently decoded CDR picture shall be inferred to be marked as “unused_for_reference” in subclauses 8.2.2.2 through 8.2.2.3.

Add to section 8.2.3 Decoded reference picture marking process

- If the current picture is the CDR reset picture, then reference pictures with decode order or output order smaller than the corresponding value for the most recently decoded CDR picture shall be marked as “unused for reference”.

# Reference

- Thomas Wiegand, et. al., “WD3: Working Draft 3 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document: JCTVC-E603, 5th Meeting: Geneva, CH, 16-23 March, 2011.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).