| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F384r2 |
| --- | --- |

| Title: | CE8.1:Block based Adaptive Loop Filter with flexible syntax and additional BA mode by Sharp and Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai1 Marta Karczewicz2 In Suk Chong2 Yukinobu Yasugi1 Takanori Yamazaki1 | Tel: Email: | ikai.tomohiro@sharp.co.jp martak@qualcomm.com ichong@qualcomm.com yasugi.yukinobu@sharp.co.jp yamazaki.takanori@sharp.co.jp |
| Source: | SHARP Corporation1 and Qualcomm2 |  |  |

_____________________________

# Abstract

This contribution presents one of CE8 subtest1 proposals, which aims to improve block-based adaptive mode in ALF adopted in Geneva meeting. This proposal includes two primary aspects: 2D flexible class representation and additional block-based adaptive (BA) mode with a minor modification: finer classification. It is reported those aspects improve coding gain. In addition, the first aspect contributes to improve encoder design. The coding gains of 0.3 %, 0.3 %, 0.4% and 0.4 % in HE-AI, RA, LB, LP were reported. The decoding time ratio was 100 % to 102 % and the encoding time ratio was 100 % to 101 % in HE case. The proposal was cross-checked by MediaTek (JCTVC-F067).

The proposed method is integrated with the other CE8 subtest1 proposal (JCTVC-F321), and the integrated contribution (JCTVC-F522) shows those gain of both proposals are additive.

It is recommended to adopt the 2D flexible class representation and the additional BA mode as the integrated one in HM.

# Introduction

In Geneva meeting, Block-based / Region-based ALF (JCTVC-E323 [1] / JCTVC-E046 [2]) was adopted in which Block-based Adaptation (BA) and Region-based Adaptation (RA) were introduced. In Geneva meeting, integration with another BA proposal (JCTVC-E141 [3]) was also suggested and enhancement of block-based ALF is encouraged in CE.  JCTVC-E141 enables two-directional merge for classification which is more flexible than one-dimensional merge, which is promising of additional coding gain.  In this contribution,  flexible two-dimensional friendly syntax named “2D flexible class representation” is proposed. Moreover “additional BA mode”, where the same kind computation as the current design is used for capturing different edge directions, is also proposed to improve BA approach. As E141 suggested, finer classification in BA mode is used.

Flexible class representation

Flexible class representation is aimed to free the block classification restriction of the HM 3.0 ALF. This representation brings not only coding efficiency improvement but also large degree of freedom of encoder algorithm such as top-down splitting, bottom-up merging, some clustering algorithm and one-dimensional algorithm and so on, which is desirable if it is implemented in many kinds of target systems.

In Figure 1, BA class and RA class is shown. Both modes have two-dimensional axes: activity and direction axes in BA class and X and Y axes in RA class. The 15 to 16 classes defined in BA and RA can be merged into small groups which can reduce the amount of coefficients. However, the current syntax is restricted to one-dimensional merge, that is, the merges of adjoining classes only can be signaled. For example, class 0 and class 1 can be merged. But class 0 and class 2 to 15 cannot be merged unless all classes (class 0 through class 2 to 15) are merged into one group. The proposed syntax resolves the above problem.

Figure 1: BA class and RA class description

Additional Block-based feature calculation

In HM 3.0, BA uses direction feature for classification. The direction feature is computed by comparing horizontal activity and vertical activity. However, there are cases the dominant edge has diagonal direction as in BasketballDrill sequence. In those cases, use of diagonal activities is better than the current horizontal and vertical activities. The additional BA mode is introduced for this assumption.

The BA and additional BA can share a common hardware, such as summention of laplacian and index calcuration, therefore the implementation cost and hardware complexity is restricted. The effectiveness of two complementary orthogonal modes is logical and the performance was reported in this contribution.

# Proposed method

The proposed “2D flexible class representation” encodes filter index of each class directly. Figure 2 shows an example of indexes. The classes has a same index is regarded as merged region where a corresponding coefficients are sent.

Figure 2: 2D Flexible class representation

The syntax text is shown in Table 1.

Table 1 Syntax text of proposed method

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_region_adaptation_flag | 2 | u(v) |
| alf_length_luma_minus_5_div2 | 2 | ue(v) |
| alf_no_filters_minus1 | 2 | ue(v) |
| for (i=1; i < AlfNumRegions; i++) { |  |  |
| if (alf_no_filters_minus1 == 1) { |  |  |
| alf_region_idx[ i ] |  | u(1) |
| } else if (alf_no_filters_minus1 > 1) { |  |  |
| if( remain_fixed_regions > 0 ) { |  |  |
| prev_alf_region_pred_flag[ i ] |  | u(1) |
| if( prev_alf_region_pred_flag[ i ] ) |  |  |
| rem_alf_region_pred_idx[ i ] |  | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| if (AlfNumFilters > 1) |  |  |
| alf_pred_method | 2 | u(1) |
| alf_min_kstart_minus1 | 2 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | u(1) |

alf_region_idx[ i ] specifies the filter index for adaptive loop filter.

prev_alf_region_pred_flag[ i ] and rem_alf_region_pred_idx [ i ] specify the filter index for adaptive loop filter . When prev_alf_region_pred_flag[i ] is equal to 0, the alf_region_idx[ i ] will be inferred from neighboring region (i.e., upper or left).

Additional BA mode

In addition to BA and RA, the additional BA mode, named BA2 mode, is proposed. The BA, BA2 and RA mode is switchable with a flag alf_region_adaptation_flag.

Conceptually BA2 mode is the same as BA. The only difference is the laplacian computations to capture the block’s edge direction.

In BA mode, the calculation text is described as,

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-1)
	varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y-1 ] – s’’[ xC+x, yC+y+1 ] |	(8-2)

While in BA2 mode, the calculation text is described as,

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y+1 ] – s’’[ xC+x+1, yC+y-1 ] |	(8-3)
	varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y-1 ] – s’’[ xC+x+1, yC+y+1 ] |	(8-4)

Other than above the laplacian computation is the same as BA mode.

The directionalities computed in BA and BA2 are illustrated in Figure 3. BA2 computes two diagonal activities while BA computes horizontal and vertical activity.

Figure 3 Feature values computation in BA and BA2

Finer classifications

In this proposal, a finer classification, which replaces 16 classes of HM3.0 with 18 classes as proposed in E141, is used. It provides better classification of block which leads to better coding gain.

# Experiments

The proposed method was implemented on HM 3.0 and simulated in accordance with the CE 8 subtest 1 test conditions. The experimental results are shown in Table 2.

Table 2 Experimental results

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1 | 0.0 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 |
| Class C | -0.5 | 0.0 | 0.0 |
| Class D | -0.4 | 0.2 | 0.2 |
| Class E | -0.3 | 0.0 | 0.0 |
| Overall | -0.3 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.3 | -0.4 | -0.1 |
| Class B | -0.3 | -0.1 | 0.0 |
| Class C | -0.3 | 0.1 | 0.0 |
| Class D | -0.3 | 0.0 | -0.1 |
| Class E |  |  |  |
| Overall | -0.3 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.3 | 0.1 | -0.3 |
| Class C | -0.4 | 0.0 | -0.2 |
| Class D | -0.3 | 0.1 | 0.4 |
| Class E | -0.5 | 0.1 | -0.5 |
| Overall | -0.4 | 0.1 | -0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.3 | 0.0 | -0.1 |
| Class C | -0.4 | 0.1 | -0.1 |
| Class D | -0.4 | 0.0 | -0.2 |
| Class E | -0.6 | -0.1 | -0.5 |
| Overall | -0.4 | 0.0 | -0.2 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |

# Conclusion

This contribution presents one of CE8 subtest1 proposals, which aims to improve block-based adaptive mode in ALF adopted in Geneva meeting. This proposal includes two primary aspects: 2D flexible class representation and additional block-based adaptive (BA) mode with a minor modification: finer classification. It is reported those aspects improve coding gain. In addition, the first aspect contributes to improve encoder design. The coding gains of 0.3 %, 0.3 %, 0.4% and 0.4 % in HE-AI, RA, LB, LP were reported. The decoding time ratio was 100 % to 102 % and the encoding time ratio was 100 % to 101 % in HE case.

The proposed method is integrated with the other CE8 subtest1 proposal [5], and the integrated contribution [6] shows those gain of both proposals are additive.

It is recommended to adopt the 2D flexible class representation and the additional BA mode as the integrated one in HM.

# Reference

- I. S. Chong, M. Karczewicz, Ching-Yeh Chen, Chih-Ming Fu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei, Tomoo Yamakage, Takeshi Chujoh, Takashi Watanabe, “CE8 Subtest 2: Block based adaptive loop filter (ALF)”, JCTVC-E323, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

- Ching-Yeh Chen, Chih-Ming Fu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei, “CE8 Subtest 2: Adaptation between Pixel-based and Region-based Filter Selection”, JCTVC-E046, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

- T. Ikai and T. Yamazaki, “CE8.1: DF-combined adaptive loop filter”, JCTVC-E141, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

- T.Yamakage, Y.W. Huang, I.S. Chong, T. Ikai, M. Narroschke, Wang L. Lai, Ali Tabatabai, Madhukar Budagavi, Faouzi Kossentini, “Description of CE8: Non-deblocking loop filtering”, JCTVC-E708, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

- T. Yamakage, T.Watanabe, T. Chujoh, C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, M. Karczewicz, I. S. Chong, “CE8.1: Block based Adaptive Loop Filter by MediaTek, Qualcomm and Toshiba”, JCTVC-F321, 6th JCT-VC Meeting, Torino, Italy, July 2011.

- I. S. Chong, M. Karczewicz, T. Yamakage, T. Watanabe, T. Chujoh, C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, T. Ikai, A. Segall, T. Yamamoto, Y. Yasugi, T. Yamazaki, “Enhancing block/region based Adaptive Loop Filter by MediaTek, Qualcomm, Sharp and Toshiba”, JCTVC-F522, 6th JCT-VC Meeting, Torino, Italy, July 2011.

- C.-Y. Chen, Y.-W. Huang, “CE8 Subtest 1: Crosscheck for Sharp and Qualcomm's Adaptive Loop Filter in JCTVC-F067”, JCTVC-F522, 6th JCT-VC Meeting, Torino, Italy, July 2011.

# Patent rights declaration(s)

SHARP Corporation and Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).