| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F385 |
| --- | --- |

| Title: | CE2: Cross-check report for Qualcomm's OBMC for 2NxN and Nx2N motion partitions (JCTVC-F299) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim | Email: | ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this document, cross-check results of overlapped block motion compensation (OBMC) for 2NxN and Nx2N motion partitions (JCTVC-F299) are provided. The average gain is 0.7% with 1% encoding and decoding time increase. All results are matched with those of JCTVC-F299.

# Introduction

In this document, cross-check results of overlapped block motion compensation (OBMC) for 2NxN and Nx2N motion partitions (JCTVC-F299 [2]) are provided in the context of CE2 [1]. If a CU is partitioned into2NxN or Nx2N, OBMC is applied to the horizontal boundary or vertical boundary.

# Experimental results

In this section, test results of OBMC technique for 2NxN and Nx2N motion partitions are provided. The implementation was provided by Qualcomm and the test was performed under common test conditions [4]. Table 1 summarizes results of the test. According to Table 1, OBMC for 2NxN and Nx2N motion partitions shows 0.7% gain with 1% encoding time and decoding increase.

Table 1 Test results of OBMC for 2NxN and Nx2N motion partitions

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4 | -1.0 | -1.0 | -0.5 | -0.6 | -0.4 |
| Class B | -0.4 | -0.8 | -0.6 | -0.5 | -0.6 | -0.6 |
| Class C | -0.7 | -1.0 | -1.0 | -0.6 | -1.0 | -1.1 |
| Class D | -0.7 | -1.0 | -1.0 | -0.5 | -0.9 | -0.8 |
| Class E |  |  |  |  |  |  |
| Overall | -0.5 | -0.9 | -0.9 | -0.5 | -0.8 | -0.7 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | -0.6 | -0.8 | -0.5 | -0.6 | -0.8 |
| Class C | -0.8 | -1.3 | -1.4 | -0.8 | -0.9 | -1.0 |
| Class D | -0.9 | -0.9 | -0.9 | -0.8 | -1.0 | -1.0 |
| Class E | -1.1 | -0.9 | -1.0 | -1.1 | -0.9 | -1.2 |
| Overall | -0.8 | -0.9 | -1.0 | -0.8 | -0.8 | -1.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

These results are matched with those of JCTVC-F299.

# References

- I.-K. Kim, P. Chen and L. Guo, “CE2: Test results of asymmetric motion partition (AMP) with overlapped block motion compensation (OBMC)”, JCTVC-E316, 5th Meeting: Geneva, CH, 16-23 March, 2011

- L. Guo, P. Chen, I. S. Chong, R. Joshi, X. Wang and M. Karczewicz (Qualcomm), “CE2:Overlapped Block Motion Compensation for 2NxN and Nx2N Motion Partitions”, JCTVC-F299, 6th Meeting: Torino, 14-22 July, 2011

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0/

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011