| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F386 |
| --- | --- |

| Title: | Chroma RD cost computation in HM3.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | TK Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan Frank Bossen 3240 Hillview Ave Palo Alto, CA 94304 USA | Tel: Email: | +65 97993861 tktan.p@rd.nttdocomo.co.jp +1 650 496 4742 bossen@docomolabs-usa.com |
| Source: | NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

This contribution observes that the rate-distortion cost computation in the HM does not take into account the difference in the qp values between the luma and chroma components. This results in an inaccurate rate-distortion optimization that penalizes the quality of the chroma component at low bitrates. A rate-distortion cost computation that takes the difference into account was implemented. Simulation results show improvements in the chroma fidelity at the cost of a slight luma BD-rate loss. A summary of the BD-rate for the luma and chroma components are shown in the table below.

The average Y BD-rate loss for all coding conditions is 0.7%. The average U and V BD-rate gains for all coding conditions are 7.0% and 7.2%, respectively.

A weighted YUV BD-rate is also computed in the table, with the average Y, U and V PSNR’s of each sequence being weighted by the ratios 0.75:0.125:0.125, respectively. The average weighted YUV BD-rate gain for all coding conditions is 1.1%.

Table 1: BD-rate for the suggested rate distortion cost computation compared to HM3.0 default conditions.

The results also show a better balance in the luma and chroma BD-rate gains compared to JM17.2 under CfP anchor conditions.

# Introduction

It was observed that the RD cost computation used in RDO, RDOQ, ASO and ALF, does not take the chroma qp mapping into account. It uses a single lambda parameter that is derived from the luma qp value for all cost computations.

Chroma qp mapping is a function where the chroma component is assigned a qp value that is smaller than that of the luma component, when the luma qp is greater that 29.  Figure 1 shows the luma to chroma qp mapping as specified in the working draft [1].

Figure 1: Graph showing the luma to chroma qp mapping

However since the RD cost computation does not reflect this rate assignment, the decision functions do not result in the desired rate allocation between the luma and chroma components.

The problem manifests itself at larger qp values where the chroma mapping is in effect.  The chroma component suffers in terms of quality and sometimes the chroma PSNR becomes lower than that of JM17.2 using the alpha or beta CfP anchor conditions [2].  Figure 2 shows one example where the RD curve of the V component coded with HM3.0 HE_LD crosses below the rd curve of the V component coded with JM17.2 Beta.

Figure 2: Graph comparing RD Curve of V component of ParkScene for HM3.0 and JM17.2

The HM3.0 BD-rate measured against the JM17.2 alpha, beta and all intra anchors show that the gains in the chroma component are smaller than that of the luma component for all the common conditions except for random access and all intra LC.  Table 2 Shows the Y, U and V BD-rate compared to the JM anchors.

Table 2: HM3.0 default conditions compared to the JM17.2 alpha, beta and all intra anchors.

The average Y BD-rate gain for all coding conditions is about 32%.  The average U and V BD-rate gains for all coding conditions are only 25% and 24%, respectively.

# Solution

The solution to this mismatch is to use the appropriate lambda values when calculating the RD cost of the luma and chroma components.  In the case of RDOQ, SAO and ALF, it is possible to derive separate luma and chroma lambda values for the RD cost computation.  However, in RDO the bit rate is a combination of both luma and chroma bit rate therefore there is only one lambda value.  In this case, a weight is applied to the chroma distortion, instead.

In HM3.0, the lambda parameter is dependent on the qp parameter in two ways.  The first is the commonly used dependency where

λ=const⋅2qp62=const⋅2qp3							(1)

The second applies to intermediate B and P frames where the lambda parameter is further scaled by a scaling parameter equivalent to

scaleint=max2.0,min4.0,qp6							(2)

Since the effect of the second dependency is small and only applies to intermediate B and P frames in hierarchical coding structures, it is ignored in the following derivation.

From equation (1) we derive the chroma lambda, λchroma and the weight for chroma distortion, wchroma as follows

λchroma=const⋅2qpchroma62=const⋅2qpchroma3						(3)

wchroma=λλchroma=const⋅2qp3const⋅2qpchroma3=2qp−qpchroma3						(4)

For RDOQ, SAO and ALF, λchroma is used instead of λ when calculating the RD cost related to the chroma component.

For RDO, the RD cost is calculated as

J=Dluma+wchroma⋅Dchroma+λ⋅RwhereDluma and Dchroma are the luma and chroma distortion, respectively, andR is the combined luma and chroma bit rate. 		(5)

# Results

The summary of the results for each of the common coding conditions are shown below.

The results show that the coding gain in the chroma component and the coding loss in the luma component are consistent for all coding conditions tested.

Sections 3.1 and 3.2 show the Y, U and V RD curves of one selected sequence for each class under the HE-RA and HE-LD coding conditions compared to the JM17.2 with alpha and beta coding conditions, respectively.  All sequences show improved U and V PSNR at the lower bitrate points, with negligible reduction in the Y PSNR that is hardly perceptible on the graphs.

## HM3.0 and HM3.0 with chroma rd cost (HE-RA) cf. JM17.2 (alpha anchor).

Figure 3: RD curves of Y, U and V components of PeopleOnStreet for JM17.2 (alpha anchor), HM3.0 and HM3.0 with chroma rd cost (HE-RA).

Figure 4: RD curves of Y, U and V components of ParkScene for JM17.2 (alpha anchor), HM3.0 and HM3.0 with chroma rd cost (HE-RA).

Figure 5: RD curves of Y, U and V components of PartyScene for JM17.2 (alpha anchor), HM3.0 and HM3.0 with chroma rd cost (HE-RA).

Figure 6: RD curves of Y, U and V components of BQSquare for JM17.2 (alpha anchor), HM3.0 and HM3.0 with chroma rd cost (HE-RA).

## HM3.0 and HM3.0 with chroma rd cost (HE-LD) cf. JM17.2 (beta anchor).

Figure 7: RD curves of Y, U and V components of ParkScene for JM17.2 (beta anchor), HM3.0 and HM3.0 with chroma rd cost (HE-LD).

Figure 8: RD curves of Y, U and V components of Vidyo3 for JM17.2 (beta anchor), HM3.0 and HM3.0 with chroma rd cost (HE-LD).

Figure 9: RD curves of Y, U and V components of PartyScene for JM17.2 (beta anchor), HM3.0 and HM3.0 with chroma rd cost (HE-LD).

Figure 10: RD curves of Y, U and V components of BQSquare for JM17.2 (beta anchor), HM3.0 and HM3.0 with chroma rd cost (HE-LD).

## BD-rate summary for HM3.0 with chroma rd cost cf. JM17.2.

Table 3 shows the summary of BD-rates of HM3.0 with chroma rd cost compared to JM17.2.

Table 3: HM3.0 with chroma rd cost compared to the JM17.2 alpha, beta and all intra anchors.

Note on Class A***:  NebutaFestival and SteamLocomotiveTrain are not included in this set of class A data.

The average Y BD-rate gain for all coding conditions is reduced by 0.8% to 31.2%.  However, the average U and V BD-rate gain for all coding conditions have improved from 25% and 24% to 32.3% and 31.6%, respectively.

## Subjective observation of HM3.0 and HM3.0 with chroma rd cost (HE-LD).

Subjective observations of the sequences at the lowest rate point show that the chroma quality is improved when the chroma RD cost is taken into account.  However it should be noted that chroma artifacts were reduced but not completely removed.  At low bit rates, chroma artifacts and colour bleeding are still observed.

# Conclusion

The results show that the large drop in U and V PSNR can be corrected by factoring the relative qp settings of the luma and chroma components into the RD cost computation.  This results in a more balanced gain in both the luma and chroma components when compared to JM17.2.

It is recommended that this be implemented into the HM encoder so that a better balance in the performance gains of both luma and chroma components, is achieved.

# Reference

- Thomas Wiegand, et. al., “WD3: Working Draft 3 of High-Efficiency Video Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document: JCTVC-E603, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11 Coding of Moving Pictures and Audio, “Joint Call for Proposals on Video Compression Technology”, ISO/IEC JTC1/SC29/WG11/N11113, Kyoto, 22 January 2010.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).