| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F387 |
| --- | --- |

| Title: | CE8.1: Cross-check result of Samsung adaptive loop filter (JCTVC-F301) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Yukinobu Yasugi | Tel: Email: | ikai.tomohiro@sharp.co.jp yasugi.yukinobu@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In this contribution, a CE8.1 cross-check result of the Block-based approach improvement by Samsung (JCTVC-F301) is presented. The proposal is 1/4 subsampling (1/2 horizontal and 1/2 vertical) base feature calculation in Block adaptation mode (BA mode) which aims at computation reduction. Our results perfectly match with those of proponents, which are 0.1%, 0.2%, and 0.2% loss in HE-AI, HE-RA, and HE-LB case respectively. The encoder time ratio is 99 % to 100 % and the decoding time ratio is 96 % to 99 %.

# feature computation simplification

As shown Figure 1, the proposal uses four points in each 4x4 block which is used an adaptation unit in Block adaptation mode (BA mode).

Figure 1 Calculation point of each 4x4 block

# Performance experiments

The experimental results are summarized in Table 1. Detailed results are included in the attachment (JCTVC-F387.xls).

Table 1 Experiment result by Cross-checker

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1 | 0.0 | 0.0 |
| Class B | 0.1 | 0.0 | 0.0 |
| Class C | 0.2 | 0.0 | 0.0 |
| Class D | 0.1 | -0.1 | -0.1 |
| Class E | 0.2 | -0.1 | -0.1 |
| Overall | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.2 | 0.1 | 0.2 |
| Class B | 0.3 | 0.0 | 0.1 |
| Class C | 0.2 | 0.1 | 0.0 |
| Class D | 0.1 | 0.0 | -0.2 |
| Class E |  |  |  |
| Overall | 0.2 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.2 | 0.2 | 0.0 |
| Class C | 0.2 | 0.0 | -0.1 |
| Class D | 0.1 | 0.0 | 0.2 |
| Class E | 0.3 | 0.1 | 0.0 |
| Overall | 0.2 | 0.1 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

# Conclusion

This contribution provides the cross-check result of the Block-based approach improvement by Samsung in the context of CE8 subtest 1 [JCTVC E708]. The proposal is 1/4 subsampling (1/2 horizontal and 1/2 vertical) base feature calculation in Block adaptation mode (BA mode) which aims at computation reduction. Our results perfectly match with those of proponents, which are 0.1%, 0.2%, and 0.2% loss in HE-AI, HE-RA, and HE-LB case respectively. The encoder time ratio is 99 % to 100 % and the decoding time ratio is 96 % to 99 %, which shows decoding time decrease.

Because there are other block-based approaches in CE8.1 aiming to enhance coding gain, it is recommended that it should be discussed among CE8 members for possible approach which achieves better performance-complexity point.

# Reference

- T.Yamakage, Y.W. Huang, I.S. Chong, T. Ikai, M. Narroschke, Wang L. Lai, Ali Tabatabai, Madhukar Budagavi, Faouzi Kossentini, “Description of CE8: Non-deblocking loop filtering”, JCTVC-E708, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

- P. Lai, F. C. A. Fernandes, E. Alshina, I.-K Kim, “CE8 Subtest 1: Block-based filter adaptation with features on subset of pixels” , JCTVC-F301, 6th JCT-VC Meeting, Torino, Italy, July 2011.