| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F388 |
| --- | --- |

| Title: | CE9: Cross-check report for SP03, SP06S1 and SP06S2 (JCTVC-F084, JCTVC-F088) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim | Email: | ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this document, the results of three tests (SP03, SP06S1 and SP06S2) of JCTVC-F084 and JCTVC-F088 are provided. SP03 of JCTVC-F084 is about fixing the ref_idx to zero for temporal motion vector prediction derivation of merge-skip mode. SP06S1 of JCTVC-F088 is about checking the motion vector predictors from same reference frame first and the other candidates are checked later. SP04 of JCTVC-F088 is about reducing the number of scaling for spatial motion vector predictors. Once the scaling is done for a spatial predictor, the scaling process will not be applied to the other spatial predictor processing. For SP03, average 0.1% loss without encoding and decoding time was obtained. Especially, average 0.3% loss was observed for low delay configurations. For SP06S1, average 0.0% loss without encoding and decoding time change was shown. For SP06S2, an averaged result of this test is 0.0% loss without encoding and decoding time change. All test results are matched with those of JCTVC-F084 and JCTVC-F088.

# Introduction

In this document, the results of three tests (SP03, SP06S1 and SP06S2) of JCTVC-F084 [2] and JCTVC-F088 [3] are provided. SP03 of JCTVC-F084 is about fixing the ref_idx to zero for temporal motion vector prediction derivation of merge-skip mode. SP06S1 of JCTVC-F088 is about checking the motion vector predictors from same reference frame first and the other candidates are checked later. SP04 of JCTVC-F088 is about reducing the number of scaling for spatial motion vector predictors. Once the scaling is done for a spatial predictor, the scaling process will not be applied to the other spatial predictor processing.

# Experimental results

The test are done under common test conditions [4]. Table 1 summarizes results of SP03. Average 0.1% loss without encoding and decoding time was obtained. Especially, average 0.3% loss was observed for low delay configurations.

Table 1 Test results of SP03

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | 0.0 | 0.0 | 0.0 | -0.1 | -0.2 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.2 | -0.3 | 0.3 | 0.8 | 0.9 |
| Class C | 0.1 | 0.2 | 0.1 | 0.1 | 0.5 | 0.4 |
| Class D | 0.3 | 0.1 | 0.6 | 0.4 | 1.1 | 1.1 |
| Class E | 0.2 | -0.1 | -0.1 | 0.2 | 1.4 | 0.9 |
| Overall | 0.2 | 0.1 | 0.1 | 0.3 | 0.9 | 0.9 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 2 summarizes results of SP06S1. The table shows average 0.0% loss without encoding and decoding time change.

Table 2 Test results of SP06S1

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | -0.1 |
| Class D | -0.1 | -0.4 | 0.5 | 0.0 | -0.2 | 0.0 |
| Class E | 0.0 | 0.2 | -0.3 | -0.1 | -0.4 | 0.4 |
| Overall | 0.0 | -0.1 | 0.1 | 0.0 | -0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 shows results of SP-6S2. An averaged result of this test is 0.0% loss without encoding and decoding time change.

Table 3 Test results of SP06S2

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | -0.1 | 0.1 | 0.1 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.4 | 0.0 | -0.1 | -0.2 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | -0.1 |
| Class D | -0.1 | -0.1 | 0.3 | 0.0 | 0.2 | 0.0 |
| Class E | 0.0 | 0.2 | 0.2 | 0.0 | -0.3 | 0.1 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

All test results are matched with those of JCTVC-F084 and JCTVC-F088.

# References

- Y.-W Huang, B. Bross, M. Zhou, W.-J Chien and I.-K Kim, “Description of Core Experiment 9: MV Coding and Skip/Merge Operations”, JCTVC-E709, 5th Meeting: Geneva, CH, 16-23 March, 2011

- Minhua Zhou, “CE9:  Evaluation results  on SP01, SP02, SP03 and SP14”, JCTVC-F084, 6th Meeting: Torino, IT, 14-22 July, 2011

- Minhua Zhou, Mahmut Sinangil, Vivienne Sze, Seungwook Park, Joonyoung Par and Byeongmoon Jeon, “CE9: Simplified AMVP design (SP06S1, SP06S2)”, JCTVC-F088, 6th Meeting: Torino, IT, 14-22 July, 2011

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011