| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F389 |
| --- | --- |

| Title: | Cross-check result of MediaTek's adaptive loop filter for LCU-based decoding (JCTVC-F054) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Yukinobu Yasugi | Tel: Email: | ikai.tomohiro@sharp.co.jp yasugi.yukinobu@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In this contribution, a cross-check result of Adaptive Loop Filter with Zero Pixel Line Buffers for LCU-based Decoding (JCTVC-F054 [1]) by MediaTek is presented. The proposal aims to remove line buffers in adaptive loop filter process of LCU-based decoding. This approach uses padding values instead of deblocking filtered values across a virtual boundary of which location is N pixels upper than LCU row boundary.

The proposal has several mode, Section 3.1 to Section 3.3, this cross-check test “Section 3.3 Virtual boundaries with unified filter shape and boundary smoothing”

The experiment results perfectly match with those of proponents. The decoding time, 101%, correspond to proponents, 100%. The encoding time is 94 % to 98 %, which is smaller than proponents, 99 % to 100 %. It is considered due to simulation environment differences. From the code review, it is confirmed the ALF method can reduce line buffers as the proponents proposes and properly implemented. From still subjective viewing for some sequences, it is reported that it’s difficult to notice the differences between anchor image and test image so that the subjective degrade is not detected. It is also reported that the virtual boundary line is not visible.

# The method

From the cross checker’s understandings, the proposed ALF’s reference samples ALF is shown in Figure 1.

Padding value is used for the reference values

ALF filtered without padding value

ALF filtered with some padding values

=>NO need to refer/wait/store below or upper region across the virtual boundary

virtual boundary

padding values

N=4

Figure 1 Reference samples

Additionally, a boundary smoothing technique is used. where the first neighboring lines around the virtual boundary are not filtered (skipped) and the second neighboring lines filtering is weakened with value’ = winear_filter(value) + value >> 1 operation.

And the technique uses 9x9 tap filter rather than 9x7 filter for luma filtering by TI_ALF_MAX_VSIZE_7    0 and the technique uses 7x7 diamond rather than 5x5 rectangle filter for chroma filtering.

# Performance experiments

The experimental results are summarized in Table 1. Detailed results are included in the attachment (JCTVC-F389.xls).

Table 1 Experiment result by Cross-checker

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1 | 0.1 | 0.1 |
| Class B | 0.1 | 0.2 | 0.1 |
| Class C | 0.1 | 0.1 | 0.2 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.2 | -0.1 | -0.2 |
| Overall | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.1 | 0.3 | 0.1 |
| Class B | 0.0 | -0.1 | -0.2 |
| Class C | 0.0 | -0.4 | -0.2 |
| Class D | -0.2 | -0.6 | -0.6 |
| Class E |  |  |  |
| Overall | 0.0 | -0.2 | -0.2 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.3 | -0.3 | -0.4 |
| Class C | -0.2 | -0.9 | -0.3 |
| Class D | -0.9 | -0.8 | -0.7 |
| Class E | 0.7 | 1.7 | 0.4 |
| Overall | -0.2 | -0.2 | -0.3 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 101% |  |  |

# Reference

C. Y. Chen, C. Y. Tsai, C. Fu, Y. W. Huang, S. Lei, “”Adaptive Loop Filter with Zero Pixel Line Buffers for LCU-based Decoding , JCTVC-F054, 6th JCT-VC Meeting, Torino, Italy, July 2011.