| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F390 WG11 Number: m20816 |
| --- | --- |

| Title: | CE5: Simplification of transform coefficient coding in CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 yasugi.yukinobu@sharp.co.jp, yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This document reports the simulation result of a run mode bypassing scheme for CAVLC (JCTVC-E156), in CE5. Bypassing run mode can simplify the coding process when the value of the last non-zero coefficient is more than one. It is reported that the simplified scheme does not impact on coding efficiency and computation time. The average of Y BD-rate shows no loss or a slight (~0.02%) gain in the coding efficiency.

# Technical description

## Run mode bypassing

The run mode bypassing (RMB) scheme for LCEC is proposed in JCTVC-E156[1] at the 5th meeting in Geneva. The RMB scheme refers the last non-zero coefficient and the type of a block to determine whether to use the run mode or not. When the level of the last non-zero coefficient is greater than one and the current block is an intra-coded luma block, this simplified process is applied.

Figure 1 shows a brief coding process with the run mode bypassing scheme.

This scheme simplifies the encoding and decoding process in the following points.

- Simplify the parsing process for run_level_one which transforming codeNum to level and run inside run mode.

- Reduce table entries associated with the case of 1).

Figure 1: Coding process of coefficients with run mode bypassing

## Text modification

### 7.3.11		Residual coding CAVLC syntax

| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| n = ( 1 << ( log2TrafoSize << 1 ) ) >> ( cIdx > 0 ? 2 : 0 ) |  |
| last_pos_level_one | ce(v) |
| lastPos = last_pos_level_one % n |  |
| levelGreaterThanOneFlag = ( last_pos_level_one > n ) |  |
| level_and_sign( levelGreaterThanOneFlag, transCoeffLevel, x0, y0, trafoDepth, cIdx, lastPos) |  |
| runModeFlag = TRUE |  |
| lumaIntraFlag = ( PredMode == MODE_INTRA && cIdx == 0) |  |
| runModeFlag = ( lumaIntraFlag && levelGreaterThanOneFlag ) ? FALSE: TRUE |  |
| n = lastPos – 1 |  |
| sumBigCoeff = 0 |  |
| switchThres = ( PredMode == MODE_INTRA && cIdx == 0)? 0 : 49 |  |
| switchThres = lumaIntraFlag? 0 : 49 |  |
| while (runModeFlag && n >= 0) { |  |
| run_level_one | ce(v) |
| runOfZeros = run_level_one % n |  |
| levelGreaterThanOneFlag = (run_level_one > n) |  |
| trOne = (trOne = = 0 ¦ ¦ levelGreaterThanOneFlag) ? 0 : Max(4, trOne + 1) |  |
| n = n – runOfZeros |  |
| if( n >= 0 ) { |  |
| level_and_sign( levelGreaterThanOneFlag, transCoeffLevel, x0, y0, trafoDepth, cIdx, n) |  |
| if ( levelGreaterThanOneFlag ) { |  |
| sumBigCoeff = sumBigCoeff + level |  |
| if ( log2TrafoSize==2 ¦ ¦ n > switchThres ¦ ¦ sumBigCoeff > 2 ) |  |
| runModeFlag = FALSE |  |
| } |  |
| } |  |
| n-- |  |
| } |  |
| while ( n >= 0 ) { |  |
| level | ce(v) |
| if ( level > 0 ) { |  |
| sign_flag | u(1) |
| sign = 1 – 2 * sign_flag |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n ] = level * sign |  |
| } |  |
| n-- |  |
| } |  |
| } |  |

### 9.2.4.3	Parsing process for run_level_one

if ( largeOnePos > 0 ) {
		if( codeNum < min(largeOnePos, maxRunIdx + 2 ) ) {
			levelGreaterThanOneFlag = 0
			runOfZeros = codeNum
		} else if( codeNum < (maxRunIdx << 1 ) + 4 – largeOnePos ) {
			if(( codeNum + largeOnePos ) & 1 ) {
				levelGreaterThanOneFlag = 0
				runOfZeros = (codeNum + largeOnePos - 1) >> 1
			} else {
				levelGreaterThanOneFlag = 1
				runOfZeros = (codeNum - largeOnePos)>>1	(9-1)
			}
		} else {
			levelGreaterThanOneFlag = 1
			runOfZeros = codeNum - maxRunIdx – 2
		}
	} else {
		if( codeNum & 1) {
			levelGreaterThanOneFlag = 0 
			runOfZeros = (codeNum - 1)>>1
		} else {
			runOfZeros = codeNum >> 1
			levelGreaterThanOneFlag = ((codeNum >> 1) <= maxRunIdx)?1:0
		}
	}

# Experimental results

The experiments were conducted following the Core Experiment Description (JCTVC-E705) [3]. The experimental results are summarized in Table 1, and detailed results are included in the attachment (JCTVC-F390.xls).

The experiments were carried out on our grid system composed of variety of PCs. The grid system was configured so that the same-performance PCs are used for the same test settings.

Table 1: Experimental Results

|  | LC_AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | 0.00 |  | 0.01 | 0.03 |  |
| Class B | 0.05 |  | 0.03 | 0.04 |  |
| Class C | -0.03 |  | -0.01 | -0.04 |  |
| Class D | -0.06 |  | -0.04 | -0.01 |  |
| Class E | 0.00 |  | 0.02 | -0.01 |  |
| All | -0.01 |  | 0.00 | 0.00 |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | LC_RA |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.06 |  | -0.28 | 0.17 |  |
| Class B | 0.02 |  | 0.02 | 0.11 |  |
| Class C | -0.02 |  | -0.08 | -0.05 |  |
| Class D | -0.02 |  | -0.20 | 0.04 |  |
| Class E |  |  |  |  |  |
| All | -0.02 |  | -0.12 | 0.07 |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | LC_LB |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |
| Class B | 0.02 |  | 0.12 | -0.08 |  |
| Class C | 0.00 |  | 0.08 | -0.36 |  |
| Class D | -0.02 |  | -0.05 | -0.20 |  |
| Class E | -0.01 |  | 0.28 | -0.88 |  |
| All | 0.00 |  | 0.10 | -0.33 |  |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |

## Usage rate of run mode bypassing

We also evaluated the frequency of the bypassing to check the usage rate of the simplification in decoding process. Table 2 shows the ratio of bypass-applied blocks for each size of luma intra blocks. The percentage in Table 2 is bypass block / total blocks at the size and class. For example, in total of LC_AI condition, the usage rate of the simplification is 10.7% at 4x4 Luma Intra blocks.

Table 2: Usage rate of run mode bypassing in luma intra blocks

|  |  | block size |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 4x4 | 8x8 | 16x16 | 32x32 | all |
| LC_AI | class A | 15.2% | 5.4% | 3.6% | 0.6% | 9.8% |
|  | class B | 8.7% | 5.8% | 3.9% | 1.4% | 7.4% |
|  | class C | 9.8% | 6.5% | 3.9% | 1.4% | 9.0% |
|  | class D | 10.0% | 6.8% | 3.8% | 1.0% | 9.3% |
|  | class E | 14.5% | 5.0% | 4.1% | 2.6% | 10.1% |
|  | ALL | 10.7% | 5.7% | 3.8% | 1.2% | 8.5% |
| LC_RA | class A | 17.0% | 5.6% | 4.6% | 0.6% | 10.0% |
|  | class B | 9.6% | 4.9% | 3.6% | 0.9% | 7.6% |
|  | class C | 10.2% | 6.4% | 3.3% | 0.8% | 9.2% |
|  | class D | 9.7% | 6.0% | 2.9% | 0.4% | 8.8% |
|  | class E | - | - | - | - | - |
|  | ALL | 11.6% | 5.3% | 4.1% | 0.6% | 8.7% |
| LC_LB | class A | - | - | - | - | - |
|  | class B | 15.5% | 6.9% | 4.3% | 1.0% | 11.4% |
|  | class C | 13.2% | 8.1% | 3.5% | 0.9% | 11.6% |
|  | class D | 11.9% | 7.3% | 2.9% | 0.6% | 10.5% |
|  | class E | 11.9% | 5.0% | 3.1% | 1.6% | 8.6% |
|  | ALL | 14.4% | 7.1% | 4.1% | 1.0% | 11.3% |

# Conclusion

In this document, the performance of the proposed simplification scheme by Run mode bypassing was presented. It is reported the scheme simplifies encoding and decoding processes, which is applied about 9 to 11 % transform coefficient coding process of luma-intra blocks on CAVLC, and removes unnecessary table entries. The experiment shows the scheme has no impact on the coding efficiency in the common condition. The encoding/decoding time ratio is 100%.

From this CE results, it is recommend that the proposed scheme to be adopted in the next HM.

# References

- Y. Yasugi and T. Yamamoto, “Simplification of transform coefficient coding for LCEC”, JCTVC-E156, 5th JCT-VC Meeting, Geneva, Switzerland, Mar. 2011

- HM 3.0 software:  http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-3.0

- Xianglin Wang, et al., “CAVLC Entropy Coding Improvements”, JCTVC-E705, 5th JCT-VC Meeting, Geneva, Switzerland, Mar. 2011

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).