| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F391 |
| --- | --- |

| Title: | CE9: Cross-check report for SP04 (JCTVC-F050) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Il-Koo Kim | Email: | ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this document, cross-check results of SP04 test in CE9, which is done in JCTVC-F050, are provided. In SP04, redundancy checking process during the top motion vector predictor derivation is removed in order to increase possibilities of parallel processing and also decrease the complexity in both encoder and decoder size. The redundancy check is done once after two spatial motion vector predictors and one temporal motion vector predictors are obtained. The software was provided by MediaTek and the test was done under common test condition of HEVC. Average loss of the test is about 0.0% with 1% encoding and decoding time decrease. These results are matched with those of JCTVC-F050.

# Introduction

In this document, cross-check results of SP04 test in CE9 [1], which is done in JCTVC-F050 [2], are provided. In SP04, redundancy checking process during the top motion vector predictor derivation is removed in order to increase possibilities of parallel processing and also decrease the complexity in both encoder and decoder size. The redundancy check is done once after two spatial motion vector predictors and one temporal motion vector predictors are obtained.

# Experimental results

The software was provided by MediaTek and the test was done under common test condition of HEVC [3]. Table 1 summarizes the results of SP04 in CE9. Average loss of the test is about 0.0% with 1% encoding and decoding time decrease. These results are matched with those of JCTVC-F050.

Table 1 Summarized test result of SP04 in CE9

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.2 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | -0.3 | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class D | -0.1 | 0.0 | 0.1 | -0.1 | -0.7 | -0.4 |
| Class E | 0.0 | 0.3 | 0.2 | 0.0 | 0.0 | 0.4 |
| Overall | 0.0 | 0.1 | 0.0 | 0.0 | -0.2 | -0.1 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# References

- Y.-W Huang, B. Bross, M. Zhou, W.-J Chien and I.-K Kim, “Description of Core Experiment 9: MV Coding and Skip/Merge Operations”, JCTVC-E709, 5th Meeting: Geneva, CH, 16-23 March, 2011

- Jian-Liang Lin, Yi-Wen Chen, Yu-Wen Huang, and Shawmin Lei, “CE9: Results of Experiment SP04”, JCTVC-F050, 6th Meeting: Torino, IT, 14-22 July, 2011

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011