| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F396r2 |
| --- | --- |

| Title: | Improvement of Sample Adaptive Offset with modified bit accuracy and restricted offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takanori Yamazaki Tomohiro Ikai Yukinobu Yasugi Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 yamazaki.takanori@sharp.co.jp, ikai.tomohiro@sharp.co.jp, yasugi.yukinobu@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In this contribution, a bit accuracy modification and offset range restriction in Sample Adaptive Offset (SAO) are proposed. The proposed bit accuracy is 10 bits rather than 9 bits in HM-3.0. The proposed offset range is 4, 5, or 6 bits depending on SAO bit accuracy. With the proposed offset range, memory usage can be reduced up to 40 % compared to HM-3.0 when internal bit depth is 10 bits. It is reported that the bit accuracy modification and offset range restriction have negligible effect on the coding efficiency in normal qp range (22-37) while it achieves up to 0.2 % gain in lower qp range (12-27).

# Introduction

In Geneva meeting, Sample Adaptive Offset (SAO) is adopted. Although SAO achieves good gain, it is thought there are two aspects needed to be further studied. First aspect is the bit accuracy of offset value (SAO bit accuracy). It is better to decide SAO bit accuracy that is effective to various configurations such as higher internal bit depth and wider range of QPs. Second aspect is offset range. Since the number of offset values could be enormous, the memory required to store offset value could become significant. By restricting offset range, such memory can be reduced.

# Proposal

## Proposal 1: bit accuracy modification

In order to remove distortion, higher SAO bit accuracy is preferable for higher internal bit depth. On the other hand, amount of side information increases when higher SAO bit accuracy is applied. Table 1 shows the proposed and conventional SAO bit accuracy. The proposed SAO bit accuracy is decided based on experiments, in which qp is varied from 12 to 37.

Table 1: bit accuracy (proposal 1)

| Internal bit depth [bit] | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SAO bit accuracy (HM-3.0) [bit] | 8 | 8 | 9 | 9 | 9 | 9 | 9 |
| SAO bit accuracy (Proposed) [bit] | 8 | 9 | 10 | 10 | 10 | 10 | 10 |

## Proposal 2: offset range restriction

In order to reduce memory size while keeping coding efficiency, it is proposed to restrict the offset range depending on SAO bit accuracy. Table 2 shows the proposed offset range. Offset range equal to 4 bit means that an offset value is ranging from -23 to 23-1.

Based on the proposed offset range, offset range is 6 bit when internal bit depth is 10 bit. In this case, the memory size can be reduced by 40% compared to HM-3.0. The memory size for storing offset could become 40,960 bits per picture when the number of blocks is 256, the number of offset in each block is 16, and the offset range is 10 bit in HM-3.0. It can be reduced to 24,576 bits by applying the proposed offset range.

Table 2: offset range restriction (Proposal 2)

| SAO bit accuracy [bit] | 8 | 9 | 10 |
| --- | --- | --- | --- |
| Proposed offset range [bit] | 4 | 5 | 6 |

# Experiment

The proposed SAO bit accuracy and offset range were implemented into HM-3.0 software. The experiment was carried out based on common test condition. In addition, the experiment with lower qp values was also carried out. The results are summarized in Table 3. In low complexity, the proposed SAO bit accuracy is the same as that of HM-3.0, and restriction is thought to have no effect on coding efficiency because in lower qp, offset values would be smaller. So the results of lower qp condition in low complexity are not included

In common test condition (qp 22-37), the impact on coding efficiency is negligible. In lower qp condition (qp 12-27), experiments result shows up to 0.2% gain. Further details of experimental result can be found in the attached documents: “JCTVC-F396_proposal_qp12-27.xls”, “JCTVC-F396_proposal_qp17-32.xls” and “JCTVC-F396_ proposal_qp22-37.xls”.

Table 3 Simulation results

|  | HE-AI |  |  | LC-AI |
| --- | --- | --- | --- | --- |
|  | Y BDRate |  |  | Y BDRate |
|  | qp12-27 | qp17-32 | qp22-37 | qp22-37 |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | -0.1 | 0.0 | 0.0 |
| Class D | -0.1 | -0.1 | 0.0 | 0.0 |
| Class E | -0.1 | -0.1 | 0.0 | 0.0 |
| Overall | -0.1 | -0.1 | 0.0 | 0.0 |
|  | HE-RA |  |  | LC-RA |
|  | Y BDRate |  |  | Y BDRate |
|  | qp12-27 | qp17-32 | qp22-37 | qp22-37 |
| Class A | -0.1 | -0.1 | -0.1 | 0.0 |
| Class B | -0.3 | -0.1 | 0.0 | 0.0 |
| Class C | -0.2 | -0.1 | 0.0 | 0.0 |
| Class D | -0.1 | 0.0 | 0.1 | 0.0 |
| Class E |  |  |  |  |
| Overall | -0.2 | -0.1 | 0.0 | 0.0 |
|  | HE-LB |  |  | LC-LB |
|  | Y BDRate |  |  | Y BDRate |
|  | qp12-27 | qp17-32 | qp22-37 | qp22-37 |
| Class A |  |  |  |  |
| Class B | -0.1 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | 0.1 | 0.0 |
| Class D | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E | -0.2 | -0.1 | 0.1 | 0.1 |
| Overall | -0.1 | 0.0 | 0.1 | 0.0 |

# WD Text changes

7.4.3.4 Sample adaptive offset parameter semantics

sao_offset[ saoDepth ][ x0 ][ y0 ][ i ] indicates the offset value of i-th category for the region specified by saoDepth, x0 and y0. The value shall be in the range of -1<<(SaoBitsRange-1) to (1<<(SaoBitsRange-1) – 1) where a value of SaoBitRange is derived as follows,

SaoBitRange = Min( BitDepthY, 10 ) - 4 		 	 	 	 	 	 (7-11)

The value of sao_offset_val, which is used in the sample adaptive filter process, is derived as follows

sao_offset_val[ saoDepth ][ x0 ][ y0 ][ i ] = sao_offset[ saoDepth ][ x0 ][ y0 ][ i ] << (BitDepthY - Min( BitDepthY, 10 )) 									 (7-12)

# Additional experiments

## Test of offset range restriction

The proposal consists of accuracy changes and restriction of range. An additional experiment was conducted to check the restriction of range’s effect to confirm the restriction has no coding loss. The test result it shown in Table 4. The detail is in the attached document: “JCTVC-F396_restrict_only_qp22-37.xls”.

Table 4 Results of offset range restriction

|  | HE-AI Y BDRate | LC-AI Y BDRate |
| --- | --- | --- |
|  | qp22-37 | qp22-37 |
| Class A | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 |
|  | HE-RA Y BDRate | LC-RA Y BDRate |
|  | qp22-37 | qp22-37 |
| Class A | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 |
| Class E |  |  |
| Overall | 0.0 | 0.0 |
|  | HE-LB Y BDRate | LC-LB Y BDRate |
|  | qp22-37 | qp22-37 |
| Class A |  |  |
| Class B | -0.1 | 0.0 |
| Class C | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 |
| Class E | 0.1 | 0.1 |
| Overall | 0.0 | 0.0 |

## Test of other internal bit depths

To check effectiveness in wide range of internal bit depth. We conducted the second additional test, where the internal bit depth is 8 bits or 12 bits. The results are shown in Table 5. The test is conducted in 49 frames for Class A and B and in 150 frames for Class C, D, and E.

The results show when the qp range is low, the proposal achieve up to 0.2 % gain in 12 bit as well as 10 bit. In the 8 bit case and normal qp range, the proposal doesn’t change the coding efficiency.

Table 5 Results of other internal bit depths (49 to 150 frames test)

|  | HE-AI Y BDRate |  |  |  |
| --- | --- | --- | --- | --- |
|  | Internal bit depth 8bit |  | Internal bit depth 12bit |  |
|  | qp12-27 | qp22-37 | qp12-27 | qp22-37 |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 |
|  | HE-RA Y BDRate |  |  |  |
|  | Internal bit depth 8bit |  | Internal bit depth 12bit |  |
|  | qp12-27 | qp22-37 | qp12-27 | qp22-37 |
| Class A | 0.1 | 0.0 | -0.1 | 0.0 |
| Class B | 0.0 | 0.0 | -0.2 | 0.0 |
| Class C | 0.0 | 0.0 | -0.2 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 |
| Class E |  |  |  |  |
| Overall | 0.0 | 0.0 | -0.2 | 0.0 |
|  | HE-LB Y BDRate |  |  |  |
|  | Internal bit depth 8bit |  | Internal bit depth 12bit |  |
|  | qp12-27 | qp22-37 | qp12-27 | qp22-37 |
| Class A |  |  |  |  |
| Class B | -0.1 | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | 0.0 | -0.1 | 0.1 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 |
| Class E | 0.0 | 0.0 | -0.2 | 0.0 |
| Overall | 0.0 | 0.0 | -0.1 | 0.0 |

# Conclusion

In this contribution, a bit accuracy modification and offset range restriction in Sample Adaptive Offset (SAO) are proposed. With the proposed restriction, memory usage can be reduced up to 40 % compared to HM-3.0 when internal bit depth is 10 bits. It is reported that the proposed modifications introduces negligible effect on the coding efficiency in normal qp range (22-37) while it achieves up to 0.2 % gain in lower qp range (12-27). Based on these results, it is recommended to adopt the proposed SAO bit accuracy and offset range in HM.

# Reference

[1] C.-M. Fu, C.-Y. Chen, Y.-W. Huang, and S. Lei, “CE13: Sample Adaptive Offset with LCU-Independent Decoding,” Document of Joint Collaborative Team on Video Coding, JCTVC-E049, Mar. 2011.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).