| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F397 |
| --- | --- |

| Title: | Weighted Prediction with Parameter Estimation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Seishi Takamura, Yukihiro Bandoh, Masaaki Matsumura, Shohei Matsuo, Kazuto Kamikura, Hirohisa Jozawa NTT Cyber Space laoratories 1-1 Hikarino-oka, Yokosuka, Kanagawa, 239-0847 Japan | Tel: +81-46-859-2371 Email: takamura.seishi@lab.ntt.co.jp |  |
| Source: | NTT Corporation |  |  |

_____________________________

# Abstract

This report summarizes the performance evaluation of weighted prediction tool using fade parameter estimation. For faded sequences, For low delay configuration, WP with given parameter (ground truth) provided 17.6-28.7% gain(Y), WP with estimated parameter provided 17.0-28.2% gain(Y). Encoding complexity was 100-128% while decoding complexity was 82-107%.

- Introduction

Triggered by the revisit of Weighted Prediction (WP) tool at the last Geneva meeting [1], the AhG 18 on WP was formed whose mandate includes:

- Develop software for support of weighted prediction (WP) in HEVC in the same way as in AVC for study toward support of WP capability.

- Discuss and develop analysis software for estimating weighting parameters for explicit WP.

This report studies one of the methods for weighting parameter estimation [2] for explicit WP using the software developed by the AhG.

- Weighting parameter estimation

A robust method for weighting parameter estimation, based on the global feature of each video frame, was proposed by Aoki and Miyamoto [2] which is as follows:

Let P(x,y,t) be the pixel value at position (x,y) of the frame at time t, M(t) be its mean; i.e, M(t) = x,yP(x,y,t) / x,y1, and V(t) be its L1-norm from the centroid; i.e., V(t) = x,y|P(x,y,t) − M(t)|. Then the scaling parameter w and offset parameter o, between the reference frame (at time tref) and the frame at time t, are estimated as w = V(t) / V(tref) and o = M(t) – w*M(tref).

# Experimental results

An HM3.0-based software which supports WP (AHG18_HM3.0_WP_20110627_v6), as well as the faded sequences were provided by AhG 18.  For the detail of the WP syntax and fade generation, please refer to [3]. For parameter estimation, it took less than 4 seconds for every sequence, which is negligible compared to the encoding time (see JCTVC-F397_Estimation_time.xls). In the following subsections, ideal results with given parameter (ground truth) and results with estimated parameter are shown.

## Black Fade

## White Fade

# Conclusions

For faded sequences, WP with given parameter provided 17.6-28.7% gain(Y), WP with estimated parameter provided 17.0-28.2% gain(Y). Encoding complexity was 100-128% while decoding was 82-107%. These results demonstrate that WP’s performance is promising and also that this particular estimation method is practical enough.

# References

- Philippe Bordes, “Weighted Prediction,” JCT-VC Document, JCTVC-E041, Geneva, March 2011.

- Hirofumi Aoki and Yoshihiro Miyamoto, “An H.264 weighted prediction parameter estimation method for fade effects in video scenes,” Proc. ICIP2008, pp. 2112-2115, October 2008

- Philippe Bordes, “Weighted Prediction,” JCT-VC Document, JCTVC-F265, Turin, July 2011

# Patent rights declaration(s)

NTT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).