| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F399 |
| --- | --- |

| Title: | Restart of CABAC after coding of ALF and SAO slice header data |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matthias Narroschke Thomas Wedi Semih Esenlik Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com Thomas.Wedi@eu.panasonic.com Semih.Esenlik@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

Typically, ALF and SAO control parameters are estimated after the determination of the syntax elements to code the blocks of a slice. In order to allow decoders a convenient block-by-block decoding including the application of ALF and SAO, the control parameters of ALF and SAO are coded before the slice block data within the slice header. In HM3.2, the slice block data is coded dependently on the ALF/SAO control parameters in one arithmetic codeword. Thus, the encoder needs to buffer all syntax elements of the slice block data until the ALF and SAO control parameters are estimated and coded. Afterwards, the buffered syntax elements are coded. Thus, the encoding is associated with the two problems: Large buffer and delay. In order to overcome these two problems, it is proposed to restart CABAC after the coding of the ALF/SAO control parameters by the use of a cabac_restart_flag, which is coded in the terminating mode within the slice header. By a restart of CABAC, all block data of a slice is coded independently from the ALF/SAO control parameters. As a consequence, the syntax elements of slice block data can be coded without delay and without being buffered until the ALF/SAO control parameters are estimated and coded. The influences on the coding efficiency and on the encoder and decoder run times are negligible. It is proposed to adopt the proposed technique into the next version of the HM.

# Introduction

HM3.2 applies an adaptive loop filter (ALF) and sample adaptive offsets (SAO) as noise reduction techniques. Both, ALF and SAO, are controlled by parameters, which are estimated for each slice on the encoder side. One typical criterion minimized in the estimation process is the mean squared reconstruction error. For an optimal estimation of these control parameters, each of ALF and SAO requires the signal it operates on and the original input image. The signals, on which SAO and ALF operate on, are available after the encoding and internal decoding of all blocks of a whole slice. In order to allow a block-by-block decoding on the decoder side including the application of SAO and ALF, the control parameters of ALF and SAO are coded within the slice header and thus before the slice block data as illustrated in Figure 1. In the case of CABAC, the ALF and SAO control parameters and the slice block data are coded by one arithmetic codeword. Thus, the slice block data is coded dependently on the ALF and SAO data. This dependency leads to the following two problems at the encoder side:

- Delay: The encoding of all syntax elements of slice block data needs to be delayed until the ALF/SAO control parameters are estimated and encoded.

- Buffer: A large buffer is required to store the syntax elements of all slice block data until the ALF/SAO control parameters are estimated and encoded.

Figure 1: Coding of slice data according to HM3.0 [1].

# Restart of CABAC after coding of ALF/SAO slice header data

In order to overcome the two mentioned problems, the slice block data and the ALF/SAO control parameters are coded independently, achieved by a restart of CABAC after the coding of ALF and SAO data as shown in Figure 2. In this case, the syntax elements of all slice block data can be coded without delay. Thus it is not necessary to store the syntax elements of the block data of the whole slice and encode them with a delay. However, it is still necessary to store the arithmetic codeword of the slice block data.

Figure 2: Proposed coding of slice data.

# Syntax

A cabac_restart_flag is introduced in the slice header, which indicates the restart of CABAC to the decoder. This flag is coded using the termination mode in the same way as the end_of_slice_flag and the I_PCM flag. By the use of this flag, the encoder can select if CABAC is restarted or not. In the following table, the modified slice header syntax is shown. The introduced changes compared to the current HEVC working draft JCTVC-E603_d7 [2] are highlighted in green.

| slice_header( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| … |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if( entropy_coding_mode_flag) |  |
| cabac_restart_flag | ae(v) |
| } else |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| } |  |

# Experiments and results

The mechanism to restart CABAC using the introduced cabac_restart_flag has been integrated into the reference software of HM3.2 [1]. Two experiments have been performed. In the first experiment, the encoder selects no restart of CABAC for all slices, which means the encoder sets cabac_restart_flag=0. Figure 3 shows the measured BD-bit rates and the run time ratios. It can be seen that the bit rates as well as the run times stay unchanged.

Figure 3: Measured BD-bit rates and run time ratios compared to the reference HM3.2 [1] in the case that the encoder selects no restart of CABAC after the encoding of the ALF/SAO control parameters in the slice header.

In the second experiment, the encoder selects a restart of CABAC for all slices, which means the encoder sets cabac_restart_flag=1. Figure 4 shows the measured BD-bit rates and the run time ratios. In average over all test cases, the bit rate increases only marginally by 0.03% and the run times stay unchanged.

Figure 4: Measured BD-bit rates and run time ratios compared to the reference HM3.2 [1] in the case that the encoder selects a restart of CABAC after the encoding of the ALF/SAO control parameters in the slice header.

# Conclusion

Typically, ALF and SAO control parameters are estimated after the determination of the syntax elements to code the blocks of a slice. In order to allow decoders a convenient block-by-block decoding including the application of ALF and SAO, the control parameters of ALF and SAO are coded before the slice block data within the slice header. In HM3.2, the slice block data is coded dependently on the ALF and SAO control parameters in one arithmetic codeword. Thus, the encoder needs to buffer all syntax elements of the slice block data until the ALF and SAO control parameters are estimated and coded. Afterwards, the buffered syntax elements are coded. Thus, the encoding is associated with the two problems: Large buffer and delay. In order to overcome the problems, it is proposed to restart CABAC after the coding of the ALF/SAO control parameters by the use of a cabac_restart_flag, which is coded in the terminating mode within the slice header. By a restart of CABAC, all block data of a slice is coded independently from the ALF/SAO control parameters. As a consequence, the syntax elements of slice block data can be coded without delay and without being buffered until the ALF/SAO control parameters are estimated and coded. The influences on the coding efficiency and on the encoder and decoder run times are negligible. It is proposed to adopt the proposed technique into the next version of the HM.

# References

- HM3.2 software: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-3.2.

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan, WD3: Working Draft 3 of High-Efficiency Video Coding, JCTVC-E603, Geneva, CH, March 2011.

- JCT-VC, Test Model under Consideration, JCTVC-B205_draft007, Geneva, Switzerland, 21-28 July 2010.

- A. Norkin, K. Andersson, K. Chono, I. S. Chong, M. Narroschke, B. Jeon, D. Flynn, T. Suzuki, BoG report on subjective viewing test for deblocking filter proposals, JCTVC-E501, Geneva, CH, March 2011

- A. Norkin, Xun Guo, B. Jeon, M. Narroschke, Description of Core Experiment 12 (CE12): Deblocking filtering, JCTVC-E712, Geneva, CH, March 2011.

- M. Narroschke, T. Wedi, Decisions for deblocking, JCTVC-E251, Geneva, CH, March 2011.

- F. Bossen, Common test conditions and software reference configurations, JCTVC-E700, Geneva, CH, March 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).