| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F400 |
| --- | --- |

| Title: | CE4 Subtest 2: Delta QP prediction results of test 2.3.g and 2.3.f combined |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Kobayashi, Masato Shima 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: Email: | +81-3-3758-2111 kobayashi.masaaki517@canon.co.jp shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of the combination of QP prediction based on intra prediction (2.3.g of CE4 Subetst2) and prediction mode dependent QP prediction (2.3.f of CE4 Subtest2).  The experimental results show that the proposed QP prediction combination achieves 0.5% BD-rate reduction for AI_HE configuration, 0.4% BD-rate reduction for AI_LC configuration, 0.3% BD-rate reduction for RA_HE, RA_LC, LB_LC configurations, 0.2% BD-rate reduction for LD_HE configuration under the test condition defined in CE4 Subtest2.

# Introduction

In CE4 Subtest2, it was reported that the performance of the QP prediction based on intra prediction (2.3.g) was the best for AI_HE and AI_LC configurations under the test condition for CE4 Subtest2.  However, there were several other tools performing better for RA and LB configurations because the QP prediction based on intra prediction improves only QP coding of intra-coded CUs.  Accordingly, it was agreed within CE4 that the combination of QP prediction based on intra prediction and other QP prediction tools.  This contribution reports the experimental results of the combination of QP prediction based on intra prediction (2.3.g) and prediction mode dependent QP prediction (2.3.f).

# Algorithm description of combined QP prediction method

Tested combined QP prediction method is described here.

## Test 2.3.g: QP prediction based on intra prediction

With this QP prediction method, predicted QP is calculated from the QP of one of the neighboring CUs indicated by IntraPredMode.  Figure 1 shows position of neighboring CUs to be referenced for calculating delta QP.  The calculating method and neighboring CU are specified by the value of IntraPredMode shown in Figure 2.  Figure 2 shows intra prediction direction indicated by IntraPredMode.  This method is applicable when "PredMode" is intra. If "PredMode" is not intra, predicted QP is calculated with the prediction mode dependent QP prediction method (2.3.f) to be described in the next chapter.

The detailed description of QP prediction is shown as follows:

(1) If IntraPredMode is 3, 18, 10, 19, 4, 20, 11, 21, 0, 22, 12, 23, 5, 24, 13, 25, or 6,
	predicted QP is calculated from the QP of top CU.

(2) If IntraPredMode is 26, 14, 27, 7, 28, 15, 29, 1, 30, 16, 31, 8, 32, 17, 33, or 9,
	predicted QP is calculated from the QP left CU.

(3) Otherwise
	predicted QP is calculated with the prediction mode dependent QP prediction method (2.3.f) to be described in the next chapter.

Figure 1: Delta QP calculation by intra prediction

Figure 2: Intra prediction direction indicated by IntraPredMode

## Test 2.3.f: Prediction mode dependent QP prediction

With this QP prediction method, predicted QP is calculated from QPs of left and top CUs with the same “PredMode” as the current CU, where “PredMode” indicates whether the CU is coded with inter, intra, or other modes.

The detailed description of calculating predicted QP is shown as follows:

(1) If "PredMode" of current CU is the same as "PredMode" of left CU but not the same as “PredMode” of top CU,
	predicted QP is calculated as the QP of left CU.

(2) If "PredMode" of current CU is the same as "PredMode" of top CU but not the same as “PredMode” of left CU,
	predicted QP is calculated as the QP of top CU.

(3) If "PredMode" of left CU and top CU are same and "PredMode" of current CU is the same as them,
	predicted QP is calculated as the average of QPs of left CU and top CU.

(4) If "PredMode" of current CU is different from "PredMode" of left CU or top CU.
	predicted QP is calculated as the QP of left CU.

# Experiment

Experimental results on the combination of Test 2.3.g and Test 2.3.f defined in the CE4 description document (JCTVC-E704) are shown here.

## Experimental results on Test 2.3.g and 2.3.f combined

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | -0.3 | -0.2 | -7.9% | -0.2 | -0.2 | -0.2 | -7.0% |
| Class B | -0.4 | -0.4 | -0.4 | -11.4% | -0.3 | -0.3 | -0.3 | -10.1% |
| Class C | -0.5 | -0.5 | -0.5 | -13.4% | -0.5 | -0.5 | -0.5 | -12.5% |
| Class D | -0.4 | -0.5 | -0.4 | -12.7% | -0.4 | -0.4 | -0.4 | -12.6% |
| Class E | -0.8 | -0.9 | -0.8 | -17.8% | -0.7 | -0.7 | -0.7 | -15.5% |
| All | -0.5 | -0.5 | -0.4 | -12.3% | -0.4 | -0.4 | -0.4 | -11.3% |
| Enc Time[%] | 102% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | 0.0 | -0.2 | -4.4% | -0.1 | -0.1 | -0.1 | -4.0% |
| Class B | -0.2 | -0.2 | -0.2 | -6.5% | -0.2 | -0.3 | -0.2 | -6.4% |
| Class C | -0.3 | -0.3 | -0.3 | -9.4% | -0.4 | -0.3 | -0.3 | -9.9% |
| Class D | -0.2 | -0.1 | -0.2 | -7.9% | -0.4 | -0.3 | -0.4 | -8.7% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.3 | -0.1 | -0.2 | -7.0% | -0.3 | -0.3 | -0.3 | -7.2% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -0.2 | -0.4 | -4.5% | -0.2 | -0.1 | -0.3 | -4.3% |
| Class C | -0.3 | -0.4 | -0.4 | -7.7% | -0.4 | -0.2 | -0.4 | -8.3% |
| Class D | -0.3 | -0.5 | -0.1 | -6.5% | -0.4 | -0.2 | -0.8 | -7.2% |
| Class E | -0.3 | -0.3 | 0.4 | -8.6% | -0.4 | -0.3 | -0.4 | -8.1% |
| All | -0.2 | -0.4 | -0.2 | -6.6% | -0.3 | -0.2 | -0.5 | -6.8% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |

# Conclusion

This contribution reports the experimental results of QP prediction based on intra prediction technique. The experimental results show that the proposed combined QP prediction method achieves 0.5% BD-rate reduction for AI_HE configuration, 0.4% BD-rate reduction for AI_LC configuration, 0.3% BD-rate reduction for RA_HE, RA_LC, LB_LC configurations, 0.2% BD-rate reduction for LD_HE configuration under the test condition defined in CE4 Subtest2.  Therefore, it is recommended that the presented combined QP prediction be incorporated into the HM.

# Patent rights declaration(s)

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 3

The modified or added portions are highlighted with yellow colour.

### A.1. Change of subclause 7.4.9 “Transform coefficient semantics”

cu_qp_delta can change the value of QPY specifies the difference between a luma quantisation parameter to be used and its prediction, for a quantisation group of coding units where the quantisation group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantisation group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantisation group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantisation group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)

### A.2. Insertion of new subclause into clause 8.5

### 8.5.x	Derivation process for luma quantisation parameters

Inputs to this process are

–	a variable QpBdOffsetY,

–	a syntax element cu_qp_delta specifying the delta luma quantisation parameter of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Outputs of this process are

–	luma quantisation parameters QPY and QP’Y.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	Quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED.

With QPY,PRED, the value of QPY is derived as

QPY = ( ( ( QPY,PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(8-3)

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-4)

### 8.5.x.1		Quantisation parameter prediction process

Inputs to this process are

–	a variable intraPCMFlag specifying if the top-left coding unit of the current quantisation group of coding units is coded in PCM mode

–	a variable intraPredMode specifying the intra prediction mode of the top-left prediction unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The rearranged intra prediction order intraPredOrder is derived as follows.

–	If one of the following conditions is true, intraPredOrder is set to 0.

- intraPCMFlag is equal to 1.

- intraPredMode is equal to 2 (Intra_DC).

–	Otherwise, if intraPredMode is equal to 0 (Intra_Vertical), intraPredOrder is set to 9.

–	Otherwise, if intraPredMode is equal to 1 (Intra_Horizontal), intraPredOrder is set to 25.

–	Otherwise, intraPredOrder is derived as specified in Table 8-6.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If intraPredOrder is more than 0 and less than 18, and the above neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if intraPredOrder is equal to or more than 18, and the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if all of the following conditions are true, then QPY, PRED is derived as follows

- The above neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the bottom-left coding unit of the above neighbouring quantisation group of coding units is the same as PredMode.

- The left neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the top-right coding unit of the left neighbouring quantisation group of coding units is the same as PredMode.

QPY,PRED = ( ( ( QPY,ABOVE + QPY,LEFT + 1 ) / 2               	(8-53)

where QPY,ABOVE is luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice, and QPY,LEFT is luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the above neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the bottom-left coding unit of the above neighbouring quantisation group of coding units is the same as PredMode, then QPY, PRED is set equal to the luma quantisation parameter of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the left neighbouring quantisation group of coding units in the current slice is available and the prediction mode of the top-right coding unit of the left neighbouring quantisation group of coding units is the same as PredMode, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, QPY, PRED is set equal to QPY, PREV, where QPY,PREV is the luma quantisation parameter of the previous quantisation group of coding units in decoding order in the current slice. For the first quantisation group of coding units in the slice, QPY,PREV is initially set equal to SliceQPY at the start of each slice.