| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F405 |
| --- | --- |

| Title: | Deblocking filter using adaptive weighting factors |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matthias Narroschke Ann-Kathrin Seifert Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com Ann-Kathrin.Seifert@panasonic.de |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, adaptive weighting factors are introduced into the deblocking filter. Each weighting factor weights the value, by which the HM3.0 deblocking filter would modify a sample value. The weighting factors are estimated at the encoder side by minimization of the mean squared error between the deblocked signal and the original input signal. The encoded weighting factors are coded and transmitted to the receiver side. An average BD-bit rate reduction of 1.1% (I-HE: 1.6%, I-LC: 1.2%, RA-HE: 1.2%, RA-LC: 0.9%, LD-HE(B): 1.0%, LD-LC(B): 0.6%, LD-HE(P): 0.9%, LD-LC(P): 0.4%) is achieved compared to HM3.0 with approximately no encoder/decoder run time increases. Combined with the individual decisions and the evaluation of the second derivation at the block boundary in the decision process as proposed in JCTVC-F191, the average bit rate reduction is increased to 1.2% (I-HE: 1.6%, I-LC: 1.1%, RA-HE: 1.2%, RA-LC: 0.8%, LD-HE(B): 1.5%, LD-LC(B): 0.8%, LD-HE(P): 1.3%, LD-LC(P): 0.4%). It is proposed to evaluate it within the next CE on deblocking filters.

# Introduction

The current HM 3.0 [1][2] applies hybrid coding. The generalized block diagram of the hybrid encoder is shown in Figure 1. In a first step, the input signal to be coded is predicted block-wise by either motion compensated prediction or Intra prediction. The resulting prediction error is block-wise transform coded by applying an approximation of the discrete cosine transform, also known as Integer DCT, followed by a quantization of the coefficients. Due to the block wise motion compensated prediction and a block wise prediction error coding, so called blocking artifacts often become visible in the decoded images. These blocking artifacts tend to be annoying for human observers. In order to reduce these annoying blocking artifacts, a deblocking filter is applied. The deblocked signal is further filtered by the use of sample adaptive offsets (SAO) and an adaptive loop filter (ALF) before being output and stored for further predictions. Both, SAO and ALF estimate control parameters based on the original input signal s and the signal, these filters operate on, which is s'' for SAO and s for ALF. This enables SAO and ALF to adapt to varying statistics of the input signals. The control parameters are coded and transmitted to the receiver. Figure 2 illustrates the signal before and after the deblocking filter for a region of the example test sequence Kimono. In the next Section 2, the deblocking filter of HM3.0 is described in detail. Section 3 presents the proposed deblocking filter using adaptive weighting factors, Section 4 the experiments and results. Conclusions are drawn in Section 5.

Figure 1: Generalized block diagram of a hybrid encoder according to HM3.0 [1].

Figure 2: Illustration of the signal before and after the deblocking filter for a region of the example test sequence Kimono.

# Deblocking filter according to HM3.0

Deblocking of images is performed based on an edge segments of 8 consecutive lines or columns. The respective sides of an edge segment are denoted as A and B. The segment A corresponds to the left neighboring partition to B for vertical edges and to the above neighboring partition to B for horizontal edges. The samples on the respective sides of an edge segment are denoted as also shown in Figure 3.

Figure 3: Notation of a segment of a vertical edge for deblocking (from [3]).

For each line and column of the luminance signal, it is decided based on various conditions, if a strong filter, a weak filter, or no filter is applied. Details on the conditions can be found in [1][2]. The filter operations of the strong and the weak filter are shown in the subsequent Sections 2.1 and 2.2.

For the chrominance signal, the decision for each line and column is just to decide if a filter is applied or not. In the case of a positive decision, the chrominance filter shown in Section 2.3 is applied.

## Strong luminance filter

p0i'=Clipp2i+2⋅p1i+2⋅p0i+2⋅q0i+q1i+4>>3

q0i'=Clipp1i+2⋅p0i+2⋅q0i+2⋅q1i+q2i+4>>3

p1i'=Clipp2i+p1i+p0i+q0i+2>>2

q1i'=Clipp0i+q0i+q1i+q2i+2>>2

p2i'=Clip2⋅p3i+3⋅p2i+p1i+p0i+q0i+4>>3

q2i'=Clipp0i+q0i+q1i+3⋅q2i+2⋅q3i+4>>3

Herby, the clipping Clipx function is defined by

Clipx=0;x<0max_value;x>max_valuex;else.

The operator “>>x” defines a bit-shift operation by xbits to the right.

## Weak luminance filter

Δweak=Clip3−tc,tc,13⋅q0i−p0i+4⋅q1i−p1i−5⋅q2i−p2i+16>>5

p0i'=Clipp0i+Δweak

q0i'=Clipq0i−Δweak

p1i'=Clipp1i+Δweak2

q1i'=Clipq1i−Δweak2

Herby, the clipping function Clip3a,b,x is defined by

Clip3a,b,x=a;x<ab;x>bx;else.

## Chrominance filter

Δchroma=Clip3−tc,tc,q0i−p0i<<2+p1i−q1i+4>>3

p0i'=Clipp0i+Δchroma

q0i'=Clipq0i−Δchroma

# Deblocking filter using adaptive weighting factors

The deblocking of a sample sk' according to HM3.0 can be written in the form

sk''=sk'+Δk.

Hereby, Δk is the value by which the deblocking filter modifies a sample sk'. The value Δk results from a filter operation in combination with a clipping operation. In the case of the weak luminance filter and in the case of the chrominance filter, these modification values are Δweak and Δchroma, see Section 2.2 and Section 2.3.

In order to enable an adaptation of the deblocking filter to varying signal statistics with a minor increase of computational expense, adaptive weighting factors are introduced into the deblocking filter, which weight the values Δk by an adaptive weighting factor α:

sk''=sk'+α⋅Δk.

Individual weighting factors are used for the luminance and chrominance components, for each sample position, for the strong and weak filter as well as for the horizontal and vertical filter operations. Thus, in total, 28 weights are used for the luminance signal and 4 weights for each chrominance signal.

All weighting factors are estimated on the encoder side based on the original input signal s and the reconstructed signal s', on which the deblocking filter operates on. The estimated weighting factors are quantized, coded and transmitted to the decoder, see Figure 4.

Figure 4: Generalized block diagram of a hybrid encoder according to HM3.0 [1], extended by the proposed deblocking filter using adaptive weighting factors.

In the following three Sections, the filter operations, which include the quantized weighting factors, are shown in detail.

## Strong luminance filter

Δp0,i=Clipp2i+2⋅p1i−6⋅p0i+2⋅q0i+q1i

Δq0,i=Clipp1i+2⋅p0i−6⋅q0i+2⋅q1i+q2i

Δp1,i=Clipp2i−3⋅p1i+p0i+q0i

Δq1,i=Clipp0i+q0i−3⋅q1i+q2i

Δp2,i=Clip2⋅p3i−5⋅p2i+p1i+p0i+q0i

Δq2,i=Clipp0i+q0i+q1i−5⋅q2i+2⋅q3i

Xi'=ClipXi<<6+αs,X⋅ΔXi+32>>6    with    X=p2,p1,p0,q0,q1,q2

## Weak luminance filter

Δ=Clip3−tc,tc,13⋅q0i−p0i+4⋅q1i−p1i−5⋅q2i−p2i+16>>5

p0i'=Clipp0i<<6+αw,p0⋅Δ+32>>6

q0i'=Clipq0i<<6−αw,q0⋅Δ+32>>6

p1i'=Clipp1i<<6+Δ2⋅αw,p1+32>>6

q1i'=Clipq1i<<6−Δ2⋅αw,q1+32>>6

## Chrominance filter

Δ=Clip3−tc<<1,tc<<1,q0i−p0i<<2+p1i−q1i+4>>2

p0i'=Clipp0i<<6+αuv,p0⋅Δ+32>>6

q0i'=Clipq0i<<6−αuv,q0⋅Δ+32>>6

## Estimation of weights

Each weight is estimated by minimization of the mean squared reconstruction error between the original input signal s and the output signal s'' of the deblocking filter:

Es−s''2=Es−s'+α⋅Δ2→min

## Coding of weights

All estimated weights α are quantized using a quantization step size of 1/64. The quantized weights are coded within the slice header using prediction and variable length coding of the prediction error. No arithmetic coding is applied.

# Experiments and results

The deblocking filter using adaptive weighting factors has been integrated into the reference software of HM3.0 [1].

Following the common conditions [7], the performance is evaluated for all six mandatory as well as for the two optional test cases, which is Intra, Random access, Low delay B, and Low delay P, each in high efficiency and low complexity operation mode. For all run time measurements, computers of the same configuration are used.

The BD-rate results as well as the encoder-/decoder run time ratios compared to the reference HM3.0 [1] are shown in Figure 5. Negative BD-rate numbers show a gain compared to the reference. Run-time ratios less than 100% show reflect that the run time is lower than the one of the reference. Compared to HM3.0, the average bit rate reduction over all test sequences and configurations is 1.1%, the encoder-/decoder run times do not change.

Figure 5: BD-bit rates and run time ratios of the deblocking filter using adaptive weighting factors compared to the reference HM3.0 [1].

In addition, the performance of the deblocking filter using adaptive weighting factors is tested in combination with individual decisions and the evaluation of the gradient change at the block boundary proposed in [8]. The BD-rate results as well as the encoder-/decoder run time ratios compared to the reference HM3.0 [1] are shown in Figure 6. Negative BD-rate numbers show a gain compared to the reference. Run-time ratios less than 100% show reflect that the run time is lower than the one of the reference. Compared to HM3.0, the average bit rate reduction over all test sequences and configurations is 1.2%, the encoder-/decoder run times do not change.

Figure 6: BD-bit rates and run time ratios of the deblocking filter using adaptive weighting factors in combination with the individual decisions and the evaluation of the second derivation at the block boundary proposed in JVCVC-F191 [8] compared to the reference HM3.0 [1].

# Conclusion

In this contribution, adaptive weighting factors are introduced into the deblocking filter. Each weighting factor weights the modification value, by which the HM3.0 deblocking filter would modify a sample value. The weighting factors are estimated by minimization of the mean squared error between the deblocked signal and the original input signal. An average BD-bit rate of 1.1% (I-HE: 1.6%, I-LC: 1.2%, RA-HE: 1.2%, RA-LC: 0.9%, LD-HE(B): 1.0%, LD-LC(B): 0.6%, LD-HE(P): 0.9%, LD-LC(P): 0.4%) is achieved compared to HM3.0 with approximately no encoder/decoder run time increases. Combined with the individual decisions and the evaluation of the second derivation at the block boundary in the decision process as proposed in JCTVC-F191, the average bit rate reduction is increased to 1.2% (I-HE: 1.6%, I-LC: 1.1%, RA-HE: 1.2%, RA-LC: 0.8%, LD-HE(B): 1.5%, LD-LC(B): 0.8%, LD-HE(P): 1.3%, LD-LC(P): 0.4%). It is proposed to evaluate it within the next CE on deblocking filters.

# References

- HM3.0 software: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-3.0.

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan, WD3: Working Draft 3 of High-Efficiency Video Coding, JCTVC-E603, Geneva, CH, March 2011.

- JCT-VC, Test Model under Consideration, JCTVC-B205_draft007, Geneva, Switzerland, 21-28 July 2010.

- A. Norkin, K. Andersson, K. Chono, I. S. Chong, M. Narroschke, B. Jeon, D. Flynn, T. Suzuki, BoG report on subjective viewing test for deblocking filter proposals, JCTVC-E501, Geneva, CH, March 2011

- A. Norkin, Xun Guo, B. Jeon, M. Narroschke, Description of Core Experiment 12 (CE12): Deblocking filtering, JCTVC-E712, Geneva, CH, March 2011.

- M. Narroschke, T. Wedi, Decisions for deblocking, JCTVC-E251, Geneva, CH, March 2011.

- F. Bossen, Common test conditions and software reference configurations, JCTVC-E700, Geneva, CH, March 2011.

- M. Narroschke, T. Wedi, CE12: results for decisions for deblocking, JCTVC-F191, Turino, IT, July 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).