| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F408-r2 |
| --- | --- |

| Title: | CE5: Run and level mode coding improvement in CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE report |  |  |
| Author(s) or Contact(s): | Seongwan Kim, Jaeho Lee, Sangyoun Lee Yonsei University Jianle Chen, Jeonghoon Park Samsung | Tel: Email: Tel: Email: | +82-2-2123-5768 knauer@yonsei.ac.kr jhlee82@yonsei.ac.kr syleee@yonsei.ac.kr +82-31-279-8826 jianle.chen@samsung.com jeonghoon@samsung.com |
| Source: | Yonsei University and Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

In this contribution, the experimental results of CAVLC improvements proposed in JCTVC-E446 are reported. For more performance improvement, modifications are performed and additional techniques are considered. The simulation results are -0.7%, 0.0%, -0.4% in all intra, low delay and random access configurations, with reducing 540 bytes.

# Introduction

In current HM, CAVLC is composed of three parts, last level position coding, run mode coding and level coding. In run mode, code number, cn is mapped to {lev, run} pairs, using formula for INTRA and run-level table for INTER. In level mode, the magnitude of level value and sign are coded, with vlcNum depend on max coded coefficient level value.

# Proposal

## Intra code table modification

For INTRA, one parameter table is added for considering 8x8 and larger sizes (16x16 and 32x32), separately, in Table 1. vlcNum table proposed in [1] is removed to avoid memory increment.

Table 1 . Reference table size

|  | Parameter table size for INTRA |  |
| --- | --- | --- |
| trSize | HM 3.0 | Proposal |
| 4x4 | 5x15 | 5x15 |
| 8x8 | 5x29 (same table) | 5x29 |
| ≥ 16x16 |  | 5x29 |

## Level coding method

In level mode, vlcNum is determined, according to previous coded max level value in current block. In other word, if previous coded max value is more than thresholds, 4, 6, 14 and 28, the vlcNum is determined as (previous vlcNum + 1).

In our approach, thresholds for determining vlcNum are defined, according to features, I, P, B and Chroma. The selected threshold is biased depending on coding modes and RQT depth, as follows.

atable = lev_thresholds[ prediction_type ] switch( prediction_type ) { case INTRA: bias = RQT_depth; break; case INTER: bias = RQT_depth + (RQT_depth>0); break; case CHROMA: bias = 0; } if (abs(level) > (atable[vlc_adaptive] - bias)) vlc_adaptive++;

## VLC table modification

In current HM, code number, cn is mapped to codeword, using vlcNum. Mapping rules for run and level mode are defined as 10 tables, one unary max and 9 tables with Golomb and Exp-Golomb.

In our approach, we consider one additional parameter. First, the unary-max coding is generalized to code any cn value. (Currently, unary max coding, vlcNum=8, is available for cn<3). Second, the prefix length for determining whether golomb or exp-golomb is used is controlled.

if(uiTableNumber == UNARY_PARAM1) { xWriteUnaryMaxSymbol( uiCodeNumber, uiParam2); } else if ((Int)uiCodeNumber < (uiParam * (1 << uiTableNumber))) { xWriteGolombSymbol( uiCodeNumber, uiTableNumber, uiParam2); } else { xWriteExpGolombSymbol( uiCodeNumber, uiTableNumber, uiParam2); }

In run mode coefficients coding, four pre-defined values (th1, th2, th3 and pre-defined constant) replaces vlcNum arrays for each context. Param1 is pre-defined value, and param2 is calculated using maxrun

if(maxrun < th1 ) xWriteParamVlc(cn, UNARY, maxrun*2+2) else if(maxrun < th2) xWriteParamVlc(cn, 1, maxrun+1) else if(maxrun < th3) xWriteParamVlc(cn, 2, (maxrun+1)>>1) else xWriteParamVlc(cn, const1, const2)

Level mode coding uses also this method. Param1 is same as HM3.0 (0, 1, 2, 3, 4), but param2 is changed whether luma or chroma.

## Run-mode coding improvements with table reduction

In JCTVC-E446 [1], the switching code number technique for INTER is proposed, based on the statistical distribution of in  and . To store switching position, 86 bytes are needed.

For reducing the memory size to store table, modification is performed. The detailed proposal is proposed in JCTVC-F407 [2].

# Simulation results

Based on test conditions specified in CE5, simulations are performed using all three low complexity configurations. The detailed results are available in the attached Excel file.

Table 2 shows the performance, when considering “intra code table modification”, “level coding method”, and “VLC table modification” in Section 2.1, 2.2 and 2.3. The performance improvements are -0.7%, -0.3%, and -0.2% in all intra, random access and low delay configurations

Table 2

| All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | U | V | Y | U | V | Y | U | V |
| -0.5 | -0.2 | -0.1 | -0.2 | 0.3 | 0.5 |  |  |  |
| -0.6 | -0.1 | -0.2 | -0.2 | 0.1 | 0.1 | -0.1 | 0.3 | 0.0 |
| -0.7 | -0.4 | -0.4 | -0.4 | -0.2 | -0.2 | -0.2 | 0.1 | -0.2 |
| -0.9 | -0.5 | -0.5 | -0.4 | -0.3 | -0.3 | -0.2 | 0.1 | -0.1 |
| -0.6 | -0.2 | -0.2 |  |  |  | -0.2 | 0.6 | -0.5 |
| -0.7 | -0.3 | -0.3 | -0.3 | 0.0 | 0.0 | -0.2 | 0.3 | -0.2 |
| 103% |  |  | 100% |  |  | 100% |  |  |
| 100% |  |  | 100% |  |  | 100% |  |  |

Additionally, with “run-mode coding improvements with table reduction” in Section 2.4 or JCTVC-F407, the performance improvements are -0.7%, -0.4% and 0.0% for all intra, random access, and low delay configurations in Table 3. Also, the memory size to storage table can be reduced from 1153 bytes to 614 bytes in Table 4.

Table 3. Simulation results.

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.5 | -0.2 | -0.2 | -0.4 | 0.2 | 0.4 |  |  |  |
| Class B | -0.6 | -0.4 | -0.4 | -0.4 | 0.3 | 0.1 | -0.1 | 1.4 | 0.7 |
| Class C | -0.8 | -0.7 | -0.7 | -0.5 | -0.2 | -0.3 | 0.0 | 0.6 | 0.5 |
| Class D | -0.9 | -0.8 | -0.9 | -0.6 | -0.4 | -0.3 | 0.0 | 0.7 | 0.5 |
| Class E | -0.6 | -0.1 | -0.2 |  |  |  | 0.2 | 0.3 | -0.8 |
| Overall | -0.7 | -0.5 | -0.5 | -0.4 | 0.0 | 0.0 | -0.0 | 0.8 | 0.3 |
| Enc Time[%] | 105% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 98% |  |  |

Table 4. Description of CAVLC table

| Table | Features | HM3.0 | Proposal | Diff |
| --- | --- | --- | --- | --- |
| INTRA(E446) | 4x4 | 5*15 | 5*15 | 145 |
|  | 8x8 | 5*29 | 5*29 |  |
|  | 16x16 |  | 5*29 |  |
|  | all | 220 | 356 |  |
| threshold for level | I | 1*5 | 1*6 | 19 |
|  | P |  | 1*6 |  |
|  | B |  | 1*6 |  |
|  | Chroma |  | 1*6 |  |
|  | all | 5 | 24 |  |
| vlcNum | INTRA | 1*29 | 4*4 (13 elems) | -68 (-73 elems) |
|  |  | 1*29 |  |  |
|  | INTER | 1*29 | 2*4 (8 elems) |  |
|  |  | 1*29 |  |  |
|  | Chroma | 0 | 3*4 (10 elems) |  |
|  | level | 0 | 2*6 |  |
|  | all | 116 | 48 |  |
| INTER (F407) | run-level table | 28*29 | 0 | -636 (-258 elems) |
|  | tables for cn0 & cn1 | x | 4*1*29+4*1*15 |  |
|  | all | 812 | 176 |  |
| sum |  | 1153 | 614 | -540 (-167 elems) |

# Conclusion

This contribution reports CE results for CAVLC improvements. In proposal, JCTVC-E446 is modified and additional techniques are considered. From the results, we recommend the proposed scheme be adopted into HM.

# References

[1] Seongwan Kim, Jaeho Lee, Sangyoun Lee “Coefficient coding improvement in LCEC”, JCTVC-E446, Geneva, Switzerland, March 2011

[2] Jaeho Lee, Seongwan Kim, Kyungmin Lim, Daehyun Pak, Sangyoun Lee, Jianle Chen, Jeonghoon Park “Run-mode coding improvement with table reduction in CAVLC”, JCTVC-F407, Torino, Italy,  July, 2011

# Patent rights declaration(s)

Yonsei University and Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Modifications to bitstream syntax

New chapter is inserted between 9.2.1 Parsing process for VLC codes (it may change to “9.2.1.1. Parsing process for pre-determined VLC codes”) and 9.2.2. Initialisation process.

9.2.1.2. Parsing process for parameterized VLC codes Inputs to this process are bits from the RBSP and 2 parameters vlcNum and vlcParam specifying the VLC code. Depending on the value of vlcNum and vlcParam, the value of codeNum is assigned as follows. - If vlcNum is less than 5, - If leadingZeroBits is less than or equal to vlcParam, codeNum = (leadingZeroBits << vlcNum) + read_bits( vlcNum ) Otherwise (leadingZeroBits is greater than 6), numBits = leadingZeroBits – vlcParam + vlcNum codeNum = (vlcParam-1)*(1<<vlcNum) + (1<< numBits) + read_bits( numBits) - If vlcNum is equal to 5, - Identical to Turncated unary binarization

9.2.4.3. and 9.2.4.4 changes as follows:

9.2.4.3. Parsing process for run_level_one

Inputs to this process are bits from slice data, the scan position n of the previous non-zero transform coefficient in inverse scan order, and the variable TrOne.

Outputs of this process are the syntax element run_level_one and the variable TrOne.

The value of run_level_one is derived as follows.

- The variable N is set equal to (1 << log2TrafoSize) >> (cIdx > 0 ? 2 : 0).

- The variable blockType is derived as

blockType = (cIdx == 0?(PredMode==MODE_INTRA?0: slice_type + 1) + (N > 8 ? 5 : 2) : cIdx – 1)										 (9-1)

- The variable maxRun is set equal to n.

- The variable vlcThIdx is derived as

If blockType is less than 2,

vlcThIdx = (trOne == 0 ? (N == 4 ? 6 : 7) : 8)

Otherwise, if block type is equal 2 or 5,

vlcThIdx = (N==4 ? 0: (N == 8 ? 1 : (trOne == 0 ? 2: 3)))

Otherwise

vlcThIdx = (N <= 8 ? 4 : 5))

- The variable vlcNum and vlcParam is derived as

tmprunParam = maxRun < th1 ? 160 : (maxRun < th2 ? (33+ maxRun) : (maxRun<th3 ? ((maxRun+1)>>1)): cParam);

vlcNum = tmp >> 5

vlcParam = tmp & 31

  - The constant th1,th2,th3 and cParam is drives from N, blockType and TrOne as shown Table 9-9.

Table 9-9. Derivation of thresholds and runParam from vlcThIdx

| vlcThIdx | Th1 | Th2 | Th3 | cParam |
| --- | --- | --- | --- | --- |
| 0 | 3 | Inf. | NA | NA |
| 1 | 3 | 14 | 28 | 83 |
| 2 | 4 | 18 | 19 | 32 |
| 3 | 5 | 18 | 28 | 69 |
| 4 | 3 | 10 | 28 | 67 |
| 5 | 2 | 6 | 23 | 102 |
| 6 | 1 | 6 | Inf. | NA |
| 7 | 1 | 5 | 18 | 64 |
| 8 | 2 | 10 | 28 | 67 |

- The parsing process described in subclause Error: Reference source not found1.2 is invoked with vlcNum and vlcParam as input and the variable codeNum as output.If blockType is equal to 2 or blockType is equal to 5, the value of run_level_one is derived as follows.

  - If N is equal to 4, the variable largeOnePos is derived from trOne and maxRunIdx as shown in Table 9-5.

  - Otherwise, if N is equal to 8, the variable largeOnePos is derived from trOne and maxRunIdx as shown in Table 9-5.

  - Otherwise (N is greater than 8), the variable largeOnePos is derived from trOne and maxRunIdx as shown in Table 9-6.

  - The variables levelGreaterThanOneFlag and runOfZeros are derived as follows.

if ( largeOnePos > 0 ) {
		if( codeNum < min(largeOnePos, maxRunIdx + 2 ) ) {
			levelGreaterThanOneFlag = 0
			runOfZeros = codeNum
		} else if( codeNum < (maxRunIdx << 1 ) + 4 – largeOnePos ) {
			if(( codeNum + largeOnePos ) & 1 ) {
				levelGreaterThanOneFlag = 0
				runOfZeros = (codeNum + largeOnePos - 1) >> 1
			} else {
				levelGreaterThanOneFlag = 1
				runOfZeros = (codeNum - largeOnePos)>>1	(9-2)
			}
		} else {
			levelGreaterThanOneFlag = 1
			runOfZeros = codeNum - maxRunIdx – 2
		}
	} else {
		if( codeNum & 1) {
			levelGreaterThanOneFlag = 0 
			runOfZeros = (codeNum - 1)>>1
		} else {
			runOfZeros = codeNum >> 1
			levelGreaterThanOneFlag = ((codeNum >> 1) <= maxRunIdx)?1:0
		}
	}

  - The syntax element run_level_one is derived as

run_level_one = levelGreaterThanOneFlag ? runOfZeros + maxRun : runOfZeros	(9-3)

- Otherwise (blockType is not equal to 2 and blockType is not equal to 5), the value of run_level_one is derived as follows. [or refer F407]

  - The variable levelGreaterThanOneFlag is derived as

levelGreaterThanOneFlag = codeNum <= maxRun + 1 ? 0 : 1	(9-4)

  - If maxRun is less than 28, the variable runOfZeros is derived from maxRun and codeNum as shown in Error: Reference source not found.

  - Otherwise (maxRun is not less than 28), the variable runOfZeros is derived as

runOfZeros = codeNum <= maxRun + 1 ? codeNum : codeNum – maxRun – 2	(9-5)

  - The syntax element run_level_one is derived as

run_level_one = levelGreaterThanOneFlag ? runOfZeros + n : runOfZeros	(9-6)

- The variable trOne is derived as

trOne = (trOne = = 0 | | levelGreaterThanOneFlag) ? 0 : Max(4, trOne + 1) 	(9-7)

Table 9-5 – Derivation of largeOnePos from trOne and maxRunIdx when N is equal to 4

|  | maxRunIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| trOne | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 2 | 2 | 4 | 4 | 2 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 2 |
| 2 | 1 | 2 | 1 | 2 | 2 | 2 | 2 | 4 | 4 | 4 | 2 | 2 | 2 | 2 | NA |
| 3 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | NA | NA |
| 4 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 | NA | NA | NA |

Table 9-6 – Derivation of largeOnePos from trOne and maxRunIdx when N is equal to 8

|  | maxRunIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| trOne | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| 0 | 0 | 1 | 1 | 2 | 2 | 2 | 2 | 4 | 4 | 2 | 2 | 2 | 2 | 4 | 4 |
| 1 | 1 | 3 | 4 | 4 | 4 | 4 | 4 | 8 | 6 | 6 | 6 | 8 | 6 | 8 | 10 |
| 2 | 1 | 3 | 4 | 4 | 4 | 4 | 4 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| 3 | 1 | 2 | 2 | 2 | 2 | 2 | 4 | 6 | 4 | 4 | 4 | 4 | 4 | 4 | 6 |
| 4 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 4 | 2 | 2 | 4 | 4 | 4 | 4 | 4 |

|  | maxRunIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| trOne | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| 0 | 4 | 4 | 4 | 2 | 4 | 4 | 4 | 6 | 6 | 4 | 6 | 6 | 6 | 4 |
| 1 | 12 | 8 | 10 | 8 | 8 | 10 | 8 | 10 | 12 | 10 | 8 | 10 | 14 | 12 |
| 2 | 10 | 8 | 8 | 6 | 8 | 10 | 8 | 10 | 12 | 10 | 10 | 10 | 12 | 12 |
| 3 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 10 | 6 | 10 | 8 | 10 | 10 |
| 4 | 4 | 6 | 4 | 4 | 4 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 8 | 8 |

Table 9-7 – Derivation of largeOnePos from trOne and maxRunIdx when N is greater than 8

|  | maxRunIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| trOne | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 4 | 4 | 2 | 2 | 4 | 4 | 8 | 4 |
| 1 | 1 | 3 | 3 | 3 | 5 | 4 | 4 | 6 | 8 | 7 | 6 | 6 | 8 | 12 | 13 |
| 2 | 1 | 3 | 2 | 3 | 4 | 2 | 4 | 6 | 6 | 6 | 6 | 6 | 8 | 12 | 8 |
| 3 | 1 | 1 | 1 | 2 | 3 | 2 | 2 | 4 | 4 | 4 | 4 | 6 | 6 | 8 | 6 |
| 4 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 4 | 4 | 4 | 4 |

|  | maxRunIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| trOne | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| 0 | 4 | 6 | 8 | 8 | 8 | 5 | 4 | 4 | 6 | 8 | 10 | 12 | 28 | 10 |
| 1 | 8 | 10 | 10 | 10 | 18 | 13 | 10 | 12 | 12 | 12 | 12 | 26 | 13 | 18 |
| 2 | 8 | 8 | 10 | 12 | 16 | 11 | 12 | 12 | 12 | 12 | 12 | 20 | 13 | 22 |
| 3 | 6 | 8 | 8 | 10 | 10 | 10 | 8 | 10 | 10 | 10 | 12 | 12 | 12 | 18 |
| 4 | 4 | 4 | 6 | 6 | 6 | 4 | 4 | 4 | 6 | 6 | 6 | 6 | 6 | 10 |

table 9-13 [skip]

9.2.5.4. Parsing process for level

Inputs to this process are bits from slice data and the variable vlcNumLevel.

Outputs of this process are the syntax element level and the variable vlcNumLevel.

- The syntax element level is determined by invoking the parsing process described in subclause Error: Reference source not found with vlcNumLevel and vlcParamLevel as input and level as output.

- The variable vlcNumLevel derived in the following ordered steps updated as follows.

- The variable levTableIdx is derived as

levTableIdx = (cIdx == 0 ? (PredMode==MODE_INTRA ? 0 : slice_type + 1) : 3)

- The variable vlcIdx is set equal to vlcIdx = Min(4,vlcNumLevel)

- The variable levelThreshold is derived from vlcIdx as shown in Table 9-8.

- The variable levThBias is derived as

rqtDepth=log2CUSize - log2TrafoSize

levThBias = (levTableIdx == 0 ? rqtDepth : (levTableidx <3 ? (rqtDepth + (rqtDepth>0)) : 0 ))

- The variable vlcNumLevel is derived as

vlcNumLevel = vlcNumLevel + (level > (levelThreshold - levThBias) && vlcNumLevel < 5 ? 1 : 0) 	(9-8)

- The variable vlcParamLevel is derived from cIdx as shown in Table 9-8

Table 9-8 – Derivation of levelThreshold from vlcIdx

|  | levTableIdx |  |  |  |
| --- | --- | --- | --- | --- |
| vlcIdx | 0 | 1 | 2 | 3 |
| 0 | 4 | 4 | 6 | 2 |
| 1 | 8 | 7 | 11 | 5 |
| 2 | 18 | 14 | 14 | 11 |
| 3 | 37 | 23 | 27 | 25 |
| 4 | 102 | 56 | 74 | 88 |

Table 9-9 – Derivation of vlcParamLevel from vlcIdx

|  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| cIdx == 0 | 8 | 10 | 14 | 16 | 8 | 0 |
| cIdx > 0 | 12 | 12 | 14 | 16 | 8 | 0 |