| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F412 |
| --- | --- |

| Title: | CE2: Non-Square Quadtree Transform for symmetric and asymmetric motion partition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuan Yuan, Tsinghua Xiaozhen. Zheng, HiSilicon Xiulian Peng, USTC Jizheng Xu, Microsoft Il-Koo Kim, Samsung Lingzhi Liu, HiSilicon Yunfei Wang, Tsinghua Xiaoran Cao, Tsinghua Changcai Lai, HiSilicon Jianhua Zheng, HiSilicon Yun He, Tsinghua Haoping Yu, Huawei | Tel: Email: | y-yuan08@mails.tsinghua.edu.cn zhengxiaozhen@huawei.com |
| Source: | Tsinghua University, Huawei, HiSilicon, Microsoft, University of Science and Technology of China (USTC), Samsung |  |  |

_____________________________

# Abstract

This contribution is related to core experiment 2: Motion Partitioning and OBMC. In the contribution, Non-Square Quadtree Transform (NSQT) is proposed to be used when partition type is 2NxN or Nx2N or asymmetric partition. The combination test results of NSQT + asymmetric motion partition (AMP) are provided.

The average bitrate saving of AMP + NSQT for asymmetric motion partition is 0.7% for RA_HE, 0.8% for RA_LC, 1.6% for LD_HE and 1.8% for LD_LC with about 12% encoder complexity increase. Decoder complexity is same as HM3.0.

The average bitrate saving of AMP + NSQT for symmetric and asymmetric motion partition is 0.8% for RA_HE, 0.9% for RA_LC, 1.9% for LD_HE and 2.3% for LD_LC with about 13% encoder complexity increase. Decoder complexity is same as HM3.0.

# Introduction

At current HEVC standard, residual quad transform (RQT) is used at block residual transform. A transform block can be split recursive. When a transform block cross the boundary of motion block, high frequency coefficients are generated which will impose negative influence on coding performance. For the case of Asymmetric Motion Partition (AMP) [1], this problem will be more serious because transform block at first and second level will cross motion blocks' boundary.

To solve the problem mentioned above, implicit TU split method has proposed at [2], and has been adopted to HEVC working draft. At [2], RQT root location in inter mode is tied to PU partition type when max_transform_hierarchy_depth_inter is equal to 0. Besides the implicit TU method, non-square TU has proposed at [3]. At [3], 2Nx0.5N or 0.5Nx2N TU is used at motion partitions when PU partition type is asymmetric partition.

Considering PU partition type can reflect local region's texture property, non-square TU can also be used at symmetric motion partitions. The size of a transform unit is tied to PU partition type.

Base on the method described at [2] and task of CE2 [4], this contribution reports the coding results of Non-Square Quadtree Transform (NSQT), which is the combination of non-square TU with RQT structure. The results of combination of NSQT + AMP are also provided.

# Asymmetric Motion Partition (AMP)

The Asymmetric motion partitioning (AMP) was initially proposed in JCTVC-A124[1]. It allowed rectangular shape PU splitting of a block for inter prediction.                Figure 1 shows an example of the AMP where a 64x64 block is asymmetrically partitioned into 64x16, 64x48, 16x64, or 48x16 blocks, instead of two 64x32 or 32x64 blocks. The motion partitions shown in the figure correspond to 2NxnU, 2NxnD, nLx2N, and nRx2N, respectively. AMP improves the coding efficiency, since irregular image patterns, which otherwise would be constrained to being represented by a symmetric partition, can now be more efficiently represented without requiring further splitting.

Figure 1 Example of asymmetric motion partitions for 64x64 CU

# Non-Square Quadtree Transform (NSQT)

AT NSQT design, 2Nx2N transform is still used at RQT’s first level, which is based on the consideration that 2Nx2N transform can get better coding result when residual inside a coding unit is small. What's more, use 2Nx2N TU at RQT's first level can also save signaling bits that could help to improve coding efficiency.

At RQT's second and third level, non-square TU is used when the PU partition type is 2NxN, Nx2N or asymmetric partition. Let's define 2M = 1 << log2TrafoSize. Then, for the case of 2NxN, 2NxnU and 2NxnD PU type, TU size at RQT's second and third level will be 2Mx0.5M; for the case of Nx2N, nLx2N and nRx2N PU type, TU size at RQT's second and third level will be 0.5Mx2M. For 2Nx2N and NxN PU types, square TU with the size of 2Mx2M or MxM is used.

According to the methodology described above, at RQT’s second level, a block is divided into four non-square TU along with vertical direction if PU partition type is 2NxN, or four non-square TU along with horizontal direction if PU partition type is Nx2N. At RQT’s third level, a block is divided into four non-square TU along with horizontal and vertical direction. This method preserves the residual quadtree structure, and coding performance can be improved. Figure 2a and 2b illustrate this implementation scheme.

Figure2a: Illustration of TU split at RQT's second and third level (2NxN, 2NxnU and 2NxnD PU type)

Figure2b: Illustration of TU split at RQT's second and third level (Nx2N, nLx2N and nRx2N PU type)

At current NSQT design, no new core transform is introduced. The core transform of 2Nx0.5N and 0.5Nx2N can be reused by 0.5Nx0.5N and 2Nx2N core transform. In general, a NxM block is transformed by the following steps as described in [5].

CMxN = TM x BNxM x TNT

where BNxM denotes a block with N pixels M rows. TN and TM are the transform matrices of size NxN and MxM, respectively. CNxM denotes the transformed NxMblock.

According to the above analysis, 2Nx0.5N core transform can be achieved as follow:

C2Nx0.5N = T0.5N x B2Nx0.5N x T2NT

0.5Nx2N core transform can be achieved as follow:

C0.5Nx2N = Tm x B0.5Nx2N x T0.5NT

Non additional quantization table is added to current implementation. Based on the principle of this design, non-square TU can be used at 32x24, 32x16, 32x8, 8x32, 16x32, 24x32, 16x12, 16x8, 16x4, 4x16, 8x16, 12x16, 8x4 and 4x8 partition at luma and chroma component.

# Experimental results

AMP + NSQT are implemented on HM3.0_rev.823 software. To evaluate interaction between AMP, NSQT for asymmetric partitions, and NSQT for both symmetric and asymmetric partitions, three groups of test results are provide: AMP only, AMP + NSQT for asymmetric partitions, and AMP + NSQT for both symmetric and asymmetric partitions. The test conditions and configurations is same as JCTVC-700 [6]. The coding efficiency is measured by using BD-Rate. The experimental results are shown in Table 1 ~ 3.

Table 1: AMP only

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.8 | -0.5 | -0.4 | -0.6 | -0.2 |
| Class B | -0.5 | -0.8 | -0.7 | -0.4 | -0.5 | -0.5 |
| Class C | -0.6 | -0.8 | -0.8 | -0.5 | -0.6 | -0.7 |
| Class D | -0.5 | -0.9 | -0.8 | -0.4 | -0.5 | -0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.8 | -0.7 | -0.4 | -0.6 | -0.5 |
| Enc Time[%] | 112% |  |  | 112% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | -1.1 | -1.2 | -0.5 | -0.4 | -0.3 |
| Class C | -0.7 | -1.0 | -0.9 | -0.4 | -0.4 | -0.7 |
| Class D | -0.7 | -1.2 | -0.8 | -0.5 | -0.7 | -0.2 |
| Class E | -1.5 | -1.9 | -1.7 | -1.0 | -1.0 | -0.9 |
| All | -0.8 | -1.2 | -1.1 | -0.6 | -0.6 | -0.5 |
| Enc Time[%] | 112% |  |  | 111% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 2: AMP + NSQT for asymmetric partitions

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -1.3 | -1.3 | -0.6 | -1.0 | -0.8 |
| Class B | -0.7 | -2.1 | -1.9 | -0.9 | -1.8 | -1.6 |
| Class C | -0.8 | -1.5 | -1.5 | -0.9 | -1.3 | -1.2 |
| Class D | -0.7 | -1.4 | -1.4 | -0.7 | -1.0 | -1.0 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -1.6 | -1.6 | -0.8 | -1.3 | -1.2 |
| Enc Time[%] | 112% |  |  | 113% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.2 | -4.4 | -4.4 | -1.6 | -3.2 | -3.0 |
| Class C | -1.3 | -3.4 | -3.4 | -1.4 | -2.2 | -2.2 |
| Class D | -1.5 | -3.3 | -3.0 | -1.4 | -1.4 | -1.5 |
| Class E | -2.6 | -3.9 | -4.1 | -3.1 | -2.1 | -2.8 |
| All | -1.6 | -3.8 | -3.7 | -1.8 | -2.3 | -2.4 |
| Enc Time[%] | 111% |  |  | 112% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 3: AMP + NSQT for both symmetric and asymmetric partitions

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | -1.5 | -1.3 | -0.6 | -1.2 | -1.0 |
| Class B | -0.8 | -2.4 | -2.1 | -1.0 | -1.9 | -1.7 |
| Class C | -0.9 | -1.6 | -1.6 | -1.0 | -1.4 | -1.3 |
| Class D | -0.8 | -1.5 | -1.4 | -0.8 | -0.9 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | -0.8 | -1.8 | -1.6 | -0.9 | -1.4 | -1.2 |
| Enc Time[%] | 114% |  |  | 114% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.5 | -4.7 | -5.0 | -1.9 | -3.4 | -3.4 |
| Class C | -1.5 | -3.4 | -3.6 | -1.8 | -2.3 | -2.3 |
| Class D | -1.7 | -3.4 | -2.8 | -1.9 | -1.7 | -1.3 |
| Class E | -3.1 | -4.3 | -4.8 | -4.0 | -2.3 | -3.2 |
| All | -1.9 | -4.0 | -4.1 | -2.3 | -2.5 | -2.6 |
| Enc Time[%] | 113% |  |  | 113% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

# Modification of working draft

The details of working draft modification can be found at the appendix: JCTVC-F412_change_for_wd.doc.

# Conclusion

In this contribution, Non-Square Quadtree Transform and Asymmetric Motion Partition is proposed. Compare to HM3.0 anchor, The average bitrate saving of AMP + NSQT for asymmetric motion partition is 0.7% for RA_HE, 0.8% for LD_HE, 1.6% for LD_HE and 1.8% for LD_LOCO with about 12% encoder complexity increase. Decoder complexity is same as HM3.0; the average bitrate saving of AMP + NSQT for symmetric and asymmetric motion partition is 0.8% for RA_HE, 0.9% for LD_HE, 1.9% for LD_HE and 2.3% for LD_LOCO with about 13% encoder complexity increase. Decoder complexity is same as HM3.0.

The combination of AMP + NSQT for asymmetric and symmetric partitions can provide satisfied coding performance, while the encoder complexity is low, and no additional decoder complexity is introduced. It is recommended to incorporate AMP and NSQT into HM.

# Reference

[1] JCT-VC, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, JCTVC-A124, JCT-VC Meeting, Dresden, DE, April 2010.

[2] K. Panusopone, X. Fang, L. Wang, “Proposal on RQT root location” JCT-VC document, JCTVC-E364, March 2011.

[3] Y. Yuan (Tsinghua), X. Zheng (HiSilicon), X. Peng (USTC), L. Liu (HiSilicon), Y. Wang (Tsinghua), X. Cao (Tsinghua), J. Xu (Microsoft), C. Lai (HiSilicon), J. Zheng (HiSilicon), Y. He (Tsinghua), H. Yu (Huawei), “Asymmetric Motion Partition with OBMC and Non-Square TU”, Doc. JCTVC-E376, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

[4] X. Zheng, P. Bordes, P. Chen, I-K Kim, “Core Experiment 2: Motion Partitioning and OBMC,” Doc. JCTVC-D702, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011.

[5] Mathias Wien and Achim Dahlhoff, “Adaptive Block Transforms”, Doc.VCEG-M62, Q.6/SG16 (VCEG), Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001

[6] Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, March 2011.

# Patent rights declaration(s)

Tsinghua University, Huawei Technologies, HiSilicon Technologies, Microsoft, University of Science and Technology of China (USTC) and Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).