| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F413 |
| --- | --- |

| Title: | CE6.c Cross-check of Differential Coding of Intra Mode (DCIM) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to the JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Glenn Van Wallendael Sebastiaan Van Leuven Jan De Cock Rik Van de Walle Gaston Crommenlaan 8 P.O. Box 201 9050 Ghent-Ledeberg Belgium | Tel: Email: | +32 93314957 glenn.vanwallendael@ugent.be |
| Source: | Ghent University - IBBT |  |  |

_____________________________

# Abstract

Document JCTVC-F413 provides a cross verification of CE6.c Differential Coding of Intra Mode (DCIM). As a reference for comparison, HM-3.0 is used.

Four test cases were verified: 1) DCIM default ; 2)DCIM with sub direction off; 3) DCIM with BIP off; 4)DCIM with 4TAP off.

We compiled, inspected, and ran the code of these cases with Intra HE and Intra LC configurations. RD results were identical to those provided. No irregularities were found in the code.

# Test conditions

Our computing platform used for cross-verification tests is shown below.

## Encoding

Super computer with 167 computing nodes (IBM iDataPlex dx360M2), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5520 (Intel Nehalem microarchitecture, 2.27 GHz, 8 MB L3 cache per quad-core chip), thus 8 cores / node

- memory: 12 GB RAM (DDR3 PC3-8500S 1067 MHz)

- storage (local): one 156 GB 3.5" 15k RPM SAS disk

and 194 computing nodes (IBM HS 21 XM blade), each of which contains:

- CPU: dual-socket quad-core Intel Xeon L5420 (Intel Core microarchitecture, 2.5 GHz, 6 MB L2 cache per quad-core chip), thus 8 cores / node

- memory: 16 GB RAM (DDR2 FB-DIMM PC-5300 CL5)

- storage (local): one 146 GB 2.5" 10k RPM SAS disk

The encoder executables were generated with g++ 4.1.2.

## Decoding

- CPU: quad-core Intel Core 2 Q6600 (Intel Core microarchitecture, 2.40GHz, 8MB L2 cache)

- Memory: 4 GB RAM (DDR2 PC2-6400 400MHz)

# Code

The code is conforming the proponants description.

# Simulation results

Four different test cases were cross verified.

- DCIM default

(USE_DCIM=1, DCIM_SUBDIR=1, DCIM_BIPRED=1, TAP4_FILT=1)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,21 | 0,03 | 0,12 | -0,19 | 0,00 | 0,00 |
| Class B | -0,71 | -1,00 | -1,13 | -0,47 | -0,43 | -0,54 |
| Class C | -1,21 | -1,08 | -1,08 | -1,45 | -0,71 | -0,83 |
| Class D | -0,70 | -0,35 | -0,39 | -0,61 | -0,15 | -0,22 |
| Class E | -1,04 | -1,27 | -0,86 | -0,76 | -1,04 | -0,70 |
| All | -0,76 | -0,72 | -0,68 | -0,68 | -0,43 | -0,45 |
| Enc Time[%] | 114% |  |  | 127% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |

- DCIM with sub direction off

(USE_DCIM=1, DCIM_SUBDIR=0, DCIM_BIPRED=1, TAP4_FILT=1)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,05 | 0,33 | 0,31 | -0,01 | 0,20 | 0,22 |
| Class B | -0,30 | -0,28 | -0,24 | -0,08 | 0,02 | 0,03 |
| Class C | -0,77 | -0,52 | -0,53 | -1,06 | -0,33 | -0,40 |
| Class D | -0,63 | -0,23 | -0,21 | -0,57 | -0,08 | -0,12 |
| Class E | -0,32 | -0,13 | 0,07 | -0,14 | -0,30 | -0,07 |
| All | -0,41 | -0,17 | -0,13 | -0,37 | -0,08 | -0,06 |
| Enc Time[%] | 107% |  |  | 114% |  |  |
| Dec Time[%] | 103% |  |  | 106% |  |  |

- DCIM with BIP off

(USE_DCIM=1, DCIM_SUBDIR=1, DCIM_BIPRED=0, TAP4_FILT=1)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,20 | 0,06 | 0,02 | -0,09 | 0,06 | 0,13 |
| Class B | -0,71 | -1,04 | -1,15 | -0,36 | -0,32 | -0,46 |
| Class C | -1,12 | -1,05 | -1,05 | -1,23 | -0,54 | -0,64 |
| Class D | -0,61 | -0,37 | -0,36 | -0,41 | -0,09 | -0,12 |
| Class E | -0,98 | -1,24 | -0,83 | -0,57 | -0,94 | -0,68 |
| All | -0,71 | -0,72 | -0,69 | -0,52 | -0,34 | -0,34 |
| Enc Time[%] | 112% |  |  | 121% |  |  |
| Dec Time[%] | 103% |  |  | 106% |  |  |

- DCIM with 4TAP off

(USE_DCIM=1, DCIM_SUBDIR=1, DCIM_BIPRED=1, TAP4_FILT=0)

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,25 | 0,15 | 0,10 | -0,20 | 0,01 | 0,04 |
| Class B | -0,32 | -0,59 | -0,73 | 0,11 | 0,14 | 0,02 |
| Class C | -0,57 | -0,51 | -0,41 | -0,47 | -0,06 | -0,10 |
| Class D | -0,26 | 0,07 | 0,09 | -0,02 | 0,30 | 0,24 |
| Class E | -0,37 | -0,76 | -0,31 | 0,13 | -0,38 | -0,12 |
| All | -0,35 | -0,32 | -0,27 | -0,09 | 0,03 | 0,02 |
| Enc Time[%] | 114% |  |  | 125% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |

# Conclusion

The code and results are verified and are conformant to the results stated by the proponents.

# Acknowledgement

The research activities described in this proposal were funded by Ghent University, the Interdisciplinary Institute for Broadband Technology (IBBT), the Institute for the Promotion of Innovation by Science and Technology in Flanders (IWT), the Fund for Scientific Research-Flanders (FWO-Flanders), and the European Union. Furthermore, this work was carried out using the Stevin Supercomputer Infrastructure at Ghent University.