| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F416 |
| --- | --- |

| Title: | Intra Mode Coding considering MPM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. W. Seo, J. K. Han, H.K. Kim (Sejong University) J.Lim (SKT) 910, Choongmu-Gwan, Sejong University, 98 Gunja-Dong, Gwangjin-Gu, Seoul, 143-747, Korea 9-1 Sunae-Dong, Bundang-gu, Seongnam City, Kyeonggi-Do, 463-784, Korea | Tel: Email: | +82-2-3408-3739 sachonwon@gmail.com hjk@sejong.ac.kr hkkim@sejong.ac.kr +82-31-710-5196 jeongyeon@sktelecom.com |
| Source: | Sejong University and SK telecom |  |  |

_____________________________

# Abstract

This contribution states intra mode coding method considering Most Probable Mode (MPM). At the last meeting held in Geneva, use of multiple MPMs was adopted in HM. So, it is needed to study encoding method for remaining intra modes which depend on MPMs. The encoding method for remaining intra modes is proposed in this contribution. When  multiple MPMs are used and prev_intra_luma_pred_flag is equal to ‘0’, rem_intra_luma_pred_mode may be determined without signaling the intra mode, since the number of remaining modes is only one and the mode can be expected. In this case, this contribution reduces the redundant bits for intra mode coding in HM with simple modification.

# Introduction

At the last meeting held in Geneva, the use of multiple MPMs was adopted into HM [1].  This contribution modifies the coding scheme for the intra prediction mode when the intra mode can be expected without indexing the mode since multiple MPMs and prev_intra_luma_pred_flag=0 provide the related information. In the current HM, there are 3 to 35 intra prediction modes excepting for planar mode. According to the current design of encoding module for the intra prediction mode, rem_intra_luma_pred_mode should be signaled always even if the current intra prediction mode is not included in MPMs and the mode is unique, thus the mode is obvious. This contribution proposes a scheme to skip the rem_intra_luma_pred_mode when the current intra mode is obvious.

# Algorithm description

In the current HM, MPMs are determined by the intra prediction modes of neighboring PUs as shown in Figure 1.

Figure 1. Left and Top neighbor PUs of the current PU

If two different intra prediction modes are used in PU A and B, two MPMs are considered to encode the current intra prediction mode, where the number of remaining intra prediction modes is determined as follows.

NR = NS - NMPM

where NR is the number of remaining intra prediction modes, and NS and NMPM denote the numbers of available intra prediction modes and MPMs. In the proposed method, rem_intra_luma_pred_mode is not encoded if NR is equal to ‘1’ and prev_intra_luma_pred_flag is equal to ‘0’. Decoder can determine whether rem_intra_luma_pred_mode should be parsed or not based on prev_intra_luma_pred_flag and NR. In the current HM, the maximum number of MPMs is 2. So, the proposed method can be applied for coding of 64×64 intra PU which has 3 intra prediction modes. It can be implemented by minor modification of HM codes.

# Simulation results

The performance of the proposed method is evaluated under common test conditions defined by [2]. HM3.2 is used as an anchor in the simulations. Table 1 shows the simulation results. Although proposed method does not give gain, syntax related to the mode information is simplified.

Table 1. BD-rate gain with HM3.2 for intra only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

In this contribution, intra mode coding method considering multiple MPMs is proposed. It removes unnecessary bits with minor modification. Logically, the proposed method is more appropriate to HM than the current design. Therefore it is recommended to adopt the proposed intra mode coding method in HEVC.

# References

- M. Guo, X. Guo, and S. Lei, “CE14 Subset 1: The Most Probable Mode Signaling for Luma,” JCTVC-E088, 5th JCT-VC Meeting, Geneva, Switzerland, Mar., 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700, 5th JCT-VC Meeting, Geneva, Switzerland, Mar., 2011.

# Patent rights declaration(s)

Sejong University and SK Telecom may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).