| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F417 |
| --- | --- |

| Title: | Pixel Based Illumination Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. W. Seo, J. K. Han (Sejong University) J.Lim (SKT) 910, Choongmu-Gwan, Sejong University, 98 Gunja-Dong, Gwangjin-Gu, Seoul, 143-747, Korea 9-1 Sunae-Dong, Bundang-gu, Seongnam City, Kyeonggi-Do, 463-784, Korea | Tel: Email: | +82-2-3408-3739 sachonwon@gmail.com hjk@sejong.ac.kr +82-31-710-5196 jeongyeon@sktelecom.com |
| Source: | Sejong University and SK telecom |  |  |

_____________________________

# Abstract

This contribution states a pixel based illumination compensation scheme to compensate illumination changes. The Weighted Prediction (WP) proposed at the last meeting has been designed to compensate illumination variation at slice level and it was tested with fade sequences. The WP is useful but it may be not effective for nature sequences whose brightness varies locally. The IC scheme proposed in this contribution derives IC parameters at CU-level with simple operations. The IC parameters are applied to a block or some pixels adaptively. It improves coding performance of IC. The simulation results reports that the pixel based illumination compensation scheme has coding gains about 0.2% and 0.3% for random access and low delay coding structures, respectively.

# Introduction

In the last meeting, a WP scheme [1] was proposed and an AHG for WP was established to investigate illumination compensation schemes. The WP proposed in [1] is useful but it may be not effective for nature sequences whose illumination varies locally. So, the proposed IC scheme considers local illumination variation for each CU. The IC parameters are derived by using the reconstructed neighboring pixels, and they are applied to a block or some pixels adaptively. The details are described in section 2.

# Algorithm description

- IC parameter derivation

To compensate local illumination changes with small overhead bits, the reconstructed neighboring pixels of the current block are used in the proposed IC scheme as shown in Figure 1.

Figure 1. Neighboring pixels of the current block

If there are some outliers among the neighboring pixels then it makes negative effect to calculate the optimal IC parameters. So, the outliers of the neighboring pixels are excluded in calculation of IC parameters as follows.

|  | (1) |
| --- | --- |

where  is a set of selected neighboring pixels.  and  denote lower and upper thresholds to limit the pixel values, respectively. The thresholds are calculated by using (2) and (3).

|  | (2) |
| --- | --- |
|  | (3) |

where  is mean value of the neighboring pixels and  implies variation of the neighboring pixels as follows.

|  | (4) |
| --- | --- |

In (4),  is the number of the total neighboring pixels.

In calculation of IC parameters, the selected neighboring pixels  of the current block are compared with the corresponding neighboring pixels of motion compensated block which is decided by a rounded PMV. In the proposed IC scheme, the rounded PMV0 (a PMV with index 0) is used even though multiple PMVs are available. The reason why PMV0 is rounded to integer is that IC parameters can be calculated without interpolation by using integer PMV.

Figure 2. Neighboring pixels of motion compensated block with rounded PMV0

In Figure 2,  denotes a set of pixels which are corresponding to the selected neighboring pixels  of the current block. In this contribution, the weighting factor to compensate illumination change is calculated as follows.

|  | (5) |
| --- | --- |

where  is the number of the selected pixels .  is weighting factor and  is bit precision of the weighting factor. In this contribution,  is set to ‘13’. When sum of  is zero, an offset is used to handle that case. The offset is calculated as follows.

|  | (6) |
| --- | --- |

- Prediction block generation

A prediction block is generated by applying the calculated weighting factor or offset to the MC block. If the IC is applied to some pixels which are uncorrelated to the selective neighboring pixels set , the scheme degrades the coding performance. In the proposed IC scheme, classification of pixels in MC block is applied. To classify those pixels in the MC block, the lower and upper thresholds are used as follows.

|  | (7) |
| --- | --- |

where  is a pixel at position j of MC block.  and  are calculated as:

|  | (8) |
| --- | --- |
|  | (9) |

where  is mean value of pixels in .  is calculated by the following equation.

|  | (10) |
| --- | --- |

In (10),  is the number of the neighboring pixels. Figure 3 shows an example of the classification resulted from (7).

Figure 3. Example of selected pixels in 8x8 MC block

Prediction block is generated as follows.

|  | (11) |
| --- | --- |

where  is the j th prediction pixel in the prediction block.  is sum of the corresponding neighboring pixels as following equation.

|  | (12) |
| --- | --- |

- Flag signaling

To minimize the loss of coding efficiency resulting from adding flags, the proposed IC scheme is applied only to 2N×2N PU. The proposed scheme is applied for uni-directional inter prediction only, because bi-directional inter prediction can compensate illumination changes in many cases. Therefore, 1-bit pbic_flag only for each 2N×2N PU with uni-directional inter prediction is signaled.

Since the rounded PMV0 is used in calculation of IC parameters, MVs tend to be located near the rounded PMV0. Thus, to maximize coding efficiency, the rounded PMV0 is used to predict and encode MV without signaling PMV index.

- Adaptation of application level

When sequence has global significant illumination change, the proposed pixel based IC scheme is not effective for block based coding structure since the pixel based IC increase the high frequency components of residual data and degrades the coding performance. So, in this case, the proposed IC is applied to all pixels in the current block, i.e., the proposed scheme is used with  = 0 and in equation (7). It is determined whether significant illumination change is occurred or not depending on

the weighting factor  calculated in (5). When the value of  is too small or too large, the pixel based IC is not effective. Note that  is represented with (P+1)-bits, so that , ,  implies  =0, 1.0, 2.0 in floating point values, respectively. The decision for this case is performed as follows. When  is between  and , the proposed IC is applied to the selected pixels in the current block. In other case where  has other values, the proposed IC is applied to all pixels in the current block. The procedure is described as follows.

|  | (13) |
| --- | --- |
|  | (14) |

# Simulation results

The performance of the proposed IC is evaluated with common test conditions defined by [2]. HM3.2 is used as an anchor. Error: Reference source not found shows the BD-rate gain achieved with the proposed IC scheme on random access. 0.2% bit rate reduction is obtained for high efficiency and low complexity. And encoding complexities are increased about 30% for both cases.

Table 1. BD-rate gain with HM3.2 for random access

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | -0.1 | -0.9 | -0.2 | 0.6 | -0.1 |
| Class B | -0.3 | -0.2 | -0.1 | -0.3 | 0.0 | -0.1 |
| Class C | -0.2 | -0.1 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class D | 0.0 | 0.0 | 0.1 | -0.1 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | -0.2 | -0.1 | -0.3 | -0.2 | 0.0 | -0.1 |
| Enc Time[%] | 128% |  |  | 130% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

The results of low delay coding structure are shown in Error: Reference source not found. Coding efficiency is improved 0.3% and 0.4% for high efficiency and low complexity, respectively. Encoding time is increased about 24%. And decoding time is almost same with that of anchor.

Table 2. BD-rate gain with HM3.2 for low delay

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.1 | 0.0 | -0.3 | 0.0 | 0.0 |
| Class C | -0.3 | -0.2 | -0.4 | -0.5 | -0.2 | -0.1 |
| Class D | -0.2 | -0.1 | -0.3 | -0.2 | 0.3 | 0.3 |
| Class E | -0.4 | 0.3 | -0.2 | -0.7 | 0.6 | -0.2 |
| Overall | -0.3 | -0.1 | -0.2 | -0.4 | 0.2 | 0.0 |
| Enc Time[%] | 123% |  |  | 124% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusions

In this contribution, the pixel based IC scheme is proposed. From the simulation results, the average BD-rate gains are 0.2%~0.4% for general sequences. The results show IC scheme can improve the coding efficiency even though there is no fading effect on test sequences.

# References

- P. Bordes, “Weighted Prediction,” JCTVC-E041, 5th JCT-VC Meeting, Geneva, Switzerland, Mar., 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-E700, 5th JCT-VC Meeting, Geneva, Switzerland, Mar., 2011.

# Patent rights declaration(s)

Sejong University and SK Telecom may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).