| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F418 |
| --- | --- |

| Title: | Joint coding of CU splitting flag and inter modes based on HM 3.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wen Zhang | Tel: Email: | +86-13528803560 zhang.wen4@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In HM 3.1, there is up to eight cases could be summarized for CU splitting flag and inter modes. Please refer to Table 1 for detailed information. To address this problem, a joint coding method which is different to CU-depth dependent and counter-based adaption (JCTVC-D370, JCTVC-E143) was proposed in this document. As test results reported, it has saved about 0.6% BD-rate for CAVLC configurations.

In this proposal, the representing method of CU splitting flag and inter modes had been unified both in CAVLC and CABAC. And about 0.1% BD-rate deviation was observed for coding efficiency of CABAC comparing to HM 3.1. It’s also proposed that IPCM could be enabled in intra slice while CU size greater than 16x16 and in inter slice while CU size greater than 8x8 (min CU) with a loss of 0.1% BD-rate in AI-LC configuration only.

Table 1 Case summary of CU splitting flag and inter modes

| PredMode and CU splitting | PartMode | Case and Symbol (cu_split_and_mode) |  |
| --- | --- | --- | --- |
|  |  | CU size > 8x8 (min) | CU size == 8x8(min) |
| SPLIT |  | 7 (SPLIT) | -- |
| IPCM (disabled) | PART_2Nx2N | 6 (IPCM) | -- |
| MODE_SKIP | PART_2Nx2N | 0 (SKIP) | 0 (SKIP) |
| MODE_MRG | PART_2Nx2N | 5 (MRG) | 5 (MRG) |
| MODE_INTER | PART_2Nx2N | 1 (INTER_2Nx2N) | 1 (INTER_2Nx2N) |
|  | PART_2NxN | 3 (INTER_2NxN) | 3 (INTER_2NxN) |
|  | PART_Nx2N | 4 (INTER_Nx2N) | 4 (INTER_Nx2N) |
|  | PART_NxN | -- | 6 (INTER_NxN) |
| MODE_INTRA | PART_2Nx2N | 2 (INTRA_2Nx2N) | 2 (INTRA_2Nx2N) |
|  | PART_NxN | -- | 7 (INTRA_NxN) |

# Introduction

In JCTVC-D275 and JCTVC-E072, slice-based adaptive representing methods of syntax elements related to inter modes were proposed. In JCTVC-E258, a context-dependent classification method was introduced as supplementary part of CU-depth dependent while coding CU splitting flag and inter modes. In this proposal, above basic ideas were harmonized together and further studied.

First of all, CU splitting flag and inter modes were coded in 16 types. It included 10 types for CU size greater than 8x8 (min) and 6 types for CU size equals to 8x8 (min). And it’s classified according to CU splitting and prediction mode of left and above CU. Please refer to Table 2 for detailed coding types of CU splitting flag and inter modes.

Table 2 Coding types of CU splitting flag and inter modes

| CU-depth | Prediction mode-top CU | Prediction mode-left CU | Coding Type |
| --- | --- | --- | --- |
| CU size > 8x8 (min) | SKIP | SKIP | 0 |
|  | INTER | SKIP | 1 |
|  | SKIP | INTER |  |
|  | INTRA | SKIP | 2 |
|  | SKIP | INTRA |  |
|  | CU Splitting | SKIP | 3 |
|  | SKIP | CU Splitting |  |
|  | INTER | INTER | 4 |
|  | INTRA | INTER | 5 |
|  | INTER | INTRA |  |
|  | CU Splitting | INTER | 6 |
|  | INTER | CU Splitting |  |
|  | INTRA | INTRA | 7 |
|  | CU Splitting | INTRA | 8 |
|  | INTRA | CU Splitting |  |
|  | CU Splitting | CU Splitting | 9 |
| CU size==8x8(min) | SKIP | SKIP | 10 |
|  | INTER | SKIP | 11 |
|  | SKIP | INTER |  |
|  | INTRA | SKIP | 12 |
|  | SKIP | INTRA |  |
|  | INTER | INTER | 13 |
|  | INTRA | INTER | 14 |
|  | INTER | INTRA |  |
|  | INTRA | INTRA | 15 |

Secondly, the codeword tables of above 16 coding types were updated for each inter slice. Based on statistical results of previous coded slices, all counter results of each type were sorted to refresh codeword tables for next slice with same slice QP. And the codeword tables of all coding types were maintained to be same both in encoder and decoder. Thus, there is no need to transmit explicit coding types which were described in JCTVC-D275 and JCTVC-E072.

Thirdly, unified representing method of CU splitting flag and inter modes had been implemented both in CABAC and CAVLC. The motivation was to reduce flags / bins needed to code these syntax elements.

The fourth, A Pred_split_flag was defined while CU-depth equals to 0 or 1. If this flag equals to 1, it means not only CU of current depth with split_flag equals to 1 but also all CUs here of current depth plus one with split_flags equal to 1. In order to avoid worse coding result, constrained conditions were added to control the presence of pred_split_flag in each slice.

Finally, IPCM mode proposed by JCTVC-E057 was incorporated into above joint coding method. It was proposed that IPCM could be enabled in intra slice while CU size greater than 16x16 and in inter slice while CU size greater than 8x8 (min). The impact on coding efficiency was observed only in AI-LC configuration, with a loss of 0.1% BD-rate.

- Experimental results

The proposed method was evaluated using the recommended common test condition presented in JCTVC-E700. All configurations were tested in the experiment with anchor HM 3.1. The test platform was HP server E5506/2.13 GHz with Windows NT server 2003 32 bits OS. The anchor we got in this proposal on above platform was identical to results provided by NNT/BBC.

Two software versions had been planned to be tested for most configurations, Version A (without pred_split_flag) and Version B (with pred_split_flag). The evaluation results were shown as below. And, please be noted that the green data were cross checked and match results were reported.

Software Version A, related results were included from Table 3 to Table 5.

Table 3 test results of Version A, default with IPCM disabled [anchor HM 3.1]

| All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- |
| Y | U | V | Y | U | V |
| 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Random access HE |  |  | Random access LC |  |  |
| Y | U | V | Y | U | V |
| -0.1 | -0.2 | -0.2 | -0.5 | -0.4 | -0.4 |
| Low delay (B) HE |  |  | Low delay (B) LC |  |  |
| Y | U | V | Y | U | V |
| 0.0 | 0.2 | 0.1 | -0.6 | -0.6 | -0.8 |
| Low delay (P) HE |  |  | Low delay (P) LC |  |  |
| Y | U | V | Y | U | V |
| 0.2 | 0.5 | 0.2 | -0.6 | -0.5 | -0.7 |

Table 4 test results of Version A, IPCM enabled while CU size > 8x8 (min) [anchor HM 3.1]

| All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- |
| Y | U | V | Y | U | V |
| 0.0 | -0.1 | -0.1 | 0.5 | 0.6 | 0.6 |
| Random access HE |  |  | Random access LC |  |  |
| Y | U | V | Y | U | V |
| -0.1 | -0.1 | -0.3 | -0.4 | -0.3 | -0.3 |
| Low delay (B) HE |  |  | Low delay (B) LC |  |  |
| Y | U | V | Y | U | V |
| 0.0 | 0.3 | 0.0 | -0.6 | -0.5 | -0.8 |
| Low delay (P) HE |  |  | Low delay (P) LC |  |  |
| Y | U | V | Y | U | V |
| 0.1 | 0.3 | 0.2 | -0.6 | -0.2 | -0.7 |

Table 5 test results of Version A, IPCM enabled in intra slice while CU size greater than 16x16 and in inter slice while CU size greater than 8x8 (min CU) [anchor HM 3.1] ---proposed.

| All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- |
| Y | U | V | Y | U | V |
| 0.0 | -0.1 | -0.1 | 0.1 | 0.1 | 0.1 |
| Random access HE |  |  | Random access LC |  |  |
| Y | U | V | Y | U | V |
| -0.1 | -0.1 | -0.2 | -0.5 | -0.4 | -0.4 |
| Low delay (B) HE |  |  | Low delay (B) LC |  |  |
| Y | U | V | Y | U | V |
| 0.1 | 0.3 | -0.1 | -0.6 | -0.8 | -0.8 |
| Low delay (P) HE |  |  | Low delay (P) LC |  |  |
| Y | U | V | Y | U | V |
| 0.1 | 0.0 | 0.0 | -0.6 | -0.6 | -0.8 |

Software Version B, related evaluations were included in Table 6 and Table 7.

Table 6 test results of Version B, default with IPCM disabled [anchor HM 3.1]

| All intra LC |  |  |
| --- | --- | --- |
| Y | U | V |
| -0.1 | -0.1 | -0.1 |
| Random access LC |  |  |
| Y | U | V |
| -0.6 | -0.4 | -0.4 |
| Low delay (B) LC |  |  |
| Y | U | V |
| -0.6 | -0.6 | -0.8 |
| Low delay (P) LC |  |  |
| Y | U | V |
| -0.7 | -0.7 | -0.9 |

Table 7 test results of Version B, IPCM enabled while CU size > 8x8 (min) [anchor HM 3.1]

| All intra LC |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.4 | 0.5 | 0.5 |
| Random access LC |  |  |
| Y | U | V |
| -0.5 | -0.3 | -0.3 |
| Low delay (B) LC |  |  |
| Y | U | V |
| -0.7 | -0.5 | -0.9 |
| Low delay (P) LC |  |  |
| Y | U | V |
| -0.7 | -0.5 | -0.8 |

Table 8 Analysis of proposed IPCM, impact on coding efficiency [Table 5 vs. Table 3]

| All intra HE |  |  | All intra LC |  |  |
| --- | --- | --- | --- | --- | --- |
| 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 |
| Random access HE |  |  | Random access LC |  |  |
| 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Low delay (B) HE |  |  | Low delay (B) LC |  |  |
| 0.0 | 0.1 | -0.2 | 0.0 | -0.2 | 0.0 |
| Low delay (P) HE |  |  | Low delay (P) LC |  |  |
| 0.0 | -0.5 | -0.2 | 0.0 | 0.0 | -0.1 |

Table 9 Analysis of pred_split_flag, impact on coding efficiency [Table 6 vs. Table 3]

| All intra LC |  |  |
| --- | --- | --- |
| -0.1 | -0.1 | -0.1 |
| Random access LC |  |  |
| -0.1 | -0.1 | -0.1 |
| Low delay (B) LC |  |  |
| 0.0 | 0.0 | 0.0 |
| Low delay (P) LC |  |  |
| 0.0 | -0.2 | -0.2 |

# Conclusions

Since IPCM mode had been adopted into HM 3.1 which was disabled by default. From the test results reported in Table 5, the best choice to enable IPCM mode should be Intra Slice with CU size greater than 16x16, Inter Slice with CU size greater than 8x8 (min). It’s suggested to adopt this into HM.

The joint coding of CU splitting flag and inter modes could improve about 0.5% BD-rate for CAVLC configurations comparing to HM 3.1. The above mentioned pred_split_flag brought further 0.1% BD-rate gain for AI-LC and RA-LC configurations. And the enc/dec time and memory consumption were kept almost unchanged comparing to anchor. It’s suggested to adopt the above change of CAVLC into HM.

From test results, it was reported that CABAC itself did good job while coding these syntax elements. There is almost no gain observed for coding efficiency (with 0.1% BD-rate deviation, some became worse). But for sake of unification and further investigation, it’s also recommended to consider this part (whole scheme of Table 5) adopted into HM.

# Acknowledgement

Many thanks to MediaTek and LGE to do the cross check work for this proposal. Please refer to JCTVC-F117 by LGE for cross check report of software Version A, RA_LC and LD_LC configurations. JCTVC-F279 provided by MediaTek had reported cross check results of software Version B, all CAVLC configurations.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed text

Syntax element cu_split_and_mode (PredMode, PartMode) was defined in Table 1. It could help us to unify the Spec text in coding_tree(), coding_unit() and prediction_unit(), as shown below (revised from JCTVC-E603-v8).

| coding_tree( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress ) { |  |
| cu_split_and_mode[x0][y0] | ce(v)¦u(1)¦ae(v) |
| } |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |
| cuDepth = Log2MaxCUSize – log2CUSize |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |
| cu_split_and_mode[x0][y0] != SPLIT ) |  |
| AlfCuFlagIdx++ |  |
| } |  |
| if( cu_split_and_mode[x0][y0] == SPLIT ) { |  |
| if( cu_qp_depta_enabled_flag && log2CUSize == log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CUSize – 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CUSize − 1 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CUSize − 1 ) |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CUSize − 1 ) |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| coding_unit( x0, y0, log2CUSize ) |  |
| if( !entropy_coding_mode_flag ) |  |
| moreDataFlag = more_rbsp_data( ) |  |
| else { |  |
| if( granularity_block_boundary( x0, y0, log2CUSize ) ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| } |  |
| return moreDataFlag |  |
| } |  |

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( PartMode == PART_2Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize, 0 , 0 ) |  |
| } else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, log2CUSize, log2CUSize – 1, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x0, y1, log2CUSize, log2CUSize – 1, 1 , 0 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, log2CUSize - 1, log2CUSize, 0 , log2CUSize > Log2MinCUSize ) |  |
| prediction_unit( x1, y0, log2CUSize - 1, log2CUSize, 1 , 0 ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, log2CUSize – 1, log2CUSize – 1, 0 , 0 ) |  |
| prediction_unit( x1, y0, log2CUSize – 1, log2CUSize – 1, 1 , 0 ) |  |
| prediction_unit( x0, y1, log2CUSize – 1, log2CUSize – 1, 2 , 0 ) |  |
| prediction_unit( x1, y1, log2CUSize – 1, log2CUSize – 1, 3 , 0 ) |  |
| } |  |
| if( PredMode != MODE_SKIP && PredMode != IPCM ) { |  |
| transform_tree( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 0 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 1 ) |  |
| transform_coeff( x0, y0, log2CUSize, 0, 2 ) |  |
| } |  |
| } |  |

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( PredMode == MODE_SKIP ¦¦ PredMode == MODE_MRG ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if ( PredMode == IPCM ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else if( PredMode == MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| Else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) { |  |
| if(!entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag && use_combined_inter_pred_ref( x0, y0 ) ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |