| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F420r1 |
| --- | --- |

| Title: | CE4: Result of 2.3.d and 2.3.e |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan | Tel: Email: | +81 3 5435 3308 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document provides result of CE4: 2.3.d and 2.3.e.

# Introduction

At the 5th JCTVC Mtg Sub-LCU-Level delta-QP coding has been adopted. As of QP prediction simple method similar to the one in AVC was adopted, but there have been several proposals on advanced QP prediction. The topic of CE4 subtest 2 is investigation of such advanced QP prediction, and Sony is one of the proponents.

This document provides result of CE4: 2.3.d and 2.3.e described in [3].

# Algorithm Description

The following algorithms are applied to generate predQP (predicted QP value) with test 2.3.d and 2.3.e respectively. Both methods use QP value of neighboring blocks (left, above and aboveleft).

[2.3.d]

if (Left and Above and AboveLeft) predQP=Median(Left, Above, AboveLeft)

else if (Left and Above) predQP=(Left + Above) / 2

else if (Left and Aboveleft) predQP=(Left + Aboveleft) / 2

else if (Above and Aboveleft) predQP=(Above + Aboveleft) / 2

else if (Left) predQP=Left

else if (Above) predQP=Above

else if (AboveLeft) predQP=Aboveleft

else predQP=prevQP

[2.3.e]

if (Left and Above) predQP=(Left + Above) / 2

else if (Left and Aboveleft) predQP=(Left + Aboveleft) / 2

else if (Above and Aboveleft) predQP=(Above + Aboveleft) / 2

else if (Left) predQP=Left

else if (Above) predQP=Above

else if (AboveLeft) predQP=Aboveleft

else predQP=prevQP

# Simulation

## Simulation Condition

The followings are description of anchor and to be tested:

[Anchor]

HM-3.0-dev-r863-CE4_r2 (Distributed to CE4 participants as the anchor)

[To be tested]

The proposed method is implemented by the proponent of top of the anchor software.

In “Typedef.h” macro setting for each test are as follows:

(2.3d)

#define SNY_CE4_SUBTEST2                  1

#define SNY_CE4_SUBTEST2_MEDIAN_DISABLE   0

(2.3e)

#define SNY_CE4_SUBTEST2                  1

#define SNY_CE4_SUBTEST2_MEDIAN_DISABLE   1

Intra, Intra_LC, Low-delay, Low-delay_LC, Random-access, and Random-access_LC conditions have been tested as described in [1].

The value of MaxCuDepthDQP is set as 3 (8x8).

## Simulation Result

Summary of the result with 2.3.d and 2.3.e are shown in Table 3.2.1 and Table 3.2.2 respectively.

Complete results are shown in JCTVC-F420_2.3.d.xls and JCTVC-F420_2.3.e.xls.

The result has been verified by ETRI [4] and TI [5].

Table 3.2.1

|  | Intra |  |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.2 | 0.2 | 0.2 | 5.7% | 0.2 | 0.2 | 0.2 | 7.3% |
| Class B | 0.1 | 0.1 | 0.1 | 3.2% | 0.1 | 0.1 | 0.1 | 3.3% |
| Class C | -0.1 | -0.1 | 0.0 | -1.9% | -0.1 | -0.1 | -0.1 | -1.6% |
| Class D | -0.1 | -0.1 | 0.0 | -1.5% | -0.1 | -0.1 | -0.1 | -1.1% |
| Class E | 0.1 | 0.1 | 0.1 | 2.9% | 0.1 | 0.1 | 0.1 | 2.9% |
| All | 0.0 | 0.1 | 0.1 | 1.7% | 0.1 | 0.1 | 0.1 | 2.2% |
| Enc Time[%] | 98% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |
|  | Random access |  |  |  | Random access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | 0.1 | -0.1 | 2.9% | 0.2 | 0.1 | 0.3 | 4.2% |
| Class B | 0.0 | 0.1 | 0.0 | 0.6% | 0.0 | 0.0 | 0.0 | 0.9% |
| Class C | -0.1 | 0.0 | -0.2 | -3.2% | -0.2 | -0.1 | -0.1 | -3.6% |
| Class D | 0.0 | -0.2 | 0.0 | -2.4% | -0.1 | -0.1 | -0.2 | -2.6% |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | -0.4% | 0.0 | 0.0 | 0.0 | -0.2% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |
|  | Low delay |  |  |  | Low delay LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | -0.4% | 0.0 | 0.1 | -0.1 | 0.0% |
| Class C | -0.2 | -0.1 | -0.2 | -3.7% | -0.2 | -0.1 | -0.2 | -4.2% |
| Class D | -0.1 | -0.3 | 0.1 | -3.0% | -0.2 | -0.3 | -0.4 | -3.2% |
| Class E | -0.1 | 0.4 | -0.8 | -3.2% | -0.2 | -0.3 | -0.4 | -2.7% |
| All | -0.1 | -0.1 | -0.2 | -2.4% | -0.2 | -0.1 | -0.2 | -2.4% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |

Table 3.2.2

|  | Intra |  |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | -0.1 | 0.0 | -1.7% | 0.1 | 0.1 | 0.1 | 2.2% |
| Class B | -0.2 | -0.1 | -0.1 | -4.6% | 0.0 | 0.0 | 0.0 | -1.3% |
| Class C | -0.3 | -0.3 | -0.3 | -8.4% | -0.2 | -0.2 | -0.2 | -5.5% |
| Class D | -0.3 | -0.3 | -0.3 | -8.7% | -0.3 | -0.2 | -0.3 | -6.8% |
| Class E | -0.2 | -0.2 | -0.3 | -5.4% | -0.1 | -0.1 | -0.1 | -2.0% |
| All | -0.2 | -0.2 | -0.2 | -5.7% | -0.1 | -0.1 | -0.1 | -2.6% |
| Enc Time[%] | 98% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 100% |  |  |  |
|  | Random access |  |  |  | Random access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.1 | -0.1 | -0.2 | -1.9% | 0.0 | 0.1 | 0.1 | 0.9% |
| Class B | -0.1 | -0.1 | -0.2 | -3.6% | -0.1 | -0.1 | -0.1 | -1.4% |
| Class C | -0.3 | -0.3 | -0.2 | -7.3% | -0.3 | -0.3 | -0.2 | -6.4% |
| Class D | -0.2 | -0.2 | -0.1 | -6.5% | -0.2 | -0.2 | -0.3 | -6.4% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -0.2 | -0.2 | -4.8% | -0.1 | -0.1 | -0.1 | -3.2% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |
|  | Low delay |  |  |  | Low delay LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.2 | -3.3% | 0.0 | -0.1 | -0.2 | -1.6% |
| Class C | -0.3 | -0.3 | -0.3 | -7.3% | -0.4 | 0.0 | -0.3 | -7.0% |
| Class D | -0.2 | -0.4 | 0.0 | -6.4% | -0.3 | -0.2 | -0.5 | -6.7% |
| Class E | -0.2 | 0.2 | 0.2 | -6.5% | -0.2 | -0.3 | -0.2 | -5.7% |
| All | -0.2 | -0.2 | -0.1 | -5.6% | -0.2 | -0.1 | -0.3 | -5.0% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  |

# Conclusion

This document provides result with 2.3.d and 2.3.e as described in [].

The proposed method 2.3.e contributes to -0.2%, -0.1%, -0.2%, -0.1%, -0.2% and -0.2% bit-rate saving or -5.7%, -2.6%, -4.8%, -3.2%, -5.6% and -5.0% delta-qp bit saving for IT, IT_Loco, RA, RA_Loco, LD, LD_Loco conditions. Increase in enc/dec time is at ignorable level.

The author recommends adopting 2.3.e method harmonizing with other proposals in CE4.

# References

[1] “WD3: Working Draft 3 of High-Efficiency Video Coding”, JCTVC-E603, Mar 2011

[2] “Common test conditions and software reference configurations”, JCTVC-E700, Mar 2011

[3] “Description of Core Experiment 4: Quantization”, JCTVC-E704, Mar 2011

[4] “CE4 Subtest2: Cross-check report of Sony’s proposal JCTVC-F420 (tests 2.3.d and 2.3.e) by ETRI”, JCTVC-F355, July 2011

[5] “CE4: Crosscheck of Sony proposal on dQP prediction (2.3e mod) (JCTVC-Fxxx)”, JCTVC-F238, July 2011

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).