| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F422r1 |
| --- | --- |

| Title: | Improvement of delta-QP Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Kazushi Sato W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan Jun Xu 1730 N 1st Street San Jose, CA 95112 | Tel: Email: | +81-3-5435-3308 Kenji.Kondo@jp.sony.com Kazushi.Sato@jp.sony.com +1-408- 352-4710 Jun.Xu@am.sony.com |
| Source: | Sony Corp. / Sony Electronics Inc. |  |  |

_____________________________

# Abstract

At the 5th JCTVC Mtg Sub-LCU-Level delta-QP coding has been adopted. As of QP prediction simple method similar to the one in AVC was adopted, but there have been several proposals on advanced QP prediction. One of the topics of CE4 is investigation of such advanced QP prediction, and Sony is one of the proponents.

This proposal contains two modifications of Sony’s proposal on CE4: 1) modification on the definition of neighboring QP availability, and 2) entropy coding of delta-QP.

Together with Sony’s CE4 proposal 2.3.e, the proposed method 1) provides gain by -0.21%, -0.14%, -0.24%, -0.23%, -0.35% for IT, IT_Loco, RA, RA_Loco, LD, and LD_Loco conditions respectively compared with CE4 anchor. The proposed method 1) + 2) + provides gain by -0.25%, -0.26%, -0.45% for IT, RA and LD conditions respectively.

The proposed medhod 1) is also tested on top of combination of CE4: 2.3.g + 2.3.f + 2.3.e, which is a best combination among CE4 subtest2 proposals other than the temporal QP prediction. Additional gain is obtained by 0.01%, 0.00%, -0.05%, -0.01%, -0.11% and -0.11% with IT, IT_Loco, RA, RA_Loco, LD and LD_Loco conditions respectively.

# Introduction

HEVC [1] employees Coding Unit structure, whose main difference from Macroblock in the previous codec MPEG-2 or AVC, whose size is fixed to 16x16, is that its size can be up to 128x128. One LCU (largest coding unit) would have both flat area and busy area, so having a single QP value for one LCU is not sufficient to obtain good subjective quality.

To solve this problem Sub-LCU-Level delta-QP coding has been proposed [4] [5] and adopted at the 5th JCTVC Mtg. Delta-QP can be defined CUs with the size of either 8x8, 16x16, 32x32 or 64x64. As of the QP prediction scheme simple method similar to the one in AVC has been adopted, but there have been several proposals on advanced QP prediction and this topic would be investigated in CE4 [3]. Sony is one of the proponents.

In this document it is shown that further gain is obtained with 2 modifications: 1) modification on the definition of neighboring QP availability, and 2) delta-QP entropy coding.

# Algorithm Description

## Sony’s Proposal on QP Prediction for CE4

Sony’s proposal (2.3.e) on QP prediction in CE4 [6] is originally as follows:

if (Left and Above) predQP=(Left + Above) / 2

else if (Left and Aboveleft) predQP=(Left + Aboveleft) / 2

else if (Above and Aboveleft) predQP=(Above + Aboveleft) / 2

else if (Left) predQP=Left

else if (Above) predQP=Above

else if (AboveLeft) predQP=Aboveleft

else predQP=prevQP

In addition to this algorithm the following algorithms for further improvement in coding efficiency are introduced as new proposals.

## Modification on the Definition of Neighboring QP Availability

In Section 2.1, the neighboring CU is defined as not available when it is outside of the current slice, entropy slice or frame. In this proposal, checking for CBP of the neighboring CU is added if the neighboring CU is available. This is because QP of a PU is not reliable as its CBP is zero. The new QP prediction is as follows.

if (Left and Above and Left.cbp != 0 and Above.cbp != 0) predQP=(Left + Above) / 2

else if (Left and Aboveleft and Left.cbp != 0 and Aboveleft.cbp != 0) predQP=(Left + Aboveleft) / 2

else if (Above and Aboveleft and Above.cbp != 0 and Aboveleft.cbp != 0) predQP=(Above + Aboveleft) / 2

else if (Left and Left.cbp != 0) predQP=Left

else if (Above and Above.cbp != 0) predQP=Above

else if (Aboveleft and Aboveleft.cbp != 0) predQP=Aboveleft

else predQP=prevQP

## Delta-QP Entropy Coding

In HEVC test Model (HM) 3.0, the dQP encoding process in CABAC consists of two steps:

1) 	A flag to indicate whether dQP is zero or not.

2) 	If dQP is nonzero, the signed dQP is mapped to an unsigned codenumber and the unsigned codenumber is mapped to a bin string using unary codes.

The approach in HM3.0 assigns different lengths to nonzero dQP with the same absolute values. Statistics demonstrate that the distribution of dQP has symmetric property, which is that nonzero dQPs with the same absolute values but different signs turn to have similar probabilities.

To fit the true distribution of dQP, this proposal changes the binarization of dQP in CABAC. The encoding dQP still has two steps as HM3.0 but with a different step 2) as follows.

1) A flag to indicate whether dQP is zero or not.

2) If dQP is nonzero, the absolute value of dQP is mapped to a bin string using unary codes. The sign of dQP is then encoded.

# Simulation

## Simulation -1

### Simulation Condition

The followings are the description of “anchor”, “to be tested-1” and “to be tested-2”:

[Anchor]

HM-3.0-dev-r863-CE4_r2 (Distributed to CE4 participants as the anchor) [3]

[To be tested -1]

Anchor + 2.1&2.2

[To be tested -2]

Anchor + 2.1&2.2&2.3

With “to be tested -1”, Intra, Intra_LC, Low-delay, Low-delay_LC, Random-access, and Random-access_LC conditions have been tested as described in [2].

With “to be tested -2”, Intra, Low-delay, and Random-access conditions have been tested as described in [2], as the proposed method 2.3 is applied only with CABAC.

### Simulation Result

In Table 3.1.2.1 summary of the results with “to be tested -1” are shown. Complete results are shown in JCTVC-E422_1.xls. As can be seen gain is obtained by -0.21%, -0.14%, -0.24%, -0.23%, -0.35% for IT, IT_Loco, RA, RA_Loco, LD, and LD_Loco conditions respectively.

In Table 3.1.2.2 summary of the results with “to be tested -2” are shown. Complete results are shown in JCTVC-E422_2.xls. As can be seen gain is obtained by -0.25%, -0.26%, -0.45% for IT, RA and LD conditions respectively.

The result is verified by Canon [7].

Table 3.1.2.1

|  | Intra |  |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.05 | -0.08 | -0.05 | -1.9% | 0.04 | 0.04 | 0.04 | 1.2% |
| Class B | -0.15 | -0.12 | -0.15 | -4.7% | -0.08 | -0.08 | -0.08 | -2.2% |
| Class C | -0.31 | -0.30 | -0.31 | -8.4% | -0.24 | -0.23 | -0.23 | -6.3% |
| Class D | -0.30 | -0.31 | -0.30 | -8.6% | -0.26 | -0.26 | -0.26 | -6.9% |
| Class E | -0.29 | -0.40 | -0.34 | -6.1% | -0.18 | -0.18 | -0.18 | -4.1% |
| All | -0.21 | -0.23 | -0.22 | -5.9% | -0.14 | -0.14 | -0.14 | -3.6% |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Random access |  |  |  | Random access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.16 | -0.05 | -0.57 | -3.5% | -0.10 | -0.05 | -0.07 | -2.0% |
| Class B | -0.24 | -0.34 | -0.22 | -6.6% | -0.20 | -0.21 | -0.20 | -5.2% |
| Class C | -0.34 | -0.22 | -0.35 | -8.8% | -0.33 | -0.31 | -0.25 | -8.3% |
| Class D | -0.22 | -0.40 | -0.14 | -7.8% | -0.31 | -0.20 | -0.29 | -7.6% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.24 | -0.26 | -0.32 | -6.7% | -0.23 | -0.19 | -0.20 | -5.8% |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Low delay |  |  |  | Low delay LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.33 | -0.31 | -0.36 | -7.2% | -0.29 | -0.28 | -0.49 | -6.4% |
| Class C | -0.39 | -0.21 | -0.25 | -9.1% | -0.46 | -0.23 | -0.51 | -9.1% |
| Class D | -0.31 | -0.07 | -0.38 | -7.7% | -0.44 | -0.08 | -0.58 | -8.0% |
| Class E | -0.40 | -0.01 | -0.67 | -9.3% | -0.52 | -0.78 | -0.53 | -9.2% |
| All | -0.35 | -0.17 | -0.39 | -8.2% | -0.41 | -0.31 | -0.52 | -8.0% |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |

Table 3.1.2.2

|  | Intra |  |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.07 | -0.12 | -0.13 | -2.6% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class B | -0.18 | -0.16 | -0.18 | -5.0% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class C | -0.34 | -0.34 | -0.30 | -8.9% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class D | -0.34 | -0.35 | -0.33 | -9.5% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class E | -0.35 | -0.41 | -0.33 | -6.9% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| All | -0.25 | -0.26 | -0.25 | -6.5% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Random access |  |  |  | Random access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.13 | -0.41 | -0.19 | -4.7% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class B | -0.29 | -0.24 | -0.25 | -7.4% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class C | -0.35 | -0.32 | -0.42 | -10.1% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class D | -0.27 | -0.22 | -0.30 | -9.3% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.26 | -0.30 | -0.29 | -7.9% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Low delay |  |  |  | Low delay LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.35 | -0.47 | -0.65 | -8.2% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class C | -0.53 | -0.31 | -0.41 | -10.5% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class D | -0.39 | -0.25 | -0.06 | -9.6% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Class E | -0.61 | 0.05 | -0.81 | -11.4% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| All | -0.45 | -0.28 | -0.47 | -9.7% | #VALUE! | #VALUE! | #VALUE! | 0.0% |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |

## Simulation -2

### Simulation Condition

To show that the proposed method can be combined with other dQP coding method integration with the software on CE4: 2.3g&2.3f&2.3e [8] is tested.

[Anchor]

Software included in JCTVC-F661

[To be tested]

Software included in JCTVC-F661 + 2.2

### Simulation Result

In Table 3.2.2.1 summary of the result is shown. Complete result is shown in JCTVC-E422_2.2_F661.xls. As can be seen additional gain is obtained by 0.01%, 0.00%, -0.05%, -0.01%, -0.11% and -0.11% with IT, IT_Loco, RA, RA_Loco, LD and LD_Loco conditions respectively..

Table 3.2.2.1

|  | Intra |  |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.00 | 0.01 | -0.09 | 0.1% | 0.00 | 0.00 | 0.00 | 0.1% |
| Class B | 0.01 | 0.02 | -0.01 | 0.2% | 0.01 | 0.00 | 0.00 | 0.2% |
| Class C | 0.01 | 0.01 | -0.01 | 0.2% | 0.00 | 0.00 | 0.00 | 0.1% |
| Class D | 0.01 | 0.00 | 0.00 | 0.1% | 0.01 | 0.01 | 0.01 | 0.2% |
| Class E | 0.01 | 0.00 | -0.01 | 0.1% | 0.01 | 0.01 | 0.01 | 0.2% |
| All | 0.01 | 0.01 | -0.02 | 0.1% | 0.00 | 0.00 | 0.00 | 0.1% |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Random access |  |  |  | Random access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.09 | -0.13 | -0.40 | -0.7% | 0.03 | 0.00 | 0.02 | -1.0% |
| Class B | -0.09 | -0.08 | -0.16 | -1.8% | -0.04 | -0.08 | -0.04 | -1.6% |
| Class C | -0.01 | -0.03 | -0.05 | -0.5% | 0.00 | -0.01 | 0.01 | -0.2% |
| Class D | 0.01 | 0.00 | -0.06 | -0.3% | 0.00 | 0.08 | -0.01 | -0.1% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.05 | -0.06 | -0.17 | -0.9% | -0.01 | -0.01 | -0.01 | -0.8% |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Low delay |  |  |  | Low delay LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.16 | -0.13 | 0.01 | -2.5% | -0.14 | -0.23 | -0.29 | -2.5% |
| Class C | -0.08 | -0.07 | -0.10 | -0.6% | -0.06 | -0.10 | -0.01 | -0.6% |
| Class D | -0.04 | -0.63 | -0.64 | -0.4% | -0.06 | 0.22 | 0.03 | -0.3% |
| Class E | -0.19 | 1.01 | -0.60 | -0.8% | -0.22 | -0.54 | -0.27 | -1.4% |
| All | -0.11 | -0.03 | -0.29 | -1.2% | -0.11 | -0.14 | -0.14 | -1.3% |
| Enc Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |

# Conclusion

On top of Sony’s original CE4 proposal, this document proposes further modifications for coding efficiency improvement: 1) modification on the definition of neighboring QP availability, and 2) delta-QP entropy coding.

Together with Sony’s CE4 proposal 2.3.e, the proposed method 1) provides gain by -0.21%, -0.14%, -0.24%, -0.23%, -0.35% for IT, IT_Loco, RA, RA_Loco, LD, and LD_Loco conditions respectively compared with CE4 anchor. The proposed method 1) + 2) + provides gain by -0.25%, -0.26%, -0.45% for IT, RA and LD conditions respectively.

The proposed medhod 1) is also tested on top of combination of CE4: 2.3.g + 2.3.f + 2.3.e, which is the best spatial QP prediction method among CE4 subtest2 proposals. Additional gain is obtained by 0.01%, 0.00%, -0.05%, -0.01%, -0.11% and -0.11% with IT, IT_Loco, RA, RA_Loco, LD and LD_Loco conditions respectively.

The authors recommend to adopt by harmonizing with the QP prediction method, or to investigate further in CE on Quantization.

# References

[1] “WD3: Working Draft 3 of High-Efficiency Video Coding”, JCTVC-E603, Mar 2011

[2] “Common test conditions and software reference configurations”, JCTVC-E700, Mar 2011

[3] “Description of Core Experiment 4: Quantization”, JCTVC-E704, Mar 2011

[4] “Support for Sub-LCU-Level QP in HEVC”, JCTVC-E202, Mar 2011

[5] “Preliminary Implementation on Sub-LCU-Level DeltaQP”, JCTVC-E220 Mar 2011

[6] “CE4: Result of 2.3.d and 2.3.e”, JCTVC-F420, Jul 2011

[7] “Verification result of Sony's Improvement of delta-QP Coding (JCTVC-F422)”, JCTVC-F481, Jul 2011

[8] “CE4 Subtest 2: Spatial QP prediction: combination of test 2.3.g, 2.3.f and 2.3.e”, JCTVC-F661, Jul 2011

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).