| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F423 |
| --- | --- |

| Title: | Modified MVD coding for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma, Osaka, Japan | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This proposal presents a technique for complexity reduction on the motion vector difference (MVD) parameter parsing process. In this contribution, it is proposed that the data structure change to increase parallel processing capabilities and modified context selection to reduce the dependency on neighboring blocks to reduce the line buffer are used for MVD coding. Moreover, bypass processing concatenation makes parallel processing capability further increased. The proposed solutions were implemented in HMv3 and their coding efficiencies were evaluated. The average gain for the proposal for random access and low-delay configurations is 0.02% and 0.02%, respectively.

# Introduction

The current binarization process and derivation process of context index of  MVD syntax elements (mvd_l0[ ][ ][0], mvd_l1[ ][ ][0], mvd_lc[ ][ ][0], mvd_l0[ ][ ][1], mvd_l1[ ][ ][1] and  mvd_lc[ ][ ][1] ) is specified for horizontal difference (...[0]) and vertical difference (...[1]) separately.

Table 1 shows the association for contexts and bins. The derivation process of context index for 1st bit of MVD syntax elements is defined to use neighboring data which is the condition of above and left block results. Table 2 shows the derivation process from current specification [1].

Table 1. the association for contexts and bins

|  |  | prefix |  |  |  |  |  |  |  |  | suffix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdx |  | 0..2 | 3 | 4 | 5 | 6 |  |  |  |  | bypass |
| MVD | 0 | 0 |  |  |  |  |  |  |  |  |  |
|  | 1 | 1 | 0 |  |  |  |  |  |  |  |  |
|  | 2 | 1 | 1 | 0 |  |  |  |  |  |  |  |
|  | 3 | 1 | 1 | 1 | 0 |  |  |  |  |  |  |
|  | 4 | 1 | 1 | 1 | 1 | 0 |  |  |  |  |  |
|  | 5 | 1 | 1 | 1 | 1 | 1 | 0 |  |  |  |  |
|  | 6 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |  |  |
|  | 7 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |  |
|  | 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |  |
|  | 9… | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | ExG:3 |

Table 2. Specification of MVD context derivation process

| Syntax element | condL | condA | ctxIdxInc |
| --- | --- | --- | --- |
| mvd_lc | mvd_lc[ xL ][ yL ] > 16 | mvd_lc[ xA ][ yA ] > 16 | ( condL && availableL ) + ( condA && availableA ) |
| mvd_l0 | mvd_l0[ xL ][ yL ] > 16 | mvd_l0[ xA ][ yA ] > 16 | ( condL && availableL ) + ( condA && availableA ) |
| mvd_l1 | mvd_l1[ xL ][ yL ] > 16 | mvd_l1[ xA ][ yA ] > 16 | ( condL && availableL ) + ( condA && availableA ) |

# Technical description

The line buffer is required for using neighboring data as discussed in [2]. Not to use neighboring results, only one context index is used for 1st bin coding which is also presented in JCTVC-429 [3]. In addition to the context selection, bypass processing concatenation makes parallel processing capability increase.

In addition to the line buffer, CABAC operation has high dependency for each process due to range updates and context updates. It is discussed that bypass coding bits can be operated by parallel processing and the bypass process concatenation is desirable to reduce the complexity for CABAC [4]. Figure 1 shows proposed data structure. Firstly, the prefix bins for both horizontal and vertical mvd is coded with general CABAC operation. Then, bypass coding parts which is suffix part by Exp-Golomb binarization process and sign flag are coded. Text for Working Draft is in Annex 1.

Figure 1. Proposed data structure

# Simulation Results

The proposal method is implemented in HM3.0 and checked under the common test condition described in JCTVC-E700 [5]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. The simulation results will be updated when the simulation is finished.

Table 3. BD BR for modified MVD context derivation (high efficiency)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.06 | -0.15 | -0.14 |  |  |  |
| Class B | 0.00 | -0.03 | -0.02 | -0.01 | 0.17 | 0.13 |
| Class C | 0.00 | 0.11 | -0.01 | -0.02 | -0.12 | -0.09 |
| Class D | -0.01 | 0.01 | -0.14 | -0.03 | 0.02 | 0.13 |
| Class E |  |  |  | -0.02 | -0.17 | 0.16 |
| Overall | -0.02 | -0.02 | -0.08 | -0.02 | 0.00 | 0.08 |
| EncTime[%] | 99% |  |  | 99% |  |  |
| DecTime[%] | 100% |  |  | 100% |  |  |

# Conclusion

In this contribution, a modified MVD coding method for CABAC is proposed. The proposed method can remove line buffer for MVD context derivation and reduce context size (14 to 10). An experimental result shows no performance drop in average BD-rate. It is suggested to consider the inclusion of this modification for complexity reduction in the HM/WD of HEVC.

- References

- T. Wiegand, et al., “WD3: Working Draft 3 of High-Efficiency Video Coding”, JCTVC-E603, 5th JCT-VC Meeting, Geneva, March 2011.

- V. Sze, et al, “Joint Algorithm-Architecture Optimization of CABAC”, JCTVC-E324, 5th JCT-VC Meeting, Geneva, March 2011.

- H. Sasai et al., “Modified Context Derivation for neighboring dependency reduction”, JCTVC-F429, 6th JCT-VC Meeting, Torino, July 2011.

- V. Sze, et al, “Parallel Context Processing of Coefficient Level”, JCTVC-F130, 6th JCT-VC Meeting, Torino, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, March, 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1:  Proposed specification for the WD.

### 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| ... |  |
| if( entropy_coding_mode_flag ){ |  |
| mvd_lc_prefix[ x0 ][ y0 ][ 0 ] | ae(v) |
| mvd_lc_prefix[ x0 ][ y0 ][ 1 ] | ae(v) |
| mvd_lc_suffix[ x0 ][ y0 ][ 0 ] | ae(v) |
| mvd_lc_suffix[ x0 ][ y0 ][ 1 ] | ae(v) |
| } |  |
| else{ |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| ... |  |
| if( entropy_coding_mode_flag ){ |  |
| mvd_l0_prefix[ x0 ][ y0 ][ 0 ] | ae(v) |
| mvd_l0_prefix[ x0 ][ y0 ][ 1 ] | ae(v) |
| mvd_l0_suffix[ x0 ][ y0 ][ 0 ] | ae(v) |
| mvd_l0_suffix[ x0 ][ y0 ][ 1 ] | ae(v) |
| else { |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| ... |  |
| if( entropy_coding_mode_flag ){ |  |
| mvd_l1_prefix[ x0 ][ y0 ][ 0 ] | ae(v) |
| mvd_l1_prefix[ x0 ][ y0 ][ 1 ] | ae(v) |
| mvd_l1_suffix[ x0 ][ y0 ][ 0 ] | ae(v) |
| mvd_l1_suffix[ x0 ][ y0 ][ 1 ] | ae(v) |
| } |  |
| else { |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) |
| } |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |

### 7.4.7 Prediction unit semantics

...

mvd_prefix_l0[ x0 ][ y0 ][ compIdx ], specifies the absolute value of less than 9 of the difference between a list 0 vector component to be used and its prediction. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

When any of the two components is not present, the inferred value is 0.

mvd_suffix_l0[ x0 ][ y0 ][compIdx], specifies the absolute value of difference minus 9 between a list 0 vector component to be used and its prediction. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

When any of the two components is not present, the inferred value is 0.

mvd_prefix_l1[ x0 ][ y0 ][ compIdx ] has the same semantics as mvd_prefix_l0, with l0 and list 0 replaced by l1 and list 1, respectively.

mvd_prefix_lc[ x0 ][ y0 ][ compIdx ] has the same semantics as mvd_prefix_l0, with l0 and list 0 replaced by lc and list combination, respectively.

mvd_suffix_l1[ x0 ][ y0 ][ compIdx ] has the same semantics as mvd_suffix_l0, with l0 and list 0 replaced by l1 and list 1, respectively.

mvd_suffix_lc[ x0 ][ y0 ][ compIdx ] has the same semantics as mvd_suffix_l0, with l0 and list 0 replaced by lc and list combination, respectively.

Table 9-4 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| prediction_unit() | ... | ... | ... | ... | ... |
|  | mvd__prefix_l0[ ][ ][ 0 ] | Table 9-5 |  | 0..4 | 10..14 |
|  | mvd_ prefix_lc[ ][ ][ 0 ], mvd_prefix_l1[ ][ ][ 0 ] | Table 9-5 |  |  | 10..14 |
|  | mvd_ prefix_l0[ ][ ][ 1 ] | Table 9-5 |  | 5..9 | 15..19 |
|  | mvd_ prefix_lc[ ][ ][ 1 ], mvd_prefix_l1[ ][ ][ 1 ] | Table 9-5 |  |  | 15..19 |
|  | ... | ... | ... | ... | ... |

Table 9-5 – Values of variable m and n for mvd_lc, mvd_l0, mvd_l1 ctxIdx

| Initialisation variables |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| m | 5 | 14 | 16 | 6 | 9 | 2 | 14 | 17 | 6 | 8 |
| n | 61 | 39 | 49 | 60 | 71 | 63 | 37 | 46 | 60 | 73 |
|  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| m | 1 | 11 | 14 | 4 | 7 | 1 | 11 | 15 | 5 | 8 |
| n | 67 | 49 | 56 | 69 | 77 | 65 | 48 | 52 | 67 | 76 |

| Table 9-6 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| ... | ... | ... | ... | ... | ... |
| mvd_prefix_l0[ ][ ][ 0 ] | P | prefix and suffix as given by UEG3 with signedValFlag=1, uCoff=9 | prefix: 4 | prefix: Table 9-5 | prefix: 0 |
| mvd_prefix_l0[ ][ ][ 1 ] | P |  | prefix:4 | prefix: Table 9-5 | prefix: 5 |
| mvd_prefix_l0[ ][ ][ 0 ], mvd_prefix_lc[ ][ ][ 0 ], mvd_prefix_l1[ ][ ][ 0 ] | B |  | prefix: 4 | prefix: Table 9-5 | prefix: 10 |
| mvd_ prefix_l0[ ][ ][ 1 ], mvd_prefix_lc[ ][ ][ 1 ], mvd_prefix_l1[ ][ ][ 1 ] | B |  | prefix: 4 | prefix: Table 9-5 | prefix: 15 |
| ... | ... | ... | ... | ... | ... |

| Table 9-7 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| ... | ... | ... | ... | ... | ... | ... | ... |
| mvd_prefix_l0[ ][ ][ 0 ] | Table 9-5 | 0 | 0 | 1 | 2 | 3 | 4 |
| mvd_prefix_l0[ ][ ][ 1 ] | Table 9-5 | 5 | 0 | 1 | 2 | 3 | 4 |
| mvd_prefix_l0[ ][ ][ 0 ], mvd_prefix_lc[ ][ ][ 0 ], mvd_prefix_l1[ ][ ][ 0 ] | Table 9-5 | 10 | 0 | 1 | 2 | 3 | 4 |
| mvd_ prefix_l0[ ][ ][ 1 ], mvd_prefix_lc[ ][ ][ 1 ], mvd_prefix_l1[ ][ ][ 1 ] | Table 9-5 | 15 | 0 | 1 | 2 | 3 | 4 |
| ... | ... | ... | ... | ... | ... | ... | ... |