| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July, 2011 | Document: JCTVC-F424 |
| --- | --- |

| Title: | CE9: Skip/Merge Simplification with Reduced Candidate Set (SP13) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

One of the major goals of CE9 is to evaluate possible modifications to the motion information derivation process during inter coding. In this test, the effect of reducing the number of possible Skip-Merge candidates is studied.

# Introduction

A Merge-Skip mode that uses the motion vector predictor derivation process of the merge mode [1] was adopted in place of the AMVP-Skip mode at the last meeting. In this test, the effect of reducing the set of possible Merge candidates for the Skip mode is studied.

# Approach

For this Test, the Merge-Skip candidate set is reduced to just the ‘Top’ and ‘Left’ Merge candidates when the Skip-Merge mode is signaled. If the Skip-Merge mode is not used and a 2Nx2N PU-Merge mode using the ‘Top’ or ‘Left’ Merge candidate is subsequently signaled, the QT-Root flag is inferred to be 1 and not decoded.

# Simulation Results

The proposed modifications were made in HM 3.0. Tests were conducted using the Random access and Low delay (for both the high-efficiency and low complexity) configurations, following the common testing conditions [2]. A relevant bug-fix reported in the CE9 document [3] is applied.

Experimental results are shown in Table 1 and 2.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.3 | 0.2 | 0.2 | 0.2 | 0.1 |
| Class B | 0.1 | 0.2 | 0.1 | -0.1 | -0.3 | -0.2 |
| Class C | 0.1 | 0.2 | 0.2 | 0.2 | 0.1 | 0.2 |
| Class D | 0.2 | 0.3 | 0.2 | 0.2 | 0.0 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.2 | 0.2 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 1. Results for experiment A.7 (Random access)

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.3 | 0.0 | 0.0 | -0.6 | -0.6 |
| Class C | 0.1 | 0.3 | 0.1 | 0.2 | -0.1 | -0.1 |
| Class D | 0.0 | 0.1 | 0.7 | 0.2 | -0.6 | -0.5 |
| Class E | 0.4 | 0.4 | 0.6 | 0.0 | -0.8 | 0.0 |
| All | 0.1 | 0.3 | 0.3 | 0.1 | -0.5 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

Table 2. Results for experiment A.7 (Low delay)

# Conclusions

The experiments in this contribution show that the candidate set of the Skip-Merge mode can be reduced without hurting coding performance (~0.1% degradation).

# References

[1] B. Bross et al., “BoG report of CE9: MV Coding and Skip/Merge operations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E481, 5th Meeting: Geneva, CH, Mar., 2011.

[2] F. Bossen, “Common conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D600, 4th Meeting: Daegu, KR, Jan., 2011.

[3]	Y.-W Huang, B. Bross, M. Zhou, W.-J. Chien, I. K. Kim, “Description of Core Experiment 9: MV Coding and Skip/Merge Operations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E709, 5th Meeting: Geneva, CH, Mar., 2011.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).