| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F427r1 |
| --- | --- |

| Title: | Consideration on Temporal Predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan | Tel: Email: | +81 3 5435 3308 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

HEVC employs temporal predictor as well as spatial predictors both for AMVP and Merge. As of the former motion information of surrounding PUs is stored in line buffer, but as of the latter all motion information in the reference frames should be stored in memory so increase of the size of the storage and bandwidth is concern. Especially for encoder manufacturers solution to develop low-cost encoders without the temporal predictor will be needed.

In TMuC there use to be “mv_competition_temporal_flag” in the slice header and by setting the value as 0, it is possible to develop encoder without the temporal predictor. However it is no longer contained in the latest HEVC specification.

The problem is that if all of the spatial predictors are unavailable and the temporal predictor is available in a PU, it cannot be avoided to apply the temporal predictor. This problem will happen both with AMVP and Merge cases.

To solve this problem we propose either of the following solutions:

- Insert “amvp_temporal_flag” (formerly known as “mv_competition_temporal_flag”) and “merge_temporal_flag” in a slice header

- If all of the spatial predictors unavailable, set (0,0) as a spatial predictor regardless of the availability of the temporal predictor

As shown in the rest of this document, solution (b) does not cause loss in coding efficiency.

# Introduction

HEVC employs temporal predictor as well as spatial predictors both for AMVP and Merge. As of the former motion information of surrounding PUs is stored in line buffer, but as of the latter all motion information in the reference frames should be stored in memory so increase of the size of the storage and bandwidth is concern.

In “Typedef.h” in HM-3.0 there are macros:

#define MRG_NEIGH_COL                1           ///< use of colocated MB in MERGE

#define AMVP_NEIGH_COL               1           ///< use of colocated MB in AMVP

and by setting them as 0, usage of the temporal predictor can be disabled.

In TMuC document [1] there use to be “mv_competition_temporal_flag” in the slice header and by setting the value as 0, it is possible to develop encoder without the temporal predictor. However this flag is not contained in the latest HEVC WD [2].

The problem is that if all of the spatial predictors are unavailable and the temporal predictor is available in a PU, it cannot be avoided to apply the temporal predictor. This problem will happen both with AMVP and Merge cases.

# Algorithm Description

To solve this problem we propose either of the following solutions:

- Insert “amvp_temporal_flag” (formerly known as “mv_competition_temporal_flag”) and “merge_temporal_flag” in a slice header

- If all of the spatial predictors unavailable, set (0,0) as a spatial predictor regardless the availability of the temporal predictor

# Simulation

## Simulation Condition

To show that solution (b) does not cause loss in coding efficiency the following experiment has been conducted.

Software provided among CE1 ML, which contains bug-fix on motion vector coding included in HM-3.0, is used for anchor with the following macro setting (HM-3.0):

Table 3.1.1

| MDSR | TMVP | Configuration |
| --- | --- | --- |
| HM3.0 (TL) | HM3.0 (H or C0) | #define LG_DISABLE_TMVP_H_AMVP 0 #define LG_DISABLE_TMVP_H_MRG 0 #define MTK_MV_COMPRESS_POS 0 #define MTK_TMVP_SELECT_POS 1 |

Both with anchor and to be tested temporal predictor is enabled both for AMVP and Merge. The proposed algorithm (Solution B) is implemented on this software and its performance is evaluated with BD-psnr/rate.

Simulation conditions are random-access, random-access-low-complexity, low-delay, low-delay-low-complexity as defined in [3].

To enable the proposed method macro in Typedef.h should be set as:

## Simulation Result

In Table 3.2.1 results with RandomAccess and RandomAccess_LC conditions are shown. Complete results are shown in JCTVC-F427.xls.

The result has been verified by Fujitsu [4].

Table 3.2.1

|  | Random access |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.00 | -0.04 | 0.00 | 0.01 | 0.00 | -0.13 |
| Class B | -0.06 | -0.13 | -0.04 | -0.03 | -0.08 | 0.01 |
| Class C | -0.02 | 0.00 | -0.01 | 0.00 | 0.02 | 0.10 |
| Class D | -0.08 | 0.06 | 0.02 | -0.08 | -0.13 | -0.03 |
| Class E |  |  |  |  |  |  |
| All | -0.04 | -0.03 | -0.01 | -0.02 | -0.05 | -0.01 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3.2.2

|  | Low delay |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.03 | 0.15 | -0.45 | -0.07 | -0.08 | 0.00 |
| Class C | -0.01 | -0.03 | -0.11 | -0.06 | 0.00 | -0.14 |
| Class D | -0.09 | -0.09 | 0.14 | -0.02 | 0.23 | -0.12 |
| Class E | -0.05 | 0.30 | 0.08 | -0.07 | -0.02 | 0.74 |
| All | -0.04 | 0.07 | -0.12 | -0.05 | 0.03 | 0.07 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusion

Current HEVC specification does not allow developing low-cost encoder without temporal predictor for motion vector coding. To allow manufacturers to develop low-cost encoder we propose either of the following solutions be adopted:

- Insert “amvp_temporal_flag” (formerly known as “mv_competition_temporal_flag”) and “merge_temporal_flag” in a slice header

- If all of the spatial predictors unavailable, set (0,0) as a spatial predictor regardless the availability of the temporal predictor

Simulation result shows that solution (b) does not cause loss in coding efficiency.

# References

[1] “Test Model under Consideration”, JCTVC-B205, July 2010

[2] “WD3: Working Draft 3 of High-Efficiency Video Coding”, JCTVC-E603, March 2011

[3] “Common test conditions and software reference configurations”, JCTVC-E700, March 2011

[4] ” Cross-verification result of JCTVC-F427 proposed by Sony”, JCTVC-F626, July 2011

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).