| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F429 |
| --- | --- |

| Title: | Modified Context Derivation for Complexity reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma, Osaka, Japan | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution presents a technique to reduce required memory for CABAC context selection process. In this contribution, combination of references to the neighboring blocks for the process was explored for several syntax elements, then a recommended combination is proposed. The proposed combination was also implemented in HMv3 and their coding efficiency was evaluated. Experimental result shows the modification brought 80% reduction of required line-buffer for context derivation process and 5% reduction of context size with less than 0.1% performance changes.

# Introduction

The current design of context selection uses the spatial correlation of neighboring data. A line buffer is required in parsing engine to store information. The size of line buffer depends on the width of source images, so it could not be overlooked with 4Kx2K sequences as discussed in [1]. In current HEVC WD3.0[2], to derive the context, neighboring data are used for the syntax elements alf_cu_flag, split_coding_unit_flag, skip_flag, merge_flag, intra_chroma_pred_mode, inter_pred_flag, ref_idx_lc, ref_idx_l0, ref_idx_l1, mvd_lc, mvd_l0, mvd_l1, no_residual_data_flag, cbf_luma, cbf_cb and cbf_cr.

We propose reducing the number of syntax element which uses neighboring data to reduce the overall line buffer size and memory size of the CABAC and also to enhance error robustness, especially for inter coding. The number of context size and required buffer size for those syntax elements is shown in Table 1. It is about 10% of total number of context size for inter coding in WD3.0.

Table 1. Context size and line-buffer requirement 
for syntax elements which is required to use neighboring data

|  | syntax elements | context size |  | line-buffer requirement |
| --- | --- | --- | --- | --- |
|  |  | I-Slice | inter(B) |  |
| slice_header() | alf_cu_flag | 3 | 3 | 1 |
| coding_tree() | split_coding_unit_flag | 3 | 3 | 1 |
| coding_unit() | skip_flag | na | 3 | 1 |
| Prediction_unit() | intra_chroma_pred_flag | 3 | 3 | 1 |
|  | merge_flag | na | 3 | 1 |
|  | inter_pred_flag | na | 3 | 1 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | na | 4 | 2 |
|  | mvd_lx[ ][ ][ 0 ] | na | 3 | 1 |
|  | mvd_lx[ ][ ][ 1 ] | na | 3 | 1 |
| transform_tree() | no_residual_data_flag | na | 4 | 1 |
|  | cbf_luma | 4 | 4 | 1 |
|  | cbf_cb | 4 | 4 | 1 |
|  | cbf_cr | 4 | 4 | 1 |
| Total |  | 21 | 44 | 14 |

# Technical description

In order to reduce memory size, prohibiting reference to the above blocks is effective since such a reference requires line buffer. For MVD, a simple version is reported here, and an enhanced version is presented in another contribution JCTVC-F423[3]. The two parameters “ref_idx_lx” and “inter_pred_flag” are presented in a motion vector parsing robustness contribution JCTVC-F470[4].

Basic concept of the modification here has two approaches:

Part 1: Not to refer above blocks for context derivation.

Part 2: Not to refer any neighboring blocks for context derivation.

In case of part 1, Table 2 shows modified context derivation process. For part 2, no context selection is used. Therefore, it is not necessary to selecting context index for a syntax element.  Please refer to the Annex for the proposed WD text.

Table 2. modified context derivation for part 1.

| Syntax element | condL | ctxIdxInc |
| --- | --- | --- |
| alf_cu_flag | alf_cu_flag[ xL ][ yL ] | ( condL && availableL ) |
| split_coding_unit_flag | cuDepth[ xL ][ yL ] > cuDepth[ xP ][ yP ] | ( condL && availableL ) |
| skip_flag | skip_flag[ xL ][ yL ] | ( condL && availableL ) |
| merge_flag | merge_flag[ xL ][ yL ] | ( condL && availableL ) |
| intra_chroma_pred_mode | IntraPredMode[ xL ][ yL ] = = 4 | ( condL && availableL ) |
| inter_pred_flag | inter_pred_flag[ xL ][ yL ] | ( condL && availableL ) |
| ref_idx_lc | ref_idx_lc[ xL ][ yL ] > 0 | ( condL && availableL ) |
| ref_idx_l0 | ref_idx_l0[ xL ][ yL ] > 0 | ( condL && availableL ) |
| ref_idx_l1 | ref_idx_l1[ xL ][ yL ] > 0 | ( condL && availableL ) |
| mvd_lc | mvd_lc[ xL ][ yL ] > 16 | ( condL && availableL ) |
| mvd_l0 | mvd_l0[ xL ][ yL ] > 16 | ( condL && availableL ) |
| mvd_l1 | mvd_l1[ xL ][ yL ] > 16 | ( condL && availableL ) |
| no_residual_data_flag | no_residual_data_flag[ xL ][ yL ] | ( condL && availableL ) |
| cbf_luma | cbf_luma[ xL ][ yL ] | ( condL && availableL ) |
| cbf_cb | cbf_cb[ xL ][ yL ] | ( condL && availableL ) |
| cbf_cr | cbf_cr[ xL ][ yL ] | ( condL && availableL ) |

# Simulation Results

This method was implemented in HM3.0. All parameters with part 1 and part 2 were tested individually on common condition E700[5]. To identify performance clearly, we do NOT change initial context parameters for this experiment.

## Simulation Results per single parameter

### alf_cu_flag

No test results. It should not be changed with the modification due to only one context is selected in HMv3 software.

### split_coding_unit_flag

Table 3. BD BR for referring only left block (split_coding_unit_flag)

|  | Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.03 | -0.01 | 0.04 | 0.03 | -0.07 | -0.61 |  |  |  |
| Class B | 0.04 | 0.06 | 0.04 | 0.13 | 0.12 | 0.13 | 0.19 | 0.35 | 0.22 |
| Class C | 0.03 | 0.01 | 0.04 | 0.18 | 0.26 | 0.19 | 0.16 | -0.05 | 0.06 |
| Class D | 0.04 | 0.00 | 0.04 | 0.14 | 0.18 | -0.05 | 0.07 | 0.15 | 0.70 |
| Class E | 0.10 | 0.05 | -0.01 |  |  |  | 0.54 | 1.55 | 0.52 |
| Overall | 0.05 | 0.02 | 0.03 | 0.12 | 0.12 | -0.07 | 0.22 | 0.43 | 0.36 |
| EncTime[%] | 100% |  |  | 99% |  |  | 99% |  |  |
| DecTime[%] | 104% |  |  | 101% |  |  | 102% |  |  |

Table 4. BD BR for no reference to neighboring blocks (split_coding_unit_flag)

|  | Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.08 | -0.03 | 0.02 | 0.18 | 0.10 | -0.13 |  |  |  |
| Class B | 0.11 | 0.07 | 0.04 | 0.28 | 0.23 | 0.30 | 0.37 | 0.49 | 0.55 |
| Class C | 0.06 | 0.06 | 0.10 | 0.24 | 0.41 | 0.21 | 0.29 | 0.31 | 0.34 |
| Class D | 0.06 | -0.02 | 0.01 | 0.21 | 0.14 | 0.05 | 0.22 | 0.65 | 0.63 |
| Class E | 0.19 | 0.04 | 0.07 |  |  |  | 0.65 | 0.65 | 0.06 |
| Overall | 0.09 | 0.03 | 0.05 | 0.23 | 0.22 | 0.12 | 0.37 | 0.51 | 0.42 |
| EncTime[%] | 100% |  |  | 99% |  |  | 99% |  |  |
| DecTime[%] | 103% |  |  | 100% |  |  | 101% |  |  |

### skip_flag

Table 5. BD BR for referring only left block (skip_flag)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.01 | -0.09 | 0.04 |  |  |  |
| Class B | 0.09 | 0.03 | 0.11 | 0.09 | 0.24 | 0.14 |
| Class C | 0.08 | 0.18 | 0.13 | 0.10 | 0.12 | 0.23 |
| Class D | 0.08 | -0.01 | 0.06 | 0.08 | 0.44 | 0.25 |
| Class E |  |  |  | 0.14 | 0.07 | 0.28 |
| Overall | 0.06 | 0.03 | 0.09 | 0.10 | 0.23 | 0.22 |
| EncTime[%] | 100% |  |  | 100% |  |  |
| DecTime[%] | 101% |  |  | 101% |  |  |

Table 6. BD BR for no reference to neighboring blocks (skip_flag)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.11 | 0.05 | 0.05 |  |  |  |
| Class B | 0.17 | 0.08 | 0.16 | 0.18 | 0.47 | 0.14 |
| Class C | 0.14 | 0.17 | 0.13 | 0.11 | 0.13 | 0.02 |
| Class D | 0.12 | 0.07 | 0.00 | 0.07 | 0.07 | 0.17 |
| Class E |  |  |  | 0.12 | -0.01 | -0.05 |
| Overall | 0.14 | 0.09 | 0.09 | 0.12 | 0.19 | 0.08 |
| EncTime[%] | 100% |  |  | 100% |  |  |
| DecTime[%] | 101% |  |  | 101% |  |  |

### merge_flag

Table 7. BD BR for referring only left block (merge_flag)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.08 | -0.15 | -0.02 |  |  |  |
| Class B | -0.02 | -0.08 | 0.07 | -0.02 | 0.14 | 0.27 |
| Class C | -0.01 | 0.11 | 0.07 | -0.03 | 0.04 | 0.15 |
| Class D | -0.01 | -0.09 | -0.10 | -0.02 | 0.05 | 0.26 |
| Class E |  |  |  | 0.04 | -0.46 | -0.20 |
| Overall | -0.03 | -0.05 | 0.01 | -0.01 | -0.02 | 0.15 |
| EncTime[%] | 100% |  |  | 100% |  |  |
| DecTime[%] | 102% |  |  | 101% |  |  |

Table 8. BD BR for no reference to neighboring blocks (merge_flag)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.05 | -0.14 | 0.09 |  |  |  |
| Class B | -0.01 | 0.01 | 0.03 | 0.00 | 0.33 | 0.37 |
| Class C | -0.01 | 0.09 | 0.02 | -0.03 | 0.07 | -0.08 |
| Class D | -0.01 | 0.00 | -0.13 | -0.02 | -0.12 | 0.14 |
| Class E |  |  |  | 0.02 | 0.34 | 0.07 |
| Overall | -0.02 | -0.01 | 0.01 | -0.01 | 0.16 | 0.14 |
| EncTime[%] | 100% |  |  | 100% |  |  |
| DecTime[%] | 102% |  |  | 101% |  |  |

### intra_chroma_pred_mode

Table 9. BD BR for referring only left block (intra_chroma_pred_flag)

|  | Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.01 | -0.09 | -0.04 | -0.08 | 0.13 | -0.44 |  |  |  |
| Class B | 0.00 | 0.09 | 0.04 | 0.00 | -0.02 | 0.03 | -0.01 | 0.29 | 0.03 |
| Class C | 0.00 | 0.01 | 0.02 | -0.02 | 0.24 | 0.09 | -0.01 | 0.03 | 0.10 |
| Class D | 0.01 | -0.02 | -0.06 | -0.01 | -0.10 | -0.16 | -0.02 | -0.47 | 0.24 |
| Class E | -0.01 | 0.00 | 0.09 |  |  |  | -0.17 | 1.07 | 0.14 |
| Overall | 0.01 | 0.00 | 0.01 | -0.03 | 0.06 | -0.11 | -0.04 | 0.18 | 0.12 |
| EncTime[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| DecTime[%] | 97% |  |  | 99% |  |  | 99% |  |  |

Table 10. BD BR for no reference to neighboring blocks (intra_chroma_pred_flag)

|  | Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.03 | 0.06 | 0.05 | 0.02 | 0.46 | -0.05 |  |  |  |
| Class B | 0.02 | 0.03 | 0.05 | -0.03 | 0.14 | 0.18 | -0.04 | 0.27 | -0.09 |
| Class C | 0.01 | 0.06 | 0.07 | -0.02 | 0.26 | 0.04 | -0.02 | 0.06 | 0.08 |
| Class D | 0.00 | 0.08 | 0.00 | -0.05 | -0.03 | -0.20 | -0.01 | 0.22 | -0.03 |
| Class E | -0.02 | 0.09 | 0.13 |  |  |  | 0.05 | -0.08 | -0.96 |
| Overall | 0.01 | 0.06 | 0.05 | -0.02 | 0.20 | 0.00 | -0.01 | 0.14 | -0.20 |
| EncTime[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| DecTime[%] | 97% |  |  | 99% |  |  | 98% |  |  |

### inter_pred_flag

Table 11. BD BR for referring only left block (inter_pred _flag)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.04 | -0.15 | -0.05 |  |  |  |
| Class B | 0.00 | -0.06 | 0.04 | -0.01 | 0.24 | 0.15 |
| Class C | -0.02 | 0.01 | 0.00 | -0.03 | -0.06 | -0.21 |
| Class D | -0.01 | -0.05 | -0.08 | -0.07 | -0.03 | -0.01 |
| Class E |  |  |  | 0.06 | -0.14 | -0.29 |
| Overall | -0.02 | -0.06 | -0.02 | -0.02 | 0.02 | -0.06 |
| EncTime[%] | 100% |  |  | 100% |  |  |
| DecTime[%] | 100% |  |  | 100% |  |  |

Table 12. BD BR for no reference to neighboring blocks (inter_pred_flag)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00 | -0.01 | -0.12 |  |  |  |
| Class B | 0.00 | -0.02 | -0.03 | -0.02 | 0.38 | -0.02 |
| Class C | 0.00 | 0.02 | 0.06 | 0.00 | 0.03 | 0.02 |
| Class D | 0.00 | -0.11 | -0.05 | -0.05 | -0.20 | -0.42 |
| Class E |  |  |  | 0.03 | -0.34 | -0.43 |
| Overall | 0.00 | -0.03 | -0.03 | -0.01 | 0.01 | -0.19 |
| EncTime[%] | 99% |  |  | 100% |  |  |
| DecTime[%] | 100% |  |  | 100% |  |  |

### ref_idx_lc, ref_idx_l0 and ref_idx_l1

Table 13. BD BR for referring only left block (ref_idx_lx)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.06 | -0.20 | -0.22 |  |  |  |
| Class B | 0.00 | -0.04 | -0.04 | -0.03 | 0.07 | 0.26 |
| Class C | 0.00 | 0.01 | 0.08 | -0.02 | -0.02 | 0.10 |
| Class D | 0.01 | -0.07 | -0.08 | 0.00 | -0.07 | 0.38 |
| Class E |  |  |  | 0.04 | 0.01 | -0.51 |
| Overall | -0.01 | -0.07 | -0.06 | -0.01 | 0.00 | 0.10 |
| EncTime[%] | 100% |  |  | 100% |  |  |
| DecTime[%] | 100% |  |  | 100% |  |  |

Table 14. BD BR for no reference to neighboring blocks (ref_idx_lx)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.08 | -0.30 | -0.16 |  |  |  |
| Class B | -0.02 | -0.06 | -0.01 | -0.04 | 0.30 | -0.04 |
| Class C | -0.02 | 0.10 | 0.06 | -0.03 | 0.01 | -0.09 |
| Class D | -0.02 | -0.07 | -0.16 | -0.02 | 0.17 | 0.25 |
| Class E |  |  |  | -0.02 | 0.34 | -0.35 |
| Overall | -0.04 | -0.08 | -0.06 | -0.03 | 0.20 | -0.04 |
| EncTime[%] | 99% |  |  | 99% |  |  |
| DecTime[%] | 100% |  |  | 100% |  |  |

### mvd_lc, mvd_l0 and mvd_l1

Table 15. BD BR for referring only left block (mvd_lx)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.02 | -0.06 | 0.27 |  |  |  |
| Class B | -0.01 | -0.03 | -0.02 | -0.03 | 0.14 | 0.08 |
| Class C | 0.00 | 0.07 | 0.02 | -0.05 | -0.09 | -0.03 |
| Class D | 0.03 | 0.02 | -0.08 | -0.04 | -0.16 | 0.08 |
| Class E |  |  |  | 0.04 | -0.20 | -0.22 |
| Overall | 0.01 | 0.00 | 0.05 | -0.02 | -0.05 | 0.00 |
| EncTime[%] | 100% |  |  | 100% |  |  |
| DecTime[%] | 97% |  |  | 97% |  |  |

Table 16. BD BR for no reference to neighboring blocks (mvd_lx)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.06 | -0.15 | -0.14 |  |  |  |
| Class B | 0.00 | -0.03 | -0.02 | -0.01 | 0.17 | 0.13 |
| Class C | 0.00 | 0.11 | -0.01 | -0.02 | -0.12 | -0.09 |
| Class D | -0.01 | 0.01 | -0.14 | -0.03 | 0.02 | 0.13 |
| Class E |  |  |  | -0.02 | -0.17 | 0.16 |
| Overall | -0.02 | -0.02 | -0.08 | -0.02 | 0.00 | 0.08 |
| EncTime[%] | 100% |  |  | 100% |  |  |
| DecTime[%] | 102% |  |  | 102% |  |  |

### no_residual_data_flag

Table 17. BD BR for referring only left block (no_residual_data_flag)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.04 | -0.11 | -0.08 |  |  |  |
| Class B | 0.00 | -0.11 | -0.01 | 0.01 | 0.15 | -0.19 |
| Class C | 0.01 | 0.01 | -0.01 | 0.01 | -0.15 | -0.12 |
| Class D | 0.02 | -0.09 | -0.10 | 0.02 | -0.03 | 0.03 |
| Class E |  |  |  | 0.10 | 0.48 | -0.06 |
| Overall | 0.00 | -0.08 | -0.05 | 0.03 | 0.09 | -0.09 |
| EncTime[%] | 100% |  |  | 100% |  |  |
| DecTime[%] | 103% |  |  | 103% |  |  |

Table 18. BD BR for no reference to neighboring blocks (no_residual_data_flag)

|  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00 | -0.16 | -0.10 |  |  |  |
| Class B | 0.03 | 0.01 | 0.00 | 0.01 | 0.19 | 0.13 |
| Class C | 0.04 | 0.12 | 0.02 | 0.02 | -0.01 | -0.06 |
| Class D | 0.04 | 0.01 | -0.03 | 0.02 | -0.24 | -0.04 |
| Class E |  |  |  | 0.10 | -0.14 | 0.20 |
| Overall | 0.03 | -0.01 | -0.03 | 0.03 | -0.03 | 0.05 |
| EncTime[%] | 100% |  |  | 100% |  |  |
| DecTime[%] | 103% |  |  | 103% |  |  |

### cbf_luma, cbf_cb and cbf_cr

Table 19. BD BR for referring only left block (cbf_luma, cbf_cb and cbf_cr)

|  | Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.01 | 0.05 | 0.13 | -0.05 | -0.13 | -0.11 |  |  |  |
| Class B | 0.03 | 0.08 | 0.11 | -0.02 | 0.03 | 0.27 | -0.01 | 0.40 | 0.20 |
| Class C | 0.02 | 0.11 | 0.14 | -0.01 | 0.32 | 0.09 | -0.06 | 0.13 | -0.03 |
| Class D | 0.01 | 0.20 | 0.12 | 0.05 | 0.11 | -0.53 | 0.02 | 0.38 | 0.48 |
| Class E | 0.01 | 0.22 | 0.19 |  |  |  | 0.05 | 0.31 | -0.76 |
| Overall | 0.02 | 0.12 | 0.13 | -0.01 | 0.08 | -0.05 | 0.00 | 0.31 | 0.16 |
| EncTime[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| DecTime[%] | 101% |  |  | 103% |  |  | 103% |  |  |

Table 20. BD BR for no reference to neighboring blocks (cbf_luma, cbf_cb and cbf_cr)

|  | Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.04 | 0.07 | 0.20 | -0.05 | -0.26 | 0.27 |  |  |  |
| Class B | 0.06 | 0.12 | 0.18 | 0.02 | 0.30 | 0.25 | 0.00 | 0.43 | 0.33 |
| Class C | 0.03 | 0.15 | 0.21 | -0.01 | 0.28 | 0.15 | -0.08 | 0.37 | 0.31 |
| Class D | 0.02 | 0.30 | 0.31 | 0.02 | 0.12 | 0.12 | -0.06 | 0.80 | 0.27 |
| Class E | 0.08 | 0.25 | 0.29 |  |  |  | 0.10 | 0.16 | 0.16 |
| Overall | 0.05 | 0.17 | 0.23 | 0.00 | 0.12 | 0.20 | -0.02 | 0.46 | 0.28 |
| EncTime[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| DecTime[%] | 100% |  |  | 103% |  |  | 103% |  |  |

## Summary of Results

Figure 1 shows the summary of all test results. No significant drop was observed except for split_coding_unit_flag and skip flag.

Figure 1. BD BR for luma for all parameter simulations.

## Simulation Results for Recommended Combinations

It is shown that performance drop is obvious for "split_coding_unit_flag" and "skip_flag" by disabling neighboring data dependencies. Based on this observation, following two combinations were tested in HMv3.

Table 21 shows the 1st combination, in which all of the neighboring data dependency was cut off except for split_coding_unit_flag and skip_flag. Table 22 shows the 2nd combination, in which the dependency on the left block was kept except for split_coding_unit_flag and skip_flag. It is not observed the significant difference between the 1st combination and the 2nd combination.

Table 21. 1st combination for neighboring data dependency

| syntax elements | dependency |  | context size |  | line-buffer requirement |
| --- | --- | --- | --- | --- | --- |
|  | Above | Left | I-Slice | inter(B) |  |
| alf_cu_flag | x | x | 1 | 1 | 0 |
| split_coding_unit_flag | o | o | 3 | 3 | 1 |
| skip_flag | o | o | na | 3 | 1 |
| intra_chroma_pred_flag | x | x | 1 | 1 | 0 |
| merge_flag | x | x | na | 1 | 0 |
| inter_pred_flag | x | x | na | 1 | 0 |
| ref_idx_lc, ref_idx_l0, ref_idx_l1 | x | x | na | 1 | 0 |
| mvd_lx[ ][ ][ 0 ] | x | x | na | 1 | 0 |
| mvd_l0[ ][ ][ 1 ] | x | x | na | 1 | 0 |
| no_residual_data_flag | x | x | na | 1 | 0 |
| cbf_luma | x | x | 1 | 1 | 0 |
| cbf_cb | x | x | 1 | 1 | 0 |
| cbf_cr | x | x | 1 | 1 | 0 |
| Total |  |  | 8 | 17 | 2 |

Table 22. 2nd combination for neighboring data dependency

| syntax elements | dependency |  | context size |  | line-buffer requirement |
| --- | --- | --- | --- | --- | --- |
|  | Above | Left | I-Slice | inter(B) |  |
| alf_cu_flag | x | o | 1 | 1 | 0 |
| split_coding_unit_flag | o | o | 3 | 3 | 1 |
| skip_flag | o | o | na | 3 | 1 |
| intra_chroma_pred_flag | x | o | 2 | 2 | 0 |
| merge_flag | x | o | na | 2 | 0 |
| inter_pred_flag | x | o | na | 2 | 0 |
| ref_idx_lc, ref_idx_l0, ref_idx_l1 | x | o | na | 2 | 0 |
| mvd_lx[ ][ ][ 0 ] | x | o | na | 2 | 0 |
| mvd_l0[ ][ ][ 1 ] | x | o | na | 2 | 0 |
| no_residual_data_flag | x | o | na | 2 | 0 |
| cbf_luma | x | o | 2 | 2 | 0 |
| cbf_cb | x | o | 2 | 2 | 0 |
| cbf_cr | x | o | 2 | 2 | 0 |
| Total |  |  | 12 | 27 | 2 |

### 1st combination

Table 23. BD BR for 1st combination (high efficiency)

|  | Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.06 | 0.18 | 0.22 | -0.01 | 0.36 | 0.69 |  |  |  |
| Class B | 0.08 | 0.14 | 0.21 | 0.05 | 0.29 | 0.33 | 0.01 | 1.05 | 0.36 |
| Class C | 0.04 | 0.18 | 0.25 | -0.02 | 0.34 | 0.17 | -0.11 | 0.49 | 0.37 |
| Class D | 0.02 | 0.35 | 0.34 | 0.00 | -0.03 | 0.02 | -0.05 | 0.56 | 0.32 |
| Class E | 0.06 | 0.40 | 0.39 |  |  |  | 0.06 | 2.11 | -0.59 |
| Overall | 0.05 | 0.24 | 0.27 | 0.01 | 0.24 | 0.30 | -0.03 | 0.99 | 0.18 |
| EncTime[%] | 100% |  |  | 99% |  |  | 100% |  |  |
| DecTime[%] | 97% |  |  | 97% |  |  | 96% |  |  |

### 2nd combination

Table 24. BD BR for 2nd combination (high efficiency)

|  | Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.03 | 0.01 | 0.11 | -0.04 | 0.09 | -0.02 |  |  |  |
| Class B | 0.04 | 0.13 | 0.09 | 0.01 | 0.18 | 0.15 | 0.00 | 0.53 | 0.39 |
| Class C | 0.02 | 0.09 | 0.15 | 0.02 | 0.37 | 0.11 | -0.05 | 0.27 | 0.38 |
| Class D | 0.02 | 0.20 | 0.19 | 0.01 | 0.03 | -0.07 | -0.04 | 0.70 | 0.59 |
| Class E | 0.02 | 0.24 | 0.23 |  |  |  | 0.04 | 1.45 | -0.09 |
| Overall | 0.03 | 0.13 | 0.15 | 0.00 | 0.17 | 0.05 | -0.01 | 0.68 | 0.35 |
| EncTime[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| DecTime[%] | 97% |  |  | 97% |  |  | 97% |  |  |

# Conclusion

In this contribution, a modified context derivation method is proposed. The proposed method can reduce line buffer (80%) and context size(5%) by 1st combination in which all of the neighboring data dependency was cut off except for split_coding_unit_flag and skip_flag. An experimental result shows no significant performance drop in average BD-rate. It is suggested to consider the inclusion of this modification in the HM/WD of HEVC.

- References

- V. Sze, et al., “Joint Algorithm-Architecture Optimization of CABAC”, JCTVC-E324, 5th JCT-VC Meeting, Geneva, March 2011.

- T. Wiegand, et al., “WD3: Working Draft 3 of High-Efficiency Video Coding”, JCTVC-E603, 5th JCT-VC Meeting, Geneva, March 2011.

- H. Sasai, et al., “Modified MVD coding for CABAC”, JCTVC-F423, 6th JCT-VC Meeting, Torino, July 2011.

- T. Sugio, et al., “Parsing Robustness for Merge/AMVP”, JCTVC-F470, 6th JCT-VC Meeting, Torino, July, 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700 , 5th JCT-VC Meeting, Geneva, March, 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1:  Text for the WD.

## CABAC parsing process for slice data

Table 9-25 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Table 9-26 | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0...2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_type | Error: Reference source not found | 0 | 1..4 | 5..9 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | rem_intra_luma_pred_mode | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0 | 1 | 2 |
|  | merge_flag | Table 9-27 |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Table 9-28 |  | 0..1 | 2..3 |
|  | mvd_l0[ ][ ][ 0 ] | Table 9-29 |  | 0..4 | 10..14 |
|  | mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | Table 9-29 |  |  | 10..14 |
|  | mvd_l0[ ][ ][ 1 ] | Table 9-29 |  | 5..9 | 15..19 |
|  | mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | Table 9-29 |  |  | 15..19 |
|  | mvp_idx_lc, mvp_idx_l0, mvp_idx_l1 |  |  | 0..1 | 2..3 |
| transform_tree() | no_residual_data_flag | Table 9-30 |  | 0 | 1 |
|  | split_transform_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cbf_luma | Table 9-31 | 0 | 1 | 2 |
|  | cbf_cb | Table 9-32 | 0 | 1 | 2 |
|  | cbf_cr | Table 9-33 | 0 | 1 | 2 |

Table 9-26 – Values of variables m and n for alf_cu_flag ctxIdx

| Initialisation variables | alf_cu_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| m | 0 | 0 | 0 |
| n | 64 | 64 | 64 |

Table 9-25 – Values of variable m and n for intra_chroma_pred_mode ctxIdx

| Initialisation variables | intra_chroma_pred_mode ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| m | 0 | 0 | 0 |
| n | 64 | 64 | 64 |

Table 9-27 – Values of variable m and n for merge_flag ctxIdx

| Initialisation variables | merge_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| m | 0 | 0 |
| n | 64 | 64 |

Table 9-28 – Values of variable m and n for ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx

| Initialisation variables | ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| m | -6 | 1 | -9 | -18 |
| n | 59 | 59 | 55 | 55 |

Table 9-29 – Values of variable m and n for mvd_lc, mvd_l0, mvd_l1 ctxIdx

| Initialisation variables |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| m | 5 | 14 | 16 | 6 | 9 | 2 | 14 | 17 | 6 | 8 |
| n | 61 | 39 | 49 | 60 | 71 | 63 | 37 | 46 | 60 | 73 |
|  | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| m | 1 | 11 | 14 | 4 | 7 | 1 | 11 | 15 | 5 | 8 |
| n | 67 | 49 | 56 | 69 | 77 | 65 | 48 | 52 | 67 | 76 |

Table 9-30 – Values of variable m and n for no_residual_data_flag ctxIdx

| Initialisation variables | no_residual_data_flag ctxIdx |  |
| --- | --- | --- |
|  | 0 | 1 |
| m | -14 | -26 |
| n | 72 | 88 |

Table 9-31 – Values of variable m and n for cbf_luma ctxIdx

| Initialisation variables | cbf_luma ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| m | -22 |  |  |  | -18 |  |  |  | -11 |  |  |  |
| n | 116 |  |  |  | 98 |  |  |  | 80 |  |  |  |

Table 9-32 – Values of variable m and n for cbf_cb ctxIdx

| Initialisation variables | cbf_cb ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| m | -35 |  |  |  | -46 |  |  |  | -22 |  |  |  |
| n | 116 |  |  |  | 114 |  |  |  | 52 |  |  |  |

Table 9-33 – Values of variable m and n for cbf_cr ctxIdx

| Initialisation variables | cbf_cr ctxIdx |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  |
| m | -29 |  |  |  | -43 |  |  |  | -19 |  |  |  |
| n | 104 |  |  |  | 107 |  |  |  | 45 |  |  |  |

| Table 9-34 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| alf_cu_flag | I | FL, cMax = 1 | 0 | Table 9-26 | 0 |
|  | P |  | 0 | Table 9-26 | 1 |
|  | B |  | 0 | Table 9-26 | 2 |
| intra_chroma_pred_mode ( IntraPredMode < 4 ) | I | TU, cMax = 3 | 3 | Error: Reference source not found | 0 |
|  | P |  | 3 | Error: Reference source not found | 1 |
|  | B |  | 3 | Error: Reference source not found | 2 |
| intra_chroma_pred_mode ( 4 <= IntraPredMode < 34 ) | I | TU, cMax = 4 | 4 | Error: Reference source not found | 0 |
|  | P |  | 4 | Error: Reference source not found | 1 |
|  | B |  | 4 | Error: Reference source not found | 2 |
| merge_flag | P | FL, cMax = 1 | 0 | Table 9-27 | 0 |
|  | B |  | 0 | Table 9-27 | 1 |
| inter_pred_flag | B | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
| ref_idx_lc | B | TU, cMax = num_ref_idx_lC_active_minus1 | 1 | Table 9-28 | 2 |
| ref_idx_l0 | P | TU, cMax = num_ref_idx_l0_active_minus1 | 1 | Table 9-28 | 0 |
|  | B |  | 1 | Table 9-28 | 2 |
| ref_idx_l1 | B | TU, cMax = num_ref_idx_l1_active_minus1 | 1 | Table 9-28 | 2 |
| mvd_l0[ ][ ][ 0 ] | P | prefix and suffix as given by UEG3 with signedValFlag=1, uCoff=9 | prefix: 4 suffix: na | prefix: Table 9-29 suffix: na | prefix: 0 suffix: na, (uses Decode Bypass) |
| mvd_l0[ ][ ][ 1 ] | P |  | prefix:4 suffix: na | prefix: Table 9-29 suffix: na | prefix: 5 suffix: na, (uses Decode Bypass) |
| mvd_l0[ ][ ][ 0 ], mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | B |  | prefix: 4 suffix: na | prefix: Table 9-29 suffix: na | prefix: 10 suffix: na, (uses Decode Bypass) |
| mvd_l0[ ][ ][ 1 ], mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | B |  | prefix: 4 suffix: na | prefix: Table 9-29 suffix: na | prefix: 15 suffix: na, (uses Decode Bypass) |
| no_residual_data_flag | P | FL, cMax = 1 | 0 | Table 9-30 | 0 |
|  | B |  | 0 | Table 9-30 | 1 |
| cbf_luma | I | FL, cMax = 1 | 0 | Table 9-31 | 0 |
|  | P |  | 0 | Table 9-31 | 1 |
|  | B |  | 0 | Table 9-31 | 2 |
| cbf_cb | I | FL, cMax = 1 | 0 | Table 9-32 | 0 |
|  | P |  | 0 | Table 9-32 | 1 |
|  | B |  | 0 | Table 9-32 | 2 |
| cbf_cr | I | FL, cMax = 1 | 0 | Table 9-33 | 0 |
|  | P |  | 0 | Table 9-33 | 1 |
|  | B |  | 0 | Table 9-33 | 2 |

| Table 9-35 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| alf_cu_flag | Table 9-26 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| intra_chroma_pred_mode | Error: Reference source not found | 0 | 0 | 1 | 1 | 1 | na |
|  |  | 1 | 0 | 1 | 1 | 1 | na |
|  |  | 2 | 0 | 1 | 1 | 1 | na |
| merge_flag | Table 9-27 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
| inter_pred_flag ref_idx_l0 | Error: Reference source not found Table 9-28 | 0 | 0 | na | na | na | na |
|  |  | 0 | 0 | 1 | 1 | 1 | 1 |
| ref_idx_l0, ref_idx_l1, ref_idx_lc | Table 9-28 | 2 | 0 | 1 | 1 | 1 | 1 |
| mvd_l0[ ][ ][ 0 ] | Table 9-29 | 0 | 0 | 1 | 2 | 3 | 4 |
| mvd_l0[ ][ ][ 1 ] | Table 9-29 | 5 | 0 | 1 | 2 | 3 | 4 |
| mvd_l0[ ][ ][ 0 ], mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | Table 9-29 | 10 | 0 | 1 | 2 | 3 | 4 |
| mvd_l0[ ][ ][ 1 ], mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | Table 9-29 | 15 | 0 | 1 | 2 | 3 | 4 |
| no_residual_data_flag | Table 9-30 | 0 | 0 | na | na | na | na |
| cbf_luma | Table 9-31 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| cbf_luma cbf_cb ( PredMode = = MODE_INTRA ) | Table 9-31 Table 9-32 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| cbf_cb ( PredMode = = MODE_INTRA ) cbf_cb ( PredMode != MODE_INTRA ) | Table 9-32 Table 9-32 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| cbf_cb ( PredMode != MODE_INTRA ) cbf_cr ( PredMode = = MODE_INTRA ) | Table 9-32 Table 9-33 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |
| cbf_cr ( PredMode = = MODE_INTRA ) cbf_cr ( PredMode != MODE_INTRA ) | Table 9-33 Table 9-33 | 0 | 0 | na | na | na | na |
|  |  | 1 | 0 | na | na | na | na |
|  |  | 2 | 0 | na | na | na | na |