| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F431r2 |
| --- | --- |

| Title: | Complexity Reduction of Chroma Intra Prediction by Reconstructed Luma Samples |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan | Tel: Email: | +81 3 5435 3308 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

At the 5th JCTVC Mtg chroma intra prediction by reconstructed luma samples has been adopted. It contributes to gain in chroma by as much as much as -7.8% for U and -6.1% for V in Intra and Intra_Loco cases. However there are two concerns on implementation of this tool: calculation of α&β and storage for reconstructed luma pixels.

In this document complexity reduction of chroma intra prediction by reconstructed luma samples on these 2 aspects is studied.

# Introduction

In certain texture areas, chroma pixels are high correlated to luma pixels, and chroma coding efficiency can be improved by making use of this correlation. At the 5th JCTVC Mtg JCTVC-E266 [2] proposed chroma intra prediction with reconstructed luma samples.

However there are two concerns on implementation of this tool: calculation of α&β and storage for reconstructed luma pixels.

# Algorithm Description

## Reduction of α&β Calculation

[Reduction A]

Calculation of correlation factor α&β is a concern as PU size gets larger. In this proposal sub-sampled neighboring pixels are used for α&β calculation as shown in Fig 2.1.1 (Ratio=2:1). Only neighboring pixels filled with black are used for α&β calculation and white neighboring pixels are not used in this case.

The value of N can be 4, 8 or 16 and the following ratio is applied for each of the PU sizes:

N=4: 	Ratio=1:1

N=8: 	Ratio=2:1

N=16:	Ratio=4:1

In this way size of the input for α&β calculation remains same regardless of the PU size.

Fig 2.1.1

[Reduction B]

Not 10-bit but 8-bit samples are used both with Luma and Chroma for α&β calculation.

## Reduction of Storage for Reconstructed Luma Pixels

[Reduction C]

Storing RecL’[x,y] is also a concern for implementation. Additional buffer is needed only for storing RecL’[x,y] as their positions are different from the ones of original luma pixels. According to the current HM specification all information of Y component is transmitted then U&V component follows in the unit of CU. In the worst case the size of a CU can be 128x128 for Luma, and buffer with the size of 64x64 is needed to store Rec’[x,y].

In this proposal not 10-bit but 8-bit samples are used for storing Rec’[x,y] to reduce the size of the buffer.

# Simulation

## Simulation Condition

The followings are anchor and to be tested:

Anchor:	 		HM-3.0

Off:				HM-3.0 with LM_CHROMA = 0

To be tested -1:		HM-3.0 + Reduction A + Reduction B

To be tested -2:		HM-3.0 + Reduction A + Reduction B + Reduction C

Intra and Intra_LC conditions have been tested as described in [1].

## Simulation Result

Summary of the result of “to be tested -1” is shown in Table 3.2.3. Complete results are shown in JCTVC-F431_off.xls.

Summary of the result of “to be tested -1” is shown in Table 3.2.2. Complete results are shown in JCTVC-F431_110.xls.

Summary of the result of “to be tested -2” is shown in Table 3.2.3. Complete results are shown in JCTVC-F431_111.xls.

The result is verified by Panasonic [3].

Table 3.2.1

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.2 | 18.8 | 8.3 | 1.3 | 21.2 | 10.8 |
| Class B | 0.7 | 7.1 | 3.7 | 0.8 | 7.9 | 4.3 |
| Class C | 1.0 | 6.6 | 7.5 | 1.2 | 6.9 | 7.8 |
| Class D | 0.6 | 4.7 | 4.7 | 0.8 | 5.1 | 4.9 |
| Class E | 0.0 | 1.9 | 2.8 | 0.2 | 4.5 | 5.0 |
| All | 0.7 | 8.1 | 5.5 | 0.9 | 9.3 | 6.5 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3.2.2

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.9 | -1.0 | 0.4 | 2.9 | 0.2 |
| Class B | 0.1 | 0.7 | -0.2 | 0.4 | 1.8 | 0.7 |
| Class C | 0.1 | 0.2 | 0.2 | 0.4 | 0.9 | 0.8 |
| Class D | 0.0 | 0.0 | 0.1 | 0.2 | 0.4 | 0.3 |
| Class E | 0.0 | 0.0 | 0.4 | 0.2 | 1.9 | 0.9 |
| All | 0.1 | 0.4 | -0.1 | 0.3 | 1.6 | 0.6 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3.2.3

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.9 | -1.0 | 0.4 | 2.9 | 0.2 |
| Class B | 0.1 | 0.8 | -0.1 | 0.4 | 2.0 | 0.7 |
| Class C | 0.1 | 0.2 | 0.2 | 0.3 | 1.0 | 0.9 |
| Class D | 0.0 | 0.1 | 0.0 | 0.2 | 0.5 | 0.5 |
| Class E | 0.0 | 0.0 | 0.4 | 0.2 | 1.9 | 1.1 |
| All | 0.1 | 0.4 | -0.1 | 0.3 | 1.7 | 0.7 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Conclusion

Chroma intra prediction by reconstructed luma samples has been adopted to HEVC specification. It contributes to coding efficiency improvement of chroma components. However increase in complexity is concern.

Without LM_CHROMA loss in coding efficiency is observed by 0.7% (Y), 8.1% (U) and 5.5% (V) with the IT condition and 0.9% (Y), 9.3% (U) and 6.5% (V) with the IT_Loco condition. With reduction A&B loss in coding efficiency can be decreased to 0.1% (Y), 0.4% (U) and -0.1% (V) with the IT condition, and 0.3% (Y), 1.6% (U) and 0.6% (V) with the IT_Loco condition. With reduction A&B&C loss in coding efficiency can be decreased to 0.1% (Y), 0.4% (U) and -0.1% (V) with the IT condition, and 0.3% (Y), 1.7% (U) and 0.7% (V) with the IT_Loco condition.

The author recommends the proposed reductions be adopted so that complexity of LM_CHROMA reaches at reasonable level for implementation.

# References

[1] “Common test conditions and software reference configurations”, JCTVC-E700, Mar 2011

[2] “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, JCTVC-E266, Mar 2011

[3] “Cross-verification of Sony's proposal JCTVC-F431 by Panasonic”, JCTVC-F717, Jul 2011

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).