| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F435 |
| --- | --- |

| Title: | CE2: Cross-check of Non-Square Quadtree Transform JCTVC-F379, JCTVC-F410 & JCTVC-F412 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Simon Oudin Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 simon.oudin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results of JCTVC-F319 [1], JCTVC-F410 Error: Reference source not found and JCTVC-F412 [3], a study on HEVC of non-square RQT for symmetric partitions (2NxN and Nx2N partitions) and for Asymmetric Motion Partitions (AMP). Three versions of non-square RQT are tested: one only for symmetric partitions, one only for asymmetric partitions and one with both types of non-square partitions. The AMP is tested also separately. The proposed code has been checked, compiled and the results reported by Tsinghua/Huawei/USTC/Microsoft/HiSilicon can be confirmed.

# Tested Proposals

The tested proposal has been integrated into HEVC HM 3.0 [4]. Three versions of non-square RQT are tested. The code was checked and the software reflects what is proposed. While checking the code, the following was observed:

- There are two different implementations in order to cover the four methods. It would have been possible to compact these implementations in one and reduce the high level of code redundancy for the cross-checking.

- Couple of bugs discovered with the compiler gcc 4.4.5:

  - Variables created but not used, variables not initialized.

  - Variables created outside of the macro.

- It has to be mentioned that in JCTVC-F410 [2] the results of Table2 “NSQT + implicit TU” couldn’t be checked because the source code and the references of this test have not been made available for cross-checking.

Table 1-1	Methods presentation

| Methods | Non-Square RQT for Symmetric Partitions for CU | Non-Square RQT for Asymmetric partition for CU (AMP | Asymmetric partition for CU (AMP) | Implementation |
| --- | --- | --- | --- | --- |
| 1 | X |  |  | Impl1 |
| 2 |  |  | X | Impl2 |
| 2 |  | X | X | Impl2 |
| 3 | X | X | X | Impl2 |

## Method1

Compared to HM3.0, the following was changed for the Residual Quadtree (RQT):

- Add non-square partitions for RQT for the second and the third level. When The Coding Unit is partitioned in Nx2N or 2NxN, RQT becomes non-square.

## Method2

Compared to HM3.0, the following was changed for the Coding Units partitions and the RQT:

- Reintroduction of the AMP (Asymmetric Motion Partition) initially proposed in JCTVC-A124 [5].

## Method3

Compared to HM3.0, the following was changed for the Coding Units partitions and the RQT:

- Reintroduction of the AMP (Asymmetric Motion Partition) initially proposed in JCTVC-A124 [5].

- Add non-square partitions for RQT for the second and the third level. When The Coding Unit has an AMP partition, RQT becomes non-square.

## Method4

Compared to HM3.0, the following was changed for the Coding Units partitions and the RQT:

- Reintroduction of the AMP (Asymmetric Motion Partition) initially proposed in JCTVC-A124 [5].

- Add symmetric partitions for RQT for the second and the third level. When The Coding Unit has partition Nx2N or 2NxN, RQT becomes non-square.

- Add asymmetric partitions for RQT for the second and the third level. When The Coding Unit has an AMP partition, RQT becomes non-square.

# Simulation Environment and Results

## Software

The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [6]. The HM3.0 default settings, both for configuration file and compile-time, are used in this test as anchor.

## Hardware

Two types of computers have been used to run the simulations  all running a 64 bit Ubuntu 10.10 Linux operating system. They are listed in Table 2-2.

Table 2-2	 Computers used to run the simulations.

| Type | Purpose | Cores | CPU | RAM |
| --- | --- | --- | --- | --- |
| 1 | Decoding on 1 core without yuv | 1x4 | 4 x Intel(R) Core(TM)2 Extreme CPU Q9300 @ 2.53GHz | 4 GB |
| 2 | Encoding for test SP12 and HM3.0 anchor to be compared with | 2x6 | 2 x 6 x Intel(R) Xeon(R) CPU X5670 @ 2.93GHz | 48 GB |

## Results

The coding efficiency results in this section are given in terms of BD-rate. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheets JCTVC-F435-Method1.xls, JCTVC-F435-Method2.xls, JCTVC-F435-Method3.xls and JCTVC-F435-Method4.xls.

Table 2-3	Method1

Table 2-4	Method2

Table 2-5	Method3

Table 2-6	Method4

# References

- Il-Koo Kim, W.-J Han, J. H. Park, X. Zheng, “CE2: Test result of asymmetric motion partition (AMP)”, JCTVC-F379, Turino, July, 2011.

- Y. Yuan, X. Zheng, X. Peng, J. Xu, L. Liu, Y. Wang, X. Cao, C. Lai,  J. Zheng,  Y. He, H. Yu, “CE2: Non-Square Quadtree Transform for symmetric motion partition”, JCTVC-F410, Turino, July, 2011.

- Y. Yuan, X. Zheng, X. Peng, J. Xu, Il-Koo Kim , L. Liu, Y. Wang, X. Cao, C. Lai,  J. Zheng,  Y. He, H. Yu, “CE2: Non-Square Quadtree Transform for symmetric and asymmetric motion partition”, JCTVC-F412, Turino, July, 2011.

- JCT-VC, “HEVC Test Model Software 3.0”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0

- JCT-VC, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, JCTVC-A124, JCT-VC Meeting, Dresden, DE, April 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, March, 2011.