| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F440 |
| --- | --- |

| Title: | CE4: Cross check for MediaTek’s proposal JCTVC-F119 by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd |  |  |

_____________________________

# Abstract

This report provides cross-check results of CE4 subtest 3.3.1a (the part of MediaTek’s proposal on adaptive de-quantization off-set). Three variations were testes: adaptive de-quantization offset, adaptive de-quantization offset based lambda refinement (encoder modification) and both together. The simulation results matched the data provided by the MediaTek: 0.6% Luma / 1% Chroma gain for AQO, 0.2% Luma / 1.2% Chroma gain for lambda refinement and 1.1% Luma / 2.2% Chroma gain for combination.

# Simulation Results

This test was carried out using s/w provided by MediaTek. Here is the summary of test 3.3.1a in Tables 1.1-1.3(details can be found in accompanied spreadsheet). Test was done using common test conditions defined in JCTVC-E700. 4 inter test cases were investigated: RA-HE, RA-LC, LD(B)-HE and LD(B)-LC.

Table 1.1 Summary of test 3.3.1a (only AQO)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | -1.9 | -2.3 | -1.7 | -1.7 | -1.8 |
| Class B | -0.3 | -1.1 | -1.9 | -0.5 | 0.1 | -0.9 |
| Class C | -0.2 | -0.3 | -0.6 | -0.6 | 0.2 | 0.0 |
| Class D | 0.2 | -0.9 | -1.1 | -0.2 | -0.2 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -1.1 | -1.5 | -0.7 | -0.4 | -0.8 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.8 | -0.9 | -1.8 | -1.1 | -0.4 | -1.4 |
| Class C | -0.3 | -0.7 | -0.8 | -1.0 | 0.5 | 0.7 |
| Class D | 0.0 | -1.4 | -1.6 | -0.7 | -0.9 | -0.9 |
| Class E | -0.1 | -2.7 | -2.6 | -0.4 | -3.6 | -3.3 |
| All | -0.4 | -1.3 | -1.6 | -0.8 | -0.9 | -1.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2.2 Summary of test 3.3.1a (AQO with lambda refinement)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.7 | -2.8 | -3.6 | -3.7 | -3.7 | -3.8 |
| Class B | -0.8 | -2.8 | -3.9 | -0.9 | -1.5 | -3.0 |
| Class C | -1.1 | -1.9 | -1.9 | -1.3 | -1.2 | -1.5 |
| Class D | -0.6 | -3.0 | -2.9 | -0.7 | -2.1 | -2.0 |
| Class E |  |  |  |  |  |  |
| All | -1.3 | -2.6 | -3.1 | -1.6 | -2.1 | -2.6 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | -0.9 | -1.9 | -1.0 | -0.6 | -2.0 |
| Class C | -0.4 | -1.0 | -1.1 | -0.8 | 0.3 | 0.3 |
| Class D | -0.2 | -1.7 | -2.0 | -0.6 | -0.8 | -0.8 |
| Class E | -1.2 | -4.1 | -4.3 | -1.1 | -6.2 | -5.7 |
| All | -0.7 | -1.7 | -2.2 | -0.9 | -1.5 | -1.8 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 3.3 Summary of test 3.3.1a (only lambda refinement)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -1.3 | -1.7 | -2.4 | -3.0 | -2.8 |
| Class B | -0.4 | -1.7 | -2.3 | -0.2 | -1.3 | -1.9 |
| Class C | -0.7 | -1.4 | -1.4 | -0.5 | -1.3 | -1.2 |
| Class D | -0.4 | -2.1 | -2.0 | 0.0 | -1.7 | -1.4 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -1.6 | -1.9 | -0.7 | -1.8 | -1.8 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.1 | -0.2 | 0.6 | -0.3 | -0.7 |
| Class C | 0.2 | 0.1 | 0.1 | 0.6 | 0.1 | 0.0 |
| Class D | 0.2 | 0.0 | 0.2 | 0.7 | 0.0 | -0.4 |
| Class E | -0.8 | -2.2 | -2.6 | -0.3 | -3.2 | -3.2 |
| All | 0.0 | -0.4 | -0.5 | 0.5 | -0.6 | -0.9 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Performance in this test perfectly matches with data provided by MediaTek. Our estimation for encoding time is very similar to MediaTek’s data.

From these test data we may conclude that in strongest combination (AQO with lambda refinement) this tool provides 1.1% Luma / 2.2% Chroma gain without noticeable increment of encoding or decoding time.

We also would like to report performance of strongest combination (AQO with lambde refinement) if Nebuta and BQTerrace are excluded from average gain calculation (strange BD-rate behavior was reported for these test sequences).

Table 2 Performance of AQO with lambda refinement (Nebuta and BQTerrace are excluded).

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -3.1 | -3.7 | -1.2 | -2.9 | -3.0 |
| Class B | -0.9 | -1.9 | -2.9 | -1.2 | -1.0 | -2.1 |
| Class C | -1.1 | -1.9 | -1.9 | -1.3 | -1.2 | -1.5 |
| Class D | -0.6 | -3.0 | -2.9 | -0.7 | -2.1 | -2.0 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -2.4 | -2.8 | -1.1 | -1.7 | -2.1 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | -0.4 | -1.0 | -1.1 | -0.1 | -0.9 |
| Class C | -0.4 | -1.0 | -1.1 | -0.8 | 0.3 | 0.3 |
| Class D | -0.2 | -1.7 | -2.0 | -0.6 | -0.8 | -0.8 |
| Class E | -1.2 | -4.1 | -4.3 | -1.1 | -6.2 | -5.7 |
| All | -0.6 | -1.7 | -1.9 | -0.9 | -1.4 | -1.5 |

Average performance degradation is these two sequences are excluded is 0.1-0.2%.

Recently BD-rate measure was modified to use piece-wise cubic interpolation. We also report performance using this “experimental” measure. It seems this new one provides more stable measurement.

Table 3 Performance of AQO with lambda refinement (piece-wise cubic interpolation).

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -3.1% | -3.8% | -0.9% | -3.6% | -3.7% |
| Class B | -0.7% | -2.8% | -4.0% | -0.9% | -1.5% | -3.0% |
| Class C | -1.1% | -1.9% | -1.9% | -1.3% | -1.3% | -1.5% |
| Class D | -0.6% | -3.0% | -2.9% | -0.7% | -2.1% | -2.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.8% | -2.7% | -3.2% | -0.9% | -2.1% | -2.6% |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7% | -0.9% | -1.9% | -0.9% | -0.6% | -2.0% |
| Class C | -0.4% | -1.0% | -1.1% | -0.8% | 0.2% | 0.3% |
| Class D | -0.1% | -1.7% | -2.0% | -0.6% | -0.9% | -0.8% |
| Class E | -1.2% | -4.1% | -4.3% | -1.1% | -6.2% | -5.7% |
| Overall | -0.6% | -1.7% | -2.2% | -0.8% | -1.5% | -1.8% |

# Results of code investigation

During code investigation we’ve found that in current implementation MediaTek’s AQO uses division operation in de-quantization with denominator 100. This is definitely not desirable for fast decoder realization. We hope that this division operation can be removed with minor performance change.

# Conclusions

CE4 subtest 3.3.1.a (adaptive de-quantization off-set) is verified in this report. The simulation results matched the data provided by the proponent.