| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F443 |
| --- | --- |

| Title: | CE3: Cross-check by Samsung for Motorola Mobility tests (JCTVC-F574,576) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd |  |  |

_____________________________

# Abstract

4 tests presented by Motorola Mobility in CE3 are verified in this report. Performance reported by proponent was confirmed. Together with up to 0.5% performance improvement for Luma performance the drop for Chroma (which is higher than Luma gain) was observed in all tests. About 8% increase of fractional position in MC (compare to HM3.0) was observed. Complexity was measured using actual hit-ration for each interpolation this test. Analysis of possible effect proposed by Motorola Quarter pel interpolation filter will have on MC process is provided. Explanation for chroma drop is provided as well.

# Test conditions

Test was done using common test conditions defined in JCTVC-E700. In total 6 test cases were investigated: RA-HE, RA-LC, LD(B)-HE , LD(B)-LC, LD(P)-HE and LD(P)-LC.

# Fixed filter: 7 taps 1/4 pel and 6 taps 1/2 pel (7Q + 6H)

Motorola Mobility modified Hm3.0 interpolation filter in this test, but still use minimum set of filter coefficients for 1/4 pel MC. Quarter pel and Half pel filters uses different number of taps:

Q: {-1,   3,   -8,   60,  13,  -4,   1,   0}, (7 taps),                  (1)

H: { 0,   2,   -9,  39,   39,  -9,   2,   0}, (6 taps).                   (2)

Test results are summarized in Table 1 (details can be found in attached excel sheet).

Table 1. Test results summary for 7Q + 6 H test.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | 0.1 | -0.1 | -0.4 | 0.4 | 0.7 |
| Class B | -0.1 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.4 | 0.7 | 0.6 | 1.3 | 0.6 | 0.7 |
| Class D | 0.9 | 1.1 | 0.9 | 3.1 | 1.4 | 1.5 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.4 | 0.4 | 0.9 | 0.6 | 0.7 |
| Enc Time[%] | 96% |  |  | 94% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.6 | 0.5 | -0.5 | 0.3 | -0.4 |
| Class C | 0.5 | 1.6 | 1.9 | 1.1 | 1.7 | 1.7 |
| Class D | 1.4 | 4.2 | 4.6 | 2.9 | 2.7 | 3.2 |
| Class E | -0.7 | -0.2 | -0.2 | -2.0 | -1.7 | -1.4 |
| All | 0.3 | 1.6 | 1.8 | 0.5 | 0.9 | 0.9 |
| Enc Time[%] | 96% |  |  | 94% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | only P |  |  | only P LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | 0.1 | -0.1 | -1.9 | -0.4 | -0.4 |
| Class C | -0.3 | 1.3 | 1.1 | -0.2 | 1.2 | 1.4 |
| Class D | 0.6 | 3.5 | 3.7 | 1.1 | 2.2 | 2.7 |
| Class E | -1.7 | -0.9 | -0.6 | -4.5 | -3.4 | -2.3 |
| All | -0.5 | 1.1 | 1.1 | -1.2 | 0.1 | 0.5 |
| Enc Time[%] | 97% |  |  | 95% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Y | U | V |  |  |  |
| Avg. 6 cases: | 0.05% | 0.77% | 0.87% |  |  |  |
| MinBDRate | -7.8% | -4.9% | -2.7% |  |  |  |
| MaxBDRate | 8.5% | 11.5% | 12.25 |  |  |  |

Tested modification of HM3.0 interpolation filter provides variation of performance from -7.8% (Lum a gain) to 8.5% (Luma drop). In average this variant shows almost no performance drop in Luma component but more than 0.8% drop in Chroma component.

# Fixed filter: 7 taps 1/4 pel and 8 taps 1/2 pel (7Q + 8H)

In this test Motorola Mobility modified only quarter pel interpolation filter compare to HM3.0. For half pel position HM3.0 filter is used.

Q: {-1,   3,   -8,   60,  13,  -4,   1,   0}, (7 taps),

H: {-1,  4,    -11,  40,  40,  -11,  4,  -1}, (8 taps).

Test results are summarized in Table 2 (details can be found in attached excel sheet).

Table 2. Test results summary for 7Q + 8 H test.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1 | 0.0 | 0.0 | -0.4 | 0.2 | 0.5 |
| Class B | -0.2 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |
| Class C | -0.1 | 0.3 | 0.2 | 0.3 | 0.3 | 0.4 |
| Class D | -0.2 | 0.4 | 0.3 | 0.8 | 0.5 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | 0.2 | 0.2 | 0.1 | 0.3 | 0.3 |
| Enc Time[%] | 97% |  |  | 95% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.6 | 0.3 | -0.7 | 0.1 | -0.1 |
| Class C | 0.0 | 1.2 | 1.4 | 0.3 | 1.2 | 1.1 |
| Class D | 0.3 | 3.2 | 3.6 | 0.9 | 1.3 | 2.1 |
| Class E | -0.5 | 0.0 | 0.0 | -1.6 | -1.7 | -0.9 |
| All | -0.1 | 1.3 | 1.3 | -0.2 | 0.3 | 0.6 |
| Enc Time[%] | 97% |  |  | 95% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
|  | only P |  |  | only P LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | 0.1 | -0.3 | -1.5 | -0.2 | -0.3 |
| Class C | -0.9 | 0.7 | 0.9 | -0.7 | 0.9 | 0.9 |
| Class D | -0.7 | 2.5 | 2.5 | -0.6 | 1.5 | 1.3 |
| Class E | -1.3 | -0.6 | -0.8 | -3.4 | -2.8 | -1.9 |
| All | -0.9 | 0.7 | 0.6 | -1.4 | 0.0 | 0.1 |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Y | U | V |  |  |  |
| Avg. 6 cases: | -0.46 | 0.46 | 0.53 |  |  |  |
| MinBDRate | -6.0 | -4.0 | -2.5 |  |  |  |
| MaxBDRate | 2.0 | 8.3 | 9.6 |  |  |  |

Tested modification of HM3.0 interpolation filter provides variation of performance from -6.0% (Luma gain) to 2.0% (Luma drop). In average this variant shows performance improvement for Luma component (0.46%), but 0.46/0.53% (U/V) drop in Chroma component.

# Fixed filter: 8 taps 1/4 pel and 8 taps 1/2 pel (8Q + 8H)

Once again only 1/4-pel position interpolation filter is modified compare to HM3.0:

Q: {-1,   3,  -8,   60,   14,  -6,   3,   -1}, (8 taps),                 (3)

H: {-1,  4,    -11,  40,  40,  -11,  4,  -1}, (8 taps).

Test results are summarized in Table 3(details can be found in attached excel sheet).

Table 3. Test results summary for 8Q + 8 H test.

|  |  | Random access |  |  |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V | Y |  | U |  | V |
| Class A | 0.0 | 0.1 |  | 0.3 | -0.1 |  | 0.3 |  | 0.6 |
| Class B | -0.1 | 0.1 |  | 0.1 | -0.3 |  | 0.1 |  | 0.1 |
| Class C | -0.2 | 0.2 |  | 0.1 | -0.4 |  | 0.3 |  | 0.3 |
| Class D | -0.2 | 0.1 |  | 0.0 | -0.5 |  | 0.1 |  | 0.1 |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -0.1 | 0.1 |  | 0.1 | -0.3 |  | 0.2 |  | 0.2 |
| Enc Time[%] | 98% |  |  |  | 98% |  |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |  |
|  | Y | U |  | V | Y |  |  | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1 | 0.9 |  | 0.8 | -0.2 |  |  | 0.3 | -0.1 |
| Class C | 0.0 | 1.4 |  | 1.4 | -0.5 |  |  | 1.4 | 1.3 |
| Class D | -0.2 | 3.0 |  | 3.7 | -0.7 |  |  | 1.6 | 1.7 |
| Class E | 0.3 | 0.2 |  | 1.0 | 0.2 |  |  | -0.8 | -0.5 |
| All | 0.0 | 1.4 |  | 1.7 | -0.3 |  |  | 0.7 | 0.6 |
| Enc Time[%] | 98% |  |  |  | 98% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |  |
|  | only P |  |  |  | only P LoCo |  |  |  |  |
|  | Y |  | U | V | Y |  |  | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.5 |  | 0.3 | 0.0 | -0.1 |  |  | 0.1 | -0.1 |
| Class C | -1.1 |  | 0.7 | 0.7 | -0.9 |  |  | 1.1 | 1.3 |
| Class D | -1.5 |  | 2.0 | 2.5 | -1.5 |  |  | 1.3 | 1.9 |
| Class E | -0.6 |  | 0.2 | -0.1 | 1.1 |  |  | -1.8 | -0.8 |
| All | -0.9 |  | 0.8 | 0.8 | -0.4 |  |  | 0.3 | 0.6 |
| Enc Time[%] | 99% |  |  |  | 98% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |  |
|  | Y |  | U | V |  |  |  |  |  |
| Avg. 6 cases: | -0.34 |  | 0.58 | 0.68 |  |  |  |  |  |
| MinBDRate | -4.4 |  | -2.0 | -2.0 |  |  |  |  |  |
| MaxBDRate | 1.3 |  | 8.0 | 10.2 |  |  |  |  |  |

Tested modification of HM3.0 interpolation filter provides variation of performance from -4.4% (Luma gain) to 1.3% (Luma drop). In average this variant shows performance improvement for Luma component (0.34%), but 0.58/0.68% (U/V) drop in Chroma component.

# Slice type dependent interpolation filter.

In this test Motorola Mobility modified 1/4-pel position interpolation filter in HM3.0 according slice type.

Q: {-1,   3,  -8,   60,   14,  -6,   3,   -1}, (8 taps for GPB on),

Q: {-1,   3,   -8,   60,  13,  -4,   1,   0}, (7 taps for GPB off),

Test results are summarized in Table 4(details can be found in attached excel sheet).

Table 4. Test results summary for slice dependent AIF from Motorola.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.3 | -0.1 | 0.3 | 0.6 |
| Class B | -0.1 | 0.1 | 0.1 | -0.3 | 0.1 | 0.1 |
| Class C | -0.2 | 0.2 | 0.1 | -0.4 | 0.3 | 0.3 |
| Class D | -0.2 | 0.1 | 0.0 | -0.5 | 0.1 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | 0.1 | 0.1 | -0.3 | 0.2 | 0.2 |
| Enc Time[%] | 106% |  |  | 108% |  |  |
| Dec Time[%] | 105% |  |  | 108% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.9 | 0.8 | -0.2 | 0.3 | -0.1 |
| Class C | 0.0 | 1.4 | 1.4 | -0.5 | 1.4 | 1.3 |
| Class D | -0.2 | 3.0 | 3.7 | -0.7 | 1.6 | 1.7 |
| Class E | 0.3 | 0.2 | 1.0 | 0.2 | -0.8 | -0.5 |
| All | 0.0 | 1.4 | 1.7 | -0.3 | 0.7 | 0.6 |
| Enc Time[%] | 107% |  |  | 109% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |
|  | only P |  |  | only P LoCo |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7 | 0.1 | -0.3 | -1.5 | -0.2 | -0.3 |
| Class C | -0.9 | 0.7 | 0.9 | -0.7 | 0.9 | 0.9 |
| Class D | -0.7 | 2.5 | 2.5 | -0.6 | 1.5 | 1.3 |
| Class E | -1.3 | -0.6 | -0.8 | -3.4 | -2.8 | -1.9 |
| All | -0.9 | 0.7 | 0.6 | -1.4 | 0.0 | 0.1 |
| Enc Time[%] | 105% |  |  | 107% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |
|  | Y | U | V |  |  |  |
| Avg. 6 cases: | -0.51 | 0.52 | 0.56 |  |  |  |
| MinBDRate | -6.0 | -4.0 | -2.5 |  |  |  |
| MaxBDRate | 0.6 | 8.0 | 10.2 |  |  |  |

This is table is combination of Tables 2 (bottom part) and 3 (upper part). An average gain 0.51% for Luma is accomplished by even slightly higher drop in Chroma component (0.52% for U and 0.56% for V).

# About filter coefficient generation

In total Motorola Mobility used 3 interpolation filters in their tests, which is different from HM3.0 filter. 6 taps interpolation filter for half-pel position is DCT derived interpolation filter, generation process for this filter is described in JCTVC-F247.

Here we would like to analyze introduced by Motorola 7 and 8 taps filters for 1/4 pel position.

In order to determine efficient phase shift provided by new filters let’s apply them to model linearly growing reference signal:

| p-3 | p-2 | p-1 | p0 | p1 | p2 | p3 | p4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

In ideal case the result of 1/4-pel interpolation filter application to this signal should be p = 4.25. Let’s apply interpolation filter with coefficients (1) to this model signal:

p = (-1 *1+ 3*2 -8*3+ 60*4+13*5 -4*6+ 1*7)/64 =4.20.

This correspondes to displacement by 1/5, not 1/4.

Let’s apply interpolation filter with coefficients (1) to this model signal:

p = (-1 *1+ 3*2 -8*3+ 60*4+14*5 -6*6+ 3*7 -1*8)/64 =4.1875.

here efficient displacement is even less than 1/5.

There is the formula for DCTIF combined with smoothing windowing from [1]:

.

(4)

Here is window-size, .are boundary of reference pixels template. Both odd and even numbers of filter taps are covered by this formula. Let’s set Mmin = -3, Mmax=3, =0.2, N=10.2 in (4) then we’ll obtain 7-taps filter (1):

{-1,   3,   -8,   60,  13,  -4,   1,   0},

If we’ll set Mmin = -3, Mmax=4, =0.1875, N=36 in (4) then we’ll obtain 8-taps filter (3):

{-1,   3,  -8,   60,   14,  -6,   3,   -1}.

So newly generated by Motorola quarter-pel interpolation filter actually “shifts” reference signal by ~1/5. Actual Luma motion compensation in Motorola’s scheme use fractional positions {0, 1/5, 1/2, 4/5, 1}. This non-uniform mesh of fractional positions is definitely revolution idea proposed by Motorola and this phenomenon should be carefully studied.

In conventional 1/4 pel motion compensation scheme the mesh of fractional positions for Luma {0, 1/4,1/2, 3/4, 1} is synchronized with Chroma fractional positions {0,1/8,1/4,3/8,1/2,5/8,3/4,7/8,1}. If quarter pel filter is changed and uses effective phase shift  then 1/8, 3/8, 5/8, 7/8 chroma filters should be redesigned with phase shift /2, 1/2-/2, 1/2+/2 and 1-/2 correspondently. This will restore synchronization of Luma and Chroma MC.

| Fig 1. 1/4 pel MC for Luma synchronized with 1/8 pel MC for Chroma. | Fig 2. Fractional positions for Luma and Chroma if phase shift in “1/4 pel” Luma filter is 1/5. |
| --- | --- |

Fig. 1 and 2 show the mesh of fractional positions for synchronized Luma and Chroma MC in cases conventional 1/4 pel Luma MC (phase shift 1/4) and modified “1/4 pel” Luma filter (phase shift 1/5). Chroma fractional positions are shown by “”.

Efficiently MC after quarter-pel filter modification proposed by Motorola is performed using mesh on Fig 2. This is non-uniform mesh in which “1/4 pel” positions are closer to int-pel than to half-pel. This disbalance is not that noticeable for Luma mesh but much more noticeable in Chroma fractional positions mesh. There is dead zone (between for example 1/5 and 4/10) where Chroma motion compensation is impossible. Even if Chroma 1/8-pel filter will be synchronized with Luma 1/4 pel filter then chroma drop observed in all tests will become smaller, but could not disappear because of mentioned “dead zone” phenomenon. This statement confirmed by JCTVC-F599, where proponent synchronized Luma and Chroma interpolation filters and used MC scheme equal to Fig.2. Chroma drop became smaller, but still not disappear.

Table 5. Average across 6 test cases performance of JCTVC-F574 and JCTVC-599 (compare to HM3.0).

| Luma filter | Only Luma IF modification JCTVC-F574 | Synchronized Luma and Chroma IF JCTVC-F599 |
| --- | --- | --- |
| 7Q+6H | -0.46%/0.46%/0.53% | -0.48%/0.18%/0.07% |
| 8Q+8H | -0.34%/0.58%/0.68% | -0.38%/0.10%/0.03% |

It seems a bit strange if proponent has modified both Luma and Chroma interpolation filters but still could not achieve performance improvement for Chroma component. This is important to mention high dependence of performance on test video. For example if one exclude 2 video sequences (BQTerrace and Vydio3) from average gain calculation then -0.48%/0.18%/0.07% (JCTVC-F599) will become -0.25%/0.22%/0.15% (twice reduction of Luma gain with increment of Chroma drop).

As was confirmed by proponent actual phase shift for “1/4 pel” filter generation was 3/16 (smaller than ¼). Fig. 1-2 show the difference between MC with normal ¼-pel filter and asymmetric one uses phase shift smaller than ¼.

0

1/2

α

Fig 1. Attraction area for fractional positions in MC if phase shift different from ¼ is used.

0

1/2

1/4

Fig. 2. Attraction area for fractional positions in MC if phase shift is ¼.

Because of displacement of fractional position ¼ α (α<¼) attraction area for integer position in MC is reduced (for α =3/16 by ~6%) which results in lower hit-ratio of integer pel in MC. And the number of integer pel for Chroma is reduced as well. Clear conclusion from this analysis is asymmetric interpolation filter with α≠¼ is more complex in terms of number of operations and memory band-width compare to symmetric one.

# Synchronization of Luma and Chroma MC

As it is mentioned in section 6 the usage of 6 asymmetric interpolation filter for Luma requires change of Chroma interpolation filter as well because three color components share the same MV. Synchronization of Luma and Chroma MC depends on sampling mode: 4:2:0 or 4:4:4.

In 4:2:0 color format efficient shift for Chroma MC should be twice smaller than for Luma (refer to Fig. 3) in order to have adequate displacement for positions with the same MV.

0

1/2

α

1/2

1/4

0

Fig. 3 Synchronization of Luma and Chroma MC for 4:2:0 color format.

Fractional ½ Luma position shares the same MV with ¼ Chroma fractional position. The introduction of asymmetric “1/4-pel” position for Luma requires “1/8” Chroma position filter change (1/4-pel Chroma filter requires no changes).

For 4:4:4 color format shift Luma and Chroma positions are always the same and should use the same phase shift (because MV is the same). In this color format “1/8”-pel filter for Chroma component simply not used. If we allow asymmetric “1/4” –pel filter for Luma (α≠¼) then Luma and Chroma component will share the same MV but perform different displacement. This will result in trembling of Chroma prediction signal (especially for artificially generated images, which will happen a lot in screen content coding) and provide chroma drop which in 4:4:4 format will be much higher compare to chroma drop shown in Motorola’s test before synchronization Luma and Chroma MC.

Same MV

0

1/2

α

1/2

1/4

0

The conclusion we would like to make here is following. Only if we use phase shift equal to ¼ we can synchronize Luma and Chroma MC for both color format: 4:2:0 and 4:4:4 (and any other reasonable sampling mode using dawn-sampling factor which is power of 2). By allowing asymmetric interpolation filter we break idea of usage the only one interpolation filter for all cases. Changing of color format will require introduction of new filter set for Chroma component which is not desirable.

Probability of sharing of Luma and Chroma interpolation filters is very low because Chroma signal is much smoother than Luma (not because to dawn-sampling but because of RGB to YUV conversion rules). So for Luma longer filter size will be needed to take into account fine details of Luma signal. But for Chroma extension of filter taps will lead to complexity increment which will not lead to adequate performance improvement ([6]).

# Complexity analysis

Here we present test results including complexity measurement described in [1] and agreed to be used for CE3 during BoG meeting. Table 6 shows both average gain and average number of computations and memory access in MC part measured for several (not for all) tests contributed by Motorola. Extreme characteristics such as the worst case memory band-width and mimimum/ maximum BD-rate are also reported. Test from F-247 is added for comparison of asymmetric and symmetric “1/4”- pel filters (1/2-pel filter is the sane with test 6h7q in F-574 and only ¼-pel filters are different, but both are 7 taps).

Table 6. Performance/complexity analysis for verified proposals.

| JCTVC-F | 574* | 247 | 599 | 599 |
| --- | --- | --- | --- | --- |
| description | 6h7q | 6h7q | 8h8q | 8h7q |
| Avg 6 Luma BD-rate | 0,05 | 0,07 | -0,38 | -0,48 |
| Min Luma BD-rate | -7,81 | -3,5 | -4,3 | -5,9 |
| Max Luma BD-rate | 8,45 | 3,0 | 1,2 | 2,0 |
| Avg Mult | 93% | 85% | 108% | 101% |
| Avg Adds | 91% | 83% | 108% | 99% |
| Avg MemBand | 97% | 95% | 102% | 100% |
| Worst case MemBand | 83% | 83% | 100% | 100% |

(*) Chroma performance drop ~0.8% was observed in this test.

First observation form these test results is following. Sorter interpolation filter doesn’t mandatory mean less complexity of MC.

The second observation is following. Comparing the first (7q+6h asymmetric) and the second (7q+6h symmetric) filter tests we observe that asymmetric filter provides the same Luma gain as symmetric one but uses 8% more computations and 2% more memory access on decoder side (compare to symmetric one).

Comparing two most right columns we conclude 8h+7q filter provides both better performance and less complexity compare to 8h+8q proposed by Motorola.

# Conclusions

By this analysis we’ve concluded that there are two promising interpolation filter solutions for HEVC: asymmetric 7q+8h from Motorola (providing 0.48 gain with complexity equal to HM3.0 filter) and symmetric 7q+6h filter from Samsung (providing only 0.07% loss with 16% less computations and 5% less average memory band-width and 17% reduced worst case memory band-width).

Taking into account drawbacks we’ll have from asymmetric interpolation filter we would like to propose avoid adoption of asymmetric filter solution from Motorola.

.

# Reference

[1] E.Alshina, A.Alshin “CE3: DCT derived interpolation filter test by Samsung” JCTVC-E247, 6th. JCTVC, Meeting, Turino, July. 2011

[2] Elena Alshina, Jianle Chen, Alexander Alshin, Nikolay Shlyakhov,Woo-Jin Han, “CE3: Experimental results of DCTIF by Samsung,” JCTVC-D344, Daegu, Korea, Jun. 2011

[3] J. Lou, K. Minoo, D. Baylon, K. Panusopone, L. Wang, Ajay Luthra, “Fixed interpolation filter tests by Motorola Mobility,” JCTVC-F574, Turin, Italy, July, 2011

[4] J. Lou, K. Minoo, D. Baylon, K. Panusopone, L. Wang, Ajay Luthra, “CE3: Slice-type based adaptive interpolation filter tests by Motorola Mobility,” JCTVC-F574, Turin, Italy, July, 2011

[5] Koohyar Minoo, Jian Lou, Ajay Luthra, “Chroma interpolation filters,” JCTVC-F599, Turin, Italy, July, 2011

[6] Elena Alshina, Jianle Chen, Alexander Alshin, Nikolay Shlyakhov,Woo-Jin Han, “Summary of CE4: interpolation for MC (Chroma)” JCTVC-D345, Daegu, Korea, Jun. 2011