| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F446 |
| --- | --- |

| Title: | CE10: Core transform design for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems G. Bjøntegaard, Cisco Systems M. Sadafale, Texas Instruments Inc., USA M. Budagavi, Texas Instruments Inc., USA | Tel: Email: Tel: Email Tel: Email Tel: Email | +47 92414082 arild.fuldseth@cisco.com gbjonteg@cisco.com mangesh@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Cisco Systems, Texas Instruments Inc |  |  |

_____________________________

# Abstract

This contribution proposes a set of transform matrices for HEVC, covering all transform sizes from 4x4 to 32x32. The transform matrices and the associated transform operations described in this contribution are proposed for the core transform design in HEVC. The proposed transform design has the following properties; 16 bit data representation before and after each transform stage (independent of the internal bit depth), 16 bit multipliers for all internal multiplications, no need for correction of different norms of basis vectors during quantization/dequantization, all transform sizes above 4x4 can reuse arithmetic operations for smaller transform sizes, and implementations using either pure matrix multiplication or a combination of matrix multiplication and butterfly structures are possible. Complexity analysis based on both arithmetic operation counts and performance results for optimized software is provided for all transform sizes, including partial transforms. By combining statistics from HM3.0 bit-streams and cycle counts for optimized transform implementations, it is found that the estimated CPU load of the inverse transforms when decoding  the class B sequences (1080p) using a 3.5 GHz Intel processor is between 0.3% and 5.6%. Finally, BD-rate results for increased bit depth of the intermediate transform buffers are provided. The BD-rate gains observed when increasing the transform bit depths from 16 to 18 and 20 (for internal bit depths of 10 and 12 respectively) are found to be less than 0.5%.

# Introduction

In [1] and [2] transform designs facilitating matrix multiplication and 16 bit intermediate data representations were proposed independently by Cisco and TI. These contributions were combined into a joint proposal in [3]. The transform design of [3] was compared to the existing core transforms of HM2.0 and average BD-rate gains of 0.1%-0.6% and 0.8%-2.5% were reported for normal/high QP range and low QP range respectively. In the 5th meeting of the JCT-VC it was decided to adopt the transform design of [3] in the HM software. This contribution proposes the exact same transforms as in [3] for adoption in the working draft, and provides additional bit-depth and complexity analysis.

The proposed transform design has the following properties.

- Bit width of 16 for all transform buffers. This is true for all values of internal bit depth parameter for the encoder/decoder. Experimental results in section 5 shows that increasing the bit depth of the transform buffers to 18 or 20 bit provide less than 0.5% average BD-rate gain.

- 16 bit multipliers for all internal calculations. This is true for all values of the internal bit depth parameter.

- The norms of the basis vectors are sufficiently close so that the same scaling can be used for all transform coefficients during quantization/dequantization. Also, the scaling is the same for all transform sizes, except for right shifts that depend on the transform size. This eliminates the need for large quantization/dequantization matrices.

- Coefficients and basis vectors of the smaller transforms are a subset of coefficients and basis vectors of larger transforms. This allows for re-use of logic for multiply/adds in hardware design.

- The proposed transform design is specified in terms of NxN matrices and also the associated right shift to be used after each transform stage. This allows for more additional flexibility in implementations, such as direct matrix multiplications with SIMD instruction for the innermost loop, or multi-stage butterfly implementations utilizing symmetry properties.

- 8 bit representation of transform coefficients.

- Bit width of accumulators for matrix multiplication (before right shift and storage) does not exceed 32 bit.

- Same symmetry/anti-symmetry properties as the DCT transform.

- The scaled transform coefficients are close to the DCT transform.

- The basis vectors are almost orthogonal.

- The same quantization/dequantization scheme can be used for all transform sizes. Quantization/dequantization is implemented with multipliers and shift operations, where the multipliers depend only on the value of QP and the shifts depend only on the transform size.

# Description

2.1 Basis vectors

The basis vectors of the 4x4 and 8x8 transforms are shown below. The basis vectors of the 16x16 and 32x32 transforms are shown in Appendix A. We have also designed a 64x64 transform that can be made available in the software for experimental purposes.

4x4 transform:

{64, 64, 64, 64}

{83, 36,-36,-83}

{64,-64,-64, 64}

{36,-83, 83,-36}

8x8 transform:

{64, 64, 64, 64, 64, 64, 64, 64}

{89, 75, 50, 18,-18,-50,-75,-89}

{83, 36,-36,-83,-83,-36, 36, 83}

{75,-18,-89,-50, 50, 89, 18,-75}

{64,-64,-64, 64, 64,-64,-64, 64}

{50,-89, 18, 75,-75,-18, 89,-50}

{36,-83, 83,-36,-36, 83,-83, 36}

{18,-50, 75,-89, 89,-75, 50,-18}

2.2 Intermediate scaling and quantization/dequantization.

In this subsection, intermediate scaling and quantization/dequantization are described in more detail. In addition to specifying the scaling in the dequantization and the inverse transform, we also specify the forward transform and quantizer that we have used for our simulation results.

Definitions

B	= internal bit depth (as specified by InternalBitDepth in the common config files)

N 	= transform size

M	= log2(N)

Q	= f(QP%6), 	where f(x)   =   {26214,23302,20560,18396,16384,14564}, x=0,…,5

IQ 	= g(QP%6), 	where g(x)  =   {40,45,51,57,64,72}, x=0,…,5

Forward transform

The following applies to the implementation of the forward transform.

- The forward transform is specified as a matrix multiplication in two stages, horizontal and vertical.

- After the first stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(M-1+(B-8)),		offset = (1<<(M-2+ (B-8)))

- After the second stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(M+6),		offset = (1<<(M+5))

This ensures a bit width of 16 after each transform stage.

Quantization

Quantization can be implemented as follows (RDOQ=off):

level  =  (coeff*Q + offset)>>(21+QP/6–M–(B-8))

For RDOQ, the scaling and cost function calculations are adjusted accordingly.

Dequantization

The dequantizer is specified as follows

coeffQ 	=  ((level*IQ << (QP/6)) + offset)>>(M-1+(B-8)),  offset = 1<<(M-2+(B-8))

coeffQ = min(32767,max(-32768,coeffQ))

The clipping ensures that the value of the quantized transform coefficients, coeffQ, can be represented with 16 bits. The scaling is chosen so that clipping only occur for extreme input values and/or unreasonable quantizer behaviour.

Inverse transform

The following applies to the implementation of the inverse transform.

- The inverse transform is specified as a matrix multiplication in two stages, horizontal and vertical.

- After the first stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>7			offset = 1<<(7-1)

This ensures a bit width of 16 after the first inverse transform stage assuming the quantization/dequantization scheme above.

- After the second stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(12-(B-8))		offset = 1<<(11-(B-8))

Quantization step size and QP

The quantizer multipliers, f(x) and dequantizer multipliers, g(x) are chosen so that

f(x) * g(x)  (1<<20), 	x = 0,1,…,5

Furthermore, assume that we have an N*N DC block with pixel values equal to X.  If this block is input to an orthonormal transformation containing a DC basis vector (e.g. the Discrete Cosine Transform), the value of the DC coefficient is equal to X*N.  With the proposed transforms and quantization/ dequantiztion scheme, the same relationship between input pixel values and the value of level applies when QP = 4. In other words, if all pixel values in an NxN block are equal to X, and QP=4, this will result in a DC coefficient with level = X*N after quantization. The range of QP values maps to approximately the same range of quantization step sizes as in HM2.0.

# Matrix multiplication and partial butterfly

The proposed transforms can be implemented in two ways; matrix multiplications, and partial butterfly.

3.1 Matrix multiplication

With straightforward matrix multiplication, the number of operations for each transform stage is N3 multiplications and N2(N-1) additions. Note however, that matrix multiplication is particularly well suited for SIMD instructions. Assuming a SIMD width of N, the matrix multiplication can be implemented using N2 SIMD instructions.

3.2 Partial butterfly

The basis vectors shown in section 2 and in Appendix A have symmetry properties that are inherited from the DCT transform:

- Even rows with index 0,2,4,… are symmetric with a symmetry point before coefficient no. N/2.

- Odd rows with index 1,3,5,7,… are anti-symmetric with a symmetry point before coefficient no. N/2.

- Even rows with index 0,4,8,… have additional symmetry points before coefficients no. N/4 and 3N/4.

- Even rows with index 2,6,10,… have additional anti-symmetry points before coefficients no. N/4 and 3N/4.

For larger transforms this pattern repeats itself with additional symmetry and anti-symmetry points before coefficients n*N/8, n*N/16 etc.

These symmetry properties allows for using parts of the “butterfly” structures used for fast FFT-like algorithms to reduce the number of multiply/adds (e.g. the Chen algorithm). Thus, we refer to this implementation as partial butterfly.

In the HM software, both methods are supported, matrix multiplications for readability and partial butterfly for speed.

# Complexity analysis

This section provides a detailed complexity analysis. It is structured to comply with section B.2 of the core experiment description on core transform design [4].

4.1 Arithmetic operations

The numbers of arithmetic operations for the 2D inverse transform using partial butterfly implementation are shown in Table 1.

Table 1 Number of arithmetic operations for 2D inverse transforms of various sizes

| Transform size | Mult,16 bit | Mult,32 bit | Mult, total | Add | Shift |
| --- | --- | --- | --- | --- | --- |
| 4x4 | 48 | 0 | 48 | 64 | 32 |
| 8x8 | 352 | 0 | 352 | 448 | 128 |
| 16x16 | 2752 | 0 | 2752 | 3200 | 512 |
| 32x32 | 21888 | 0 | 21888 | 23808 | 2048 |

In [5] it was shown that the number of arithmetic operations of an NxN inverse transform is reduced significantly if one assumes a partial transform where only the KxK lower frequency sub-block has non-zero values. For instance, the number of multiplications are reduced approximately by a factor of (N/K)2 for the first stage and a factor of (N/K) for the second stage. Table 2 shows the number of arithmetic operations for partial transforms of various sizes. Also, the probabilities of each partial transform size (KxK) as reported in [6] are listed in the last column of Table 2 (high efficiency configuration). (Probabilities for the 8x8 transform are taken from the data used to generate the results of Table 5.)

Table 2 Number of arithmetic operations for partial 2D inverse transforms of various sizes

| NxN | KxK | Multiplications | Add | Shift | Probability |
| --- | --- | --- | --- | --- | --- |
| 8x8 | 8x8 | 352 | 448 | 128 | 0,47 |
| 8x8 | 4x4 | 132 | 228 | 96 | 0,53 |
| 16x16 | 16x16 | 2752 | 3200 | 512 | 0,10 |
| 16x16 | 8x8 | 1032 | 1512 | 384 | 0,30 |
| 16x16 | 4x4 | 420 | 820 | 320 | 0,60 |
| 32x32 | 32x32 | 21888 | 23808 | 2048 | 0,12 |
| 32x32 | 16x16 | 8208 | 10320 | 1536 | 0,26 |
| 32x32 | 8x8 | 3400 | 5320 | 1280 | 0,16 |
| 32x32 | 4x4 | 1476 | 3060 | 1152 | 0,46 |

By weighting the number of operations with their respective probabilities, one can calculate an average operation count for each transform size NxN. This is shown in Table 3.

Table 3 Average number of arithmetic operations for 2D inverse transforms of various sizes

| NxN | Multiplications | Add | Shift |
| --- | --- | --- | --- |
| 4x4 | 48,0 | 64,0 | 32,0 |
| 8x8 | 235,4 | 331,4 | 111,0 |
| 16x16 | 836,8 | 1265,6 | 358,4 |
| 32x32 | 3619,6 | 7798,0 | 1379,8 |

4.3 Parallelizability with SIMD

An optimized version of the inverse transform (partial butterfly version) was generated by using an automated tool for generating assembly code [7]. The inverse transform was implemented in a standalone test program to estimate the number of clock cycles used for the inverse transform alone. The results for an Intel Sandy Bridge processor are shown in Table 4. The table shows results for column-row order as implemented in HM3.0 and also for row-column order providing slightly better performance.

Table 4 Estimated cycle count for partial 2D inverse transforms of various sizes

| NxN | KxK | Column-row order | Row-column order |
| --- | --- | --- | --- |
| 8x8 | 8x8 | 167 | 152 |
| 8x8 | 4x4 | 108 | 78 |
| 16x16 | 16x16 | 943 | 859 |
| 16x16 | 8x8 | 547 | 467 |
| 16x16 | 4x4 | 304 | 233 |
| 32x32 | 32x32 | 5720 | 5322 |
| 32x32 | 16x16 | 2764 | 2442 |
| 32x32 | 8x8 | 1732 | 1405 |
| 32x32 | 4x4 | 1167 | 836 |

To estimate the total number of cycles associated with the inverse transform, statistics for each transform size was collected for one second of 1080p sequences (LD-HE configuration). By multiplying with the estimated cycle counts per transform from Table 4, the total estimated cycle count from the inverse transforms when decoding a 1080p sequence can be estimated. The results are shown in Table 5. Assuming a CPU clock frequency of 3.5 GHz, the estimated number of clock cycles used for the inverse transforms corresponds to between 0.3% and 5.6% CPU load. The results of Table 5 use column-row order as in HM3.0. For row-column order, the numbers would be correspondingly lower.

Table 5 Estimated number of clock cycles used on the inverse transforms when decoding one second of 1080p sequences

| NxN |  | 4x4 | 8x8 |  | 16x16 |  |  | 32x32 |  |  |  | Transform |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KxK |  | 4x4 | 4x4 | 8x8 | 4x4 | 8x8 | 16x16 | 4x4 | 8x8 | 16x16 | 32x32 | Mega- | Transform |
|  |  | Estimated cycle count per transform (from Table 4) |  |  |  |  |  |  |  |  |  | cycles | CPU |
|  |  | 50 | 108 | 167 | 304 | 547 | 943 | 1167 | 1732 | 2764 | 5720 | per | load (%) |
| Sequence | QP | Number of inverse transforms per second (HM3.0, LD-HE configuration, 1 second) |  |  |  |  |  |  |  |  |  | second | (3.5 GHz) |
| Kimono | 22 | 27 438 | 45 401 | 2 669 | 30 142 | 23 978 | 2 221 | 1 390 | 8 322 | 13 230 | 623 | 87,3 | 2,5 |
|  | 27 | 11 589 | 24 049 | 613 | 31 301 | 6 391 | 493 | 2 682 | 11 383 | 5 348 | 149 | 55,2 | 1,6 |
|  | 32 | 5 491 | 13 090 | 118 | 22 249 | 1 398 | 120 | 3 963 | 8 387 | 1 736 | 50 | 33,6 | 1,0 |
|  | 37 | 2 986 | 8 437 | 15 | 14 074 | 258 | 12 | 4 359 | 5 470 | 395 | 7 | 21,2 | 0,6 |
| ParkScene | 22 | 204 750 | 42 876 | 38 952 | 4 767 | 4 860 | 6 619 | 764 | 571 | 617 | 735 | 39,5 | 1,1 |
|  | 27 | 94 043 | 30 898 | 11 965 | 4 407 | 3 336 | 2 687 | 675 | 261 | 269 | 404 | 20,0 | 0,6 |
|  | 32 | 50 559 | 23 568 | 3 050 | 4 662 | 2 613 | 1 046 | 729 | 214 | 257 | 247 | 12,8 | 0,4 |
|  | 37 | 25 867 | 17 114 | 477 | 5 521 | 1 864 | 190 | 891 | 304 | 376 | 110 | 9,3 | 0,3 |
| Cactus | 22 | 438 262 | 92 173 | 117 001 | 25 697 | 13 083 | 29 273 | 7 481 | 2 505 | 2 323 | 3 799 | 135,2 | 3,9 |
|  | 27 | 137 356 | 69 025 | 18 850 | 27 754 | 8 963 | 6 247 | 9 275 | 3 083 | 1 747 | 1 353 | 65,4 | 1,9 |
|  | 32 | 67 890 | 46 097 | 4 850 | 22 238 | 4 924 | 1 862 | 8 765 | 2 132 | 1 328 | 492 | 40,8 | 1,2 |
|  | 37 | 35 608 | 30 728 | 1 116 | 18 841 | 2 482 | 468 | 8 427 | 1 726 | 1 004 | 99 | 29,0 | 0,8 |
| BasketballDrive | 22 | 251 332 | 101 721 | 61 301 | 52 451 | 13 457 | 28 418 | 25 298 | 4 717 | 4 756 | 10 581 | 195,3 | 5,6 |
|  | 27 | 112 260 | 79 345 | 9 951 | 45 184 | 8 887 | 9 305 | 23 286 | 4 887 | 3 361 | 3 717 | 109,4 | 3,1 |
|  | 32 | 57 248 | 52 097 | 3 124 | 36 855 | 4 335 | 2 850 | 18 935 | 3 555 | 1 977 | 1 235 | 66,1 | 1,9 |
|  | 37 | 27 776 | 32 645 | 1 026 | 29 441 | 1 801 | 851 | 14 259 | 2 447 | 933 | 296 | 41,0 | 1,2 |
| BQTerrace | 22 | 630 406 | 18 311 | 510 667 | 3 128 | 1 595 | 21 523 | 1 652 | 356 | 589 | 2 554 | 159,7 | 4,6 |
|  | 27 | 195 869 | 22 031 | 66 015 | 3 936 | 2 480 | 12 169 | 1 897 | 435 | 786 | 2 712 | 57,9 | 1,7 |
|  | 32 | 74 501 | 17 487 | 10 159 | 4 143 | 1 943 | 4 325 | 1 082 | 231 | 349 | 1 507 | 25,0 | 0,7 |
|  | 37 | 42 765 | 14 700 | 2 798 | 4 076 | 1 642 | 990 | 1 001 | 141 | 450 | 466 | 12,6 | 0,4 |

4.3 Dynamic range analysis

The proposed transforms and the associated shifts/clipping are designed to ensure a bit width equal to 16 at all intermediate stages:

Forward transform, first stage
The scaling is adjusted to stay within 16 bit for worst case input signal.

Forward transform, second stage
The scaling is adjusted to stay within 16 bit for worst case input signal.

Dequantizer	
The scaling is chosen to stay within 16 bit for worst case input signal without quantization. Additional clipping is adjusted to stay within 16 bit for randomized quantizers.

Inverse transform, first stage
The scaling is chosen to stay within 16 bit for worst case input signal and reasonable quantizers. Alternatively, one can do clipping between the first and the second stage and adjust the shifts by one bit as described in section 5.1.

4.4 Number of sequential operations

For matrix multiplication implementation, the number of cascaded operators (multipliers/adders) in the longest execution path is 1 multiplication followed by 1 addition for all sizes of inverse transform.

For partial butterfly implementation, the number of cascaded operators (multipliers/adders) in the longest execution path is 1 multiplication followed by log2(N) additions, where N is the size of inverse transform.

4.5 Quantization/dequantization matrices

No quantization/dequantization matrices are needed to adjust for unequal norms. This implies that the size of the quantization/dequantization matrices is equal to 6, one entry for each value of QP%6.

## HW analysis

Following are gate count estimates for HW analysis in 45 nm LP.

- HM3.0 inverse transforms (all sizes) + inverse quantization + 4x4&8x8 IDST:			
450 MHz, 70 Kgates (suitable for 4K x 2K 60 fps decode).

- HM3.0 forward&inverse transforms (all sizes) + quantization& inverse quant. + 4x4&8x8 IDST:	
450 MHz, 92 Kgates (suitable for 4K x 2K 30 fps encode).

Table 6 shows the area, latency and throughput for the 32x32 inverse transform at various clock frequencies.

Table 6 HW analysis of the 32x32 inverse transform.

| Frequency (MHz) | Area (Kgates) | Latency | Thoughput |
| --- | --- | --- | --- |
| 100 | 99 | 1 | 1 |
| 150 | 121 | 1 | 1 |
| 180 | 135 | 1 | 1 |
| 200 | 148 | 1 | 1 |

# Dynamic range analysis and results

5.1 Clipping between inverse transform stages

The right shifts after the first and second stage of the inverse transform are chosen to prevent 16 bit overflow after the first transform stage with “reasonable” quantizers that might do rounding in the wrong direction. However, this choice of shift values does not prevent overflow for random quantizers. To support random quantizers without overflow one could use clipping between the first and the second transform stage. This would allow one bit of right shift to be moved from the first to the second transform stage as follows

- After the first stage:

y  =  (x + offset)>>6			offset = 1<<(6-1)

y = min(32767,max(-32768,y))

- After the second stage:

y  =  (x + offset)>>(13-(B-8))		offset = 1<<(12-(B-8))

The effect on the modification described above was evaluated using the common test conditions [8]. This change does not change BD-rate performance as can be seen in from Table 7 below.

Table 7 BD-rate results for clipping between inverse transform stages and moving one right shift.

| normal QP (22,27,32,37) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| High Efficiency |  |  | Low Complexity |  |  |
| AI | RA | LD | AI | RA | LD |
| 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| low QP (1,5,9,13) |  |  |  |  |  |
| High Efficiency |  |  | Low Complexity |  |  |
| AI | RA | LD | AI | RA | LD |
| 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| high QP (36,42,47,51) |  |  |  |  |  |
| High Efficiency |  |  | Low Complexity |  |  |
| AI | RA | LD | AI | RA | LD |
| 0,0 | 0,1 | 0,1 | 0,0 | 0,0 | 0,0 |

5.1 Increased bit depth

In the proposed transform the bit depth after each transform stage (forward and inverse) is equal to 16, independent of the internal bit depth of the encoding/decoding process. To evaluate the potential benefit of letting the transform bit depth scale with the internal bit depth of the encoding/decoding, we change the scaling and quantization/dequantization as follows (replacing B-8 with 0).

- After the first forward transform stage:

y  =  (x + offset)>>(M-1),		offset = (1<<(M-2))

- After the second stage, the resulting transform coefficients  are scaled as follows

y  =  (x + offset)>>(M+6),		offset = (1<<(M+5))

- Quantization(RDOQ=off):

level  =  (coeff*Q + offset)>>(21+QP/6–M)

- Dequantization:

coeffQ 	=  ((level*IQ << (QP/6)) + offset)>>(M-1),  offset = 1<<(M-2)

coeffQ = min((1<<(15+(B-8)))-1,max(-(1<<(15+(B-8)))-,coeffQ))

- After the first inverse transform stage:

y  =  (x + offset)>>7			offset = 1<<6

- After the second inverse transform stage:

y  =  (x + offset)>>12			offset = 1<<11

The resulting bit depth for all intermediate transform buffers is 18 bit and 20 bit for internal bit depths of 10 and 12 respectively. As can be seen from the results of Table 8 and Table 9, there are some small gains for low QP range. In our opinion, these gains do not justify the increased complexity by going beyond a bit depth of 16 for the transform buffers.

Table 8 BD-rate results for increasing the transform bit depth from 16 to 18, internal bit depth=10.

| normal QP (22,27,32,37) |  |  |
| --- | --- | --- |
| High Efficiency |  |  |
| AI | RA | LD |
| 0,0 | 0,0 | 0,0 |
| low QP (1,5,9,13) |  |  |
| High Efficiency |  |  |
| AI | RA | LD |
| -0,3 | -0,2 | -0,2 |
| high QP (36,42,47,51) |  |  |
| High Efficiency |  |  |
| AI | RA | LD |
| 0,0 | -0,1 | 0,1 |

Table 9 BD-rate results for increasing the transform bit depth from 16 to 20, internal bit depth=12.

| normal QP (22,27,32,37) |  |  |
| --- | --- | --- |
| High Efficiency |  |  |
| AI | RA | LD |
| 0,0 | 0,0 | 0,0 |
| low QP (1,5,9,13) |  |  |
| High Efficiency |  |  |
| AI | RA | LD |
| -0,4 | -0,2 | -0,2 |
| high QP (36,42,47,51) |  |  |
| High Efficiency |  |  |
| AI | RA | LD |
| 0,0 | 0,0 | 0,4 |

# Conclusions

Integer transforms for HEVC have been proposed. The proposed transforms have several implementation benefits compared to the existing transforms, including

- 16 bit data representation before and after each transform stage independent of IBDI settings.

- Only 16 bit multipliers are needed for internal calculations.

- No need for correction of different norms of basis vectors during quantization/dequantization.

- Reusability of small transforms in larger transforms.

- Allows both matrix multiplication and partial butterfly structure.

Complexity analysis based on arithmetic operation count and optimized software has been provided. It has been shown that the complexity can be significantly reduced by exploiting partial transforms where only the KxK lower frequency sub-block is non-zero. Also, it has been shown that the optimized inverse transforms typically contribute 0.5% - 5% to the CPU load when decoding the class B sequences at normal QP range. Finally, it has been shown that increasing the bit depth of the transpose buffer with the internal bit depth of the encoding/decoding process provides less than 0.5% BD-rate gain.

# References

- A. Fuldseth and G. Bjøntegaard, “Unified transform design for HEVC with 16 internal data representation,” JCTVC-D224, Daegu, January, 2011.

- M. Sadafale and M. Budagavi, “Matrix multiplication specification for HEVC transforms,” JCTVC-D036, Daegu, January, 2011.

- A. Fuldseth, G. Bjøntegaard, M. Sadafale, M. Budagavi, “Transform design for HEVC with 16 bit intermediate data representation,” JCTVC-E243, Geneva, March, 2011.

- P. Topiwala, M Budagavi, A. Fuldseth, R. Joshi, I.-K. Kim, “CE10 Core transform design,” JCTVC-E710, Geneva, March, 2011.

- M Budagavi, “IDCT pruning,” JCTVC-E386, Geneva, March, 2011.

- M Budagavi, V. Sze, “IDCT pruning and scan dependent transform order,” JCTVC-F236, Torino, July, 2011.

- A. Fuldseth, L. P. Endresen, S. Selnes, V. Arbatov, F. Franchetti, M. Püschel, “SIMD optimization of proposed HEVC core transforms,” JCTVC-F447, Torino, July, 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-E700, , Geneva, March, 2011.

# Patent rights declaration(s)

Cisco and Texas Instruments may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A

16x16 transform

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9}

32x32 transform

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 90 88 85 82 78 73 67 61 54 46 38 31 22 13  4 -4-13-22-31-38-46-54-61-67-73-78-82-85-88-90-90}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90-90-87-80-70-57-43-25 -9  9 25 43 57 70 80 87 90}

{90 82 67 46 22 -4-31-54-73-85-90-88-78-61-38-13 13 38 61 78 88 90 85 73 54 31  4-22-46-67-82-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89 89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{88 67 31-13-54-82-90-78-46 -4 38 73 90 85 61 22-22-61-85-90-73-38  4 46 78 90 82 54 13-31-67-88}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87-87-57 -9 43 80 90 70 25-25-70-90-80-43  9 57 87}

{85 46-13-67-90-73-22 38 82 88 54 -4-61-90-78-31 31 78 90 61  4-54-88-82-38 22 73 90 67 13-46-85}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{82 22-54-90-61 13 78 85 31-46-90-67  4 73 88 38-38-88-73 -4 67 90 46-31-85-78-13 61 90 54-22-82}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80-80 -9 70 87 25-57-90-43 43 90 57-25-87-70  9 80}

{78 -4-82-73 13 85 67-22-88-61 31 90 54-38-90-46 46 90 38-54-90-31 61 88 22-67-85-13 73 82  4-78}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75 75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{73-31-90-22 78 67-38-90-13 82 61-46-88 -4 85 54-54-85  4 88 46-61-82 13 90 38-67-78 22 90 31-73}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70-70 43 87 -9-90-25 80 57-57-80 25 90  9-87-43 70}

{67-54-78 38 85-22-90  4 90 13-88-31 82 46-73-61 61 73-46-82 31 88-13-90 -4 90 22-85-38 78 54-67}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{61-73-46 82 31-88-13 90 -4-90 22 85-38-78 54 67-67-54 78 38-85-22 90  4-90 13 88-31-82 46 73-61}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57-57 80 25-90  9 87-43-70 70 43-87 -9 90-25-80 57}

{54-85 -4 88-46-61 82 13-90 38 67-78-22 90-31-73 73 31-90 22 78-67-38 90-13-82 61 46-88  4 85-54}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50 50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{46-90 38 54-90 31 61-88 22 67-85 13 73-82  4 78-78 -4 82-73-13 85-67-22 88-61-31 90-54-38 90-46}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43-43 90-57-25 87-70 -9 80-80  9 70-87 25 57-90 43}

{38-88 73 -4-67 90-46-31 85-78 13 61-90 54 22-82 82-22-54 90-61-13 78-85 31 46-90 67  4-73 88-38}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{31-78 90-61  4 54-88 82-38-22 73-90 67-13-46 85-85 46 13-67 90-73 22 38-82 88-54 -4 61-90 78-31}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25-25 70-90 80-43 -9 57-87 87-57  9 43-80 90-70 25}

{22-61 85-90 73-38 -4 46-78 90-82 54-13-31 67-88 88-67 31 13-54 82-90 78-46  4 38-73 90-85 61-22}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18 18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{13-38 61-78 88-90 85-73 54-31  4 22-46 67-82 90-90 82-67 46-22 -4 31-54 73-85 90-88 78-61 38-13}

{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9 -9 25-43 57-70 80-87 90-90 87-80 70-57 43-25  9}
{ 4-13 22-31 38-46 54-61 67-73 78-82 85-88 90-90 90-90 88-85 82-78 73-67 61-54 46-38 31-22 13 -4}

Appendix B

This annex provides proposed text changes to the working draft. Only paragraphs that need to be changed are included in this appendix.

### 8.5.3 Scaling process for transform coefficients

The scaled transform coefficient array dij is derived as follows.

shift = log2(nS) – 1 + trafoPrecisionExt

dij = Clip3(-32768,32767,(( cij * LevelScale[ qP%6 ] << ( qP/6 )) + (1<<(shift-1))>>shift), with i, j = 0..nS-1

### 8.5.4 Transformation process for scaled transform coefficients

The constructed residual samples are derived as specified in the following ordered steps.

- Each (horizontal) row of scaled transform coefficients dij (i, j=0..nS-1) is transformed to eij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause  according to the size of the transform unit nS, with the (nS)x(nS) array d as the input and the output is the (nS)x(nS) array e.

- The intermediate sample values gij (i,j=0,..nS-1) are derived by

gij = (eij + 64)>>7

- Each (vertical) column of the resulting matrix eij (i, j=0..nS-1) is transformed to fij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause  according to the size of the transform unit nS, with the (nS)x(nS) array e as the input and the output is the (nS)x(nS) array f.

- The residual sample value rij is derived by

rij = ( fij + 211-trafoPrecisionExt ) >> ( 12 - trafoPrecisionExt ), with i, j=0.. (nS)x(nS)-1

#### 8.5.4.1 Transformation process

Inputs of this process are:

- a variable nS specifying the size of the current transform unit,

- an array of nS samples of the scaled transform coefficients x with elements xi, with i = 0..nS-1.

Output of this process is an array of nS samples of the residual samples y with elements yi, with i = 0..nS.

The nS-point one-dimensional inverse transformation process is defined as following:

yi 	=	ci0*x0 + ci1*x1 + … + cinS-1*xnS-1 with i = 0,..,nS-1

Estimate based on 8x8 results