| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F452 |
| --- | --- |

| Title: | Crosscheck - CABAC with Constrained Outstanding Bits (F059) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Nguyen Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results from Fraunhofer HHI for the proposal F059: “CABAC with Constrained Outstanding Bits” with a threshold of 12. The software was analyzed and no mismatch was found compared to the results provided by MediaTek.

# Tested Proposal

The tested proposal has been integrated into HEVC HM 3.0. F059 proposes to limit the number of outstanding bits of the CABAC entropy coder. Modification was found in encoder and decoder. At the encoding stage, the number of outstanding bits is compared against the limit after the coding of each bin. If the limit is exceeded, the outstanding bits are written into the bitstream. On the decoding stage, an outstanding bit counter is introduced. If the limit is exceeded, the decoder knows that outstanding bits are following. This concept limits the theoretical numbers of outstanding bits, which is suitable for hardware design. However, it introduces additional delay and complexity in terms of memory and multiplexer. Further analysis on total benefits for hardware is recommended.

# Simulation Environment

The code base for this experiment is the HM 3.0 software. Only high efficiency configuration is verified.

Due to limited resources in the simulation environment, the runtime is heavily inaccurate and should not be used to infer the complexity of the proposed method.

# Results