| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F453 |
| --- | --- |

| Title: | Crosscheck - CABAC with a Reduced LPS Range Table (F061) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Nguyen Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results from Fraunhofer HHI for the proposal F061: “CABAC with a Reduced LPS Range Table”. The software was analyzed and no mismatch was found compared to the results provided by MediaTek.

# Tested Proposal

The tested proposal has been integrated into HEVC HM 3.0. F061 proposes to reduce the LPS range table and increase the quantization granularity. Thus, the table is reduced by 1 column and the segment index, e.g. the possible quantization level, is increased by 4 to 8. The first and the second column contain the value for the LPS range of segment index 1 and 5. The last column contains a refinement index, which is an offset to add or subtract to the value of segment index 1 or 5 if the calculated segment index is not equal to 1 or 5. For hardware design, this concept introduces additional logic which cannot be processed in parallel. Hence, it is expected that the critical path will be increased. Further, the software implementation includes a multiplication operator, which is maybe avoidable. Finally, the impact from the finer quantization steps tends is small.

# Simulation Environment

The code base for this experiment is the HM 3.0 software. Only high efficiency configuration is verified.

Due to limited resources in the simulation environment, the runtime is heavily inaccurate and should not be used to infer the complexity of the proposed method.

# Results