| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F455 |
| --- | --- |

| Title: | Modified binarization and coding of MVD for PIPE/CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Nguyen D. Marpe H. Schwarz T. Wiegand Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This proposal describes a modified binarization scheme and as the result a modified coding scheme of MVD for PIPE/CABAC. Compared to the coding scheme for MVD of the current HEVC design, only 1 bin is coded with an adaptive context model, while up to 9 bins are coded with adaptive context models in the current HEVC design. The proposed method leads to a reduction of the numbers of bins coded with regular context models and increases the number of bins coded with equal probability. Also, the proposed scheme reduces the number of context models from 14 to 6. Further, an alternative configuration is presented where the number of context models is reduced to 3 and where the line buffer for MVD is not necessary anymore.

In contrast to the proposed scheme, where the evaluation of neighbours is required, JCTVC-F133, F429, F497, and F606 propose to remove both neighbours for the context modelling of the first bin without loss in coding efficiency. For that, we propose a combination of both ideas by maintaining the coding efficiency of the current HM.

# Coding of MVD in HEVC

In the current design of HEVC, the coding of MVD is similar to H.264/AVC. The integer valued MVD is separated into an absolute valued MVD part and the sign information part. While the sign information is coded in the bypass mode of CABAC, a binarization scheme based on a concatenation of truncated unary and Exp-Golomb code is used for the coding of the absolute valued MVD. The resulting bin string of the binarization process consists of a prefix part and a suffix part. The prefix part of the bin string (e.g. truncated unary part) is coded using context models, where up to 7 context models are used for each MVD component (e.g. horizontal and vertical). If the absolute valued MVD is greater than 9, e.g. the resulting bit string of the prefix part contains only ‘1’, the bin string consists of a suffix part and the resulting bins from the binarization of the difference between the absolute valued MVD and 9 is coded in the bypass mode of CABAC using a 3rd order Exp-Golomb code for the binarization process.

The context modelling for the prefix part of the bin string depends on the bin index. For the first bin of the bin string, the left and the above neighbour are evaluated and one of the first three available context models is selected. For that, the absolute MVD of the left neighbour is compared against 16. If the absolute MVD of the left neighbour is greater than 16, the ctxIdxInc is incremented by 1. Next, the absolute MVD of the above neighbour is compared against the same threshold and ctxIdxInc is also incremented by 1 if the absolute MVD is greater than the threshold. For the remaining bins of the bin string, a context model is selected depending on the index of the bin inside the bin string.

# Proposed coding scheme

The proposed method reduces the number of bins coded with regular context models by decrease the cut-off value from 9 to 1. This means that only 1 bin is coded using regular context models while the bins of the suffix part of the binarization process are coded in the bypass mode of CABAC. The context model selection for the first bin is kept as in the current design as described in [1]. The main difference is lying in the binarization process of the suffix part of the bin string. In contrast to the current design, where the order of the Exp-Golomb binarization is fixed and is equal to 3, the order of the Exp-Golomb code for the binarization process of the proposed scheme depends on the selected context model index for the first bin.

The parameter selection process is done as follows. For the first bin of the bin string, one of the first three context models are used with ctxIdxInc ranged from 0 to 2. This is identical to the current design in HEVC and is not modified by the proposed scheme. Due to the reduction of the cut-off value of the binarization process, the remaning absolute value, e.g. the absolute valued MVD minus 1 is binarized by Exp-Golomb code and the resulting bins are coded in a bypass mode. Instead of using a fixed parameter for the binarization process of the suffix part, the ctxIdxInc is used as input parameter for the binarization process of the suffix part. This means that the possible order of the Exp-Golomb binarization is 0, 1 or 2. This concept allows the reduction of bins coded with context models and increases the number of bins coded in the bypass mode of CABAC. In additional, the number of context models is decreased from 14 to 6.

In a further configuration, the evaluation of the above neighbour is disabled and only the left neighbour is taken into the evaluation process. For this alternative configuration, the thresholds are modified to 2 and 16. It means that the ctxIdxInc is equal to 0 if the absolute MVD of the left neighbour is smaller than 3, 1 if the absolute MVD of the left neighbour is between 3 and 16, and 2 otherwise. Furthermore, the horizontal and vertical component shares the same context models. For this configuration, the number of context models can be further reduced from 6 to 3.

However, this concept of reducing context models by increase the number of bypass bins is not compatible with the proposals JCTVC-F133, F429, F497, and F606. In order to combine both ideas, the scheme is modified in a way that no adaptive parameter selection for the Exp-Golomb part is made. In that scheme, the first bin is coded with a single context and the second bin is coded with a single context. Again, both components share the same contexts. In total, only 2 contexts are required for the coding of the MVD. The remaining value (e.g. absolute MVD minus 2) is coded in bypass mode using Exp-Golomb binarization with fixed order of 1.

# Simulation Environment

The code base for this experiment is the HM 3.2 software. Only high efficiency configuration is verified.

Due to limited resources in the simulation environment, the runtime is heavily inaccurate and should not be used to infer the complexity of the proposed method.

# Results

Table 1: No modifications to the context selection for the first bin.

Table 2: Only left neighbour and shared context models for components.

Table 3: Compatible scheme with other proposals without neighbour evaluation

# Conclusion

The alternative configuration reduces the number of context models from 14 to 2 while same performance is achieved compared to the current anchor. Thus, the alternative configuration is proposed to adopt into the next HEVC test model.

# Working draft changes

The specification text is for the proposed configuration only.

| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| mvd_l0[ ][ ][ 0 ] | P | prefix and suffix as given by UEG1 with signedValFlag=1, uCoff=2 | prefix: 1 suffix: na | prefix: Table … suffix: na | prefix: 0 suffix: na, (uses Decode Bypass) |
| mvd_l0[ ][ ][ 1 ] | P |  |  |  |  |
| mvd_l0[ ][ ][ 0 ], mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | B |  |  |  |  |
| mvd_l0[ ][ ][ 1 ], mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | B |  |  |  |  |

| Initialisation variables | mvd_lc, mvd_l0, mvd_l1 ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| m | 0 | 0 | na |
| n | 64 | 64 | na |

| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| mvd_l0[ ][ ][ 0 ] | Error: Reference source not found | 0 | 0 | 1 | na | na | na |
| mvd_l0[ ][ ][ 1 ] | Error: Reference source not found | 2 | 0 | 1 | na | na | na |
| mvd_l0[ ][ ][ 0 ], mvd_lc[ ][ ][ 0 ], mvd_l1[ ][ ][ 0 ] | Error: Reference source not found | 4 | 0 | 1 | na | na | na |
| mvd_l0[ ][ ][ 1 ], mvd_lc[ ][ ][ 1 ], mvd_l1[ ][ ][ 1 ] | Error: Reference source not found | 6 | 0 | 1 | na | na | na |

# References

- V. Sze, A. Chandrakasan, “Joint Algorithm-Architecture Optimization of CABAC”, JCTVC-E324, Geneva, CH, March 2011.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).