| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F456 |
| --- | --- |

| Title: | Directional intra prediction smoothing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Email: | jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

HM3 currently smooths the DC prediction signal by applying 2-tap and 3-tap filters to the border samples of the intra prediction block. This contribution presents a Directional Intra Prediction Smoothing (DIPS) technique with similar intent to be applied to the directionally predicted intra blocks. A directional 2-tap filter is applied to either top row or left column of the intra prediction block depending on the directionality of the intra sample prediction. The coding results reported in this contribution indicate -0.4 % and -0.4 % BD bitrate impact in Intra HE and Intra LC configurations, respectively. The contribution also reports performance of DIPS when combined with JCTVC-F172 (Improved intra horizontal and vertical prediction by Mitsubishi). In that case BD bitrate impact of -0.6 % and -0.6 % for Intra HE and Intra LC configurations are reported, respectively.

# Introduction

Similarly to the DC prediction, also the directional intra prediction process generates sometimes discontinuities between the prediction signal and reconstructed samples of the neighboring blocks. Figure 1 below illustrates this behavior for an 8x8 block predicted diagonally from above. Top samples of the predicted block have strong correlation with the reconstructed samples above as the predicted samples are derived from those reference samples. However, there may be discontinuities on the left boundary as the predicted border samples are generated from different set of samples.

Figure 1: Example of a discontinuity generated by HM3 directional intra prediction process.

# Proposed solution

The proposed technique utilizes a 2-tap filter on the side of the prediction blocks where directional intra prediction process sometimes generates a discontinuity. That is, the filter is applied on the border which is not the main reference for the directional intra prediction (it is applied for the left edge of the block in the case of vertical prediction and top edge in the case of the horizontal prediction). Figure 2 illustrates the process for diagonal prediction from the top-right direction.

Figure 2: Example of the filtering process in the case of prediction from the top-right direction and from directly above (filtered pixels are located inside the dashed box).

The filter is applied to only one column or one row of predicted samples within one directionally predicted intra block. The input to the 2-tap filter consists of the sample to be filtered itself and one sample across the neighboring edge in a direction defined by the directionality of the sample prediction process. The strength of the filter is also decided based on the directionality of the intra prediction. Table 1 below defines the filter taps to be used and the location of the second input sample to the filtering process. As it can be seen from Table 1 and equations below, the filtering is done in diagonal direction when the angularity of the prediction is large (Fd equals to 1 or -1 when absolute value of the intraPredAngle is larger than 9) and in lateral direction otherwise (Fd equals to 0).

Table 1: Filter strength and directionality as a function of intra prediction angle

| intraPredAngle | 1st filter tap (8 - Td) | 2nd filter tap (Td) | Filter offset (Fd) |
| --- | --- | --- | --- |
| Smaller than -9 | 6 | 2 | -1 |
| From -9 to 9 | 7 | 1 | 0 |
| Larger than 9 | 6 | 2 | 1 |

For vertical prediction modes the filtering is performed by using the parameters given in Table 1 as follows:

P ( 0, y ) = ( ( 8 – Td ) • p ( 0, y ) + Td • p ( -1, y + Fd ) + 4 ) >> 3,       y = 0, …, N-1

Similarly for horizontal modes the filtering is performed as follows

P ( x, 0 ) = ( ( 8 – Td ) • p ( x, 0 ) + Td • p ( x + Fd, -1) + 4 ) >> 3,        x = 0, …, N-1

Where P denotes the output samples of the filtering process and p the input samples to the filtering process (origin being at the top-left corner of the block).

# Experimental results

The table below summarizes the coding efficiency results in HM 3.0 environment following the JCT-VC common test conditions for Intra HE and Intra LC configurations. Table 2 and 3 presents results for the DIPS technique when applied to both luma and chroma components, or only to luma component, respectively.

Table 2: DIPS for both luma and chroma

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | 0.1 | -0.1 | -0.5 | -0.2 | -0.4 |
| Class B | -0.4 | 0.4 | 0.4 | -0.3 | 0.1 | 0.1 |
| Class C | -0.3 | 0.3 | 0.4 | -0.3 | -0.1 | 0.0 |
| Class D | -0.4 | 0.2 | 0.3 | -0.4 | -0.1 | -0.1 |
| Class E | -0.4 | 0.5 | 0.7 | -0.4 | 0.0 | 0.1 |
| Overall | -0.4 | 0.3 | 0.3 | -0.4 | 0.0 | -0.1 |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 100 % |  |  | 100 % |  |  |

Table 3: DIPS for luma only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5 | -0.4 | -0.5 | -0.5 | -0.6 | -0.6 |
| Class B | -0.3 | -0.3 | -0.2 | -0.3 | -0.4 | -0.4 |
| Class C | -0.2 | -0.4 | -0.3 | -0.2 | -0.3 | -0.4 |
| Class D | -0.3 | -0.3 | -0.4 | -0.3 | -0.4 | -0.3 |
| Class E | -0.3 | -0.4 | -0.5 | -0.3 | -0.3 | -0.5 |
| Overall | -0.3 | -0.4 | -0.4 | -0.3 | -0.4 | -0.4 |
| Enc Time[%] | 101 % |  |  | 101 % |  |  |
| Dec Time[%] | 100 % |  |  | 101 % |  |  |

# Source code

Modified HM 3.0 source code files implementing the feature are provided in the attached zip file (with #define DIR_INTRA_PRED_FILTER 1). It should be pointed out that the implementation requires only 5 lines of code to be added to a single function. The feature is implemented by adding following operation to the TComPrediction::xPredIntraAng function:

Int iFiltTap = abs(intraPredAngle) <= 9 ? 1 : 2;

Int iFiltOff = abs(intraPredAngle) <= 9 ? 1 : intraPredAngle > 0 ? 2 : 0;

for (k=0;k<blkSize;k++)

pDst[k*dstStride] = ( (8 - iFiltTap)*pDst   [k*dstStride] +
                                 iFiltTap *refSide[k+iFiltOff]  + 4 ) >> 3;

# Proposed combination with JCTVC-F172

In this section we propose a solution combining DIPS approach presented in the above sections with the JCTVC-F172 (Improved intra horizontal and vertical prediction) by Mitsubishi. In the combined solution DIPS is applied to the directions for which the absolute value of intraPredAngle is larger than nine, and JCTVC-F172 is applied when intra prediction mode indicates horizontal or vertical prediction (intraPredAngle equals 0). The code implementing this can be written as follows (source files attached):

if ( intraPredAngle == 0 && blkSize < 32 )

{

for (k=0;k<blkSize;k++)

pDst[k*dstStride] = Clip( pDst[k*dstStride] + ( refSide[k+1] - refSide[0] ) / 2);

}

else if ( abs ( intraPredAngle ) > 9 )

{

Int iFiltOff = intraPredAngle > 0 ? 2 : 0;

for (k=0;k<blkSize;k++)

pDst[k*dstStride] = ( 3*pDst[k*dstStride] + refSide[k+iFiltOff] + 2 ) >> 2;

}

Tables 4 and 5 below present results for the proposed combined solution when applied to both luma and chroma components, or only to luma component, respectively.

Table 4: Combined method both luma and chroma

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | 0.0 | -0.1 | -0.5 | -0.3 | -0.4 |
| Class B | -0.5 | 0.1 | 0.2 | -0.5 | -0.2 | -0.1 |
| Class C | -0.6 | 0.0 | 0.1 | -0.6 | -0.3 | -0.2 |
| Class D | -0.7 | -0.1 | 0.1 | -0.7 | -0.3 | -0.3 |
| Class E | -0.6 | -0.1 | 0.1 | -0.6 | -0.1 | -0.2 |
| Overall | -0.6 | 0.0 | 0.1 | -0.6 | -0.2 | -0.2 |
| Enc Time[%] | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 100 % |  |  | 101 % |  |  |

Table 5: Combined method for luma only

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | -0.5 | -0.4 | -0.5 | -0.6 | -0.6 |
| Class B | -0.4 | -0.3 | -0.3 | -0.4 | -0.4 | -0.4 |
| Class C | -0.5 | -0.5 | -0.4 | -0.5 | -0.5 | -0.5 |
| Class D | -0.5 | -0.5 | -0.5 | -0.6 | -0.5 | -0.5 |
| Class E | -0.5 | -0.5 | -0.6 | -0.5 | -0.5 | -0.6 |
| Overall | -0.5 | -0.5 | -0.4 | -0.5 | -0.5 | -0.5 |
| Enc Time[%] | 100 % |  |  | 101 % |  |  |
| Dec Time[%] | 100 % |  |  | 101 % |  |  |

# Conclusion

This document and JCTVC-F172 present two low complexity techniques to improve Intra prediction quality in HM environment. We believe the combination described in section 5 provides a very desirable complexity – coding efficiency trade-off and we propose it to be adopted in HM4 and WD4 of HEVC.

# Proposed text for the Working Draft

Text relating to the Section 5 of this contribution

The following changes are proposed to the section 8.3.3.1.6 of the HEVC Working Draft (with respect to version 8 of the WD3). Added text is marked in red font. The text provided here is in addition to the text provided in JCTVC-F172.

Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the preliminary prediction samples predSamplesPre[ x, y ] is derived by

predSamplesPre[ x, y ] = ( ( 32 – iFact )*refMain[ x+iIdx+1 ] + iFact*refMain[ x+iIdx+2] + 16 ) >> 5

- Otherwise, the value of the preliminary prediction samples predSamplesPre[ x, y ] is derived by

predSamplesPre[ x, y ] = refMain[ x+iIdx+1 ]

Table 8-10 – Specification of filter offset Fd

| intraPredAngle | Fd |
| --- | --- |
| Smaller than -9 | -1 |
| Larger than 9 | 1 |

The values of the final prediction samples predSamples[ x, y ] are derived as follows.

–	If the absolute value of intraPredAngle is less than or equal to 9,

predSamples[ x, y ] = predSamplesPre[ x, y ]

–	Otherwise, if intraPredOrder is less than 18 (vertical prediction),

- If x equals to 0,

predSamples[ 0, y ] = ( 3 * predSamplesPre[ 0, y ]  + p[ -1, y + Fd ] + 2 ) >> 2

- Otherwise,

predSamples[ x, y ] = predSamplesPre[ x, y ]

Otherwise (horizontal prediction),

- If y equals to 0,

predSamples[ x, 0 ] = ( 3 * predSamplesPre[ x, 0 ]  + p[ x + Fd, -1 ] + 2 ) >> 2

- Otherwise,

predSamples[ x, y ] = predSamplesPre[ x, y ]

Text relating to the Section 2 of this contribution

The following changes are proposed to the section 8.3.3.1.6 of the HEVC Working Draft (with respect to version 8 of the WD3). Added text is marked in red font. It is further proposed to merge the definitions of horizontal and vertical prediction modes with intraPredAngle equal to 0 in the section 8.3.3.1.6 in order to apply this text to all directional intra prediction modes.

Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the preliminary prediction samples predSamplesPre[ x, y ] is derived by

predSamplesPre[ x, y ] = ( ( 32 – iFact )*refMain[ x+iIdx+1 ] + iFact*refMain[ x+iIdx+2] + 16 ) >> 5

- Otherwise, the value of the preliminary prediction samples predSamplesPre[ x, y ] is derived by

predSamplesPre[ x, y ] = refMain[ x+iIdx+1 ]

Table 8-10 – Specification of filter tap Td and filter offset Fd

| intraPredAngle | Td | Fd |
| --- | --- | --- |
| Smaller than -9 | 2 | -1 |
| From -9 to 9 | 1 | 0 |
| Larger than 9 | 2 | 1 |

The values of the final prediction samples predSamples[ x, y ] are derived as follows.

–	If intraPredOrder is less than 18 (vertical prediction),

- If x equals to 0,

predSamples[ 0, y ] = ( ( 8 – Td )* predSamplesPre[ 0, y ]  + Td*p[ -1, y + Fd ] + 4 ) >> 3

- Otherwise,

predSamples[ x, y ] = predSamplesPre[ x, y ]

Otherwise (horizontal prediction),

- If y equals to 0,

predSamples[ x, 0 ] = ( ( 8 – Td )* predSamplesPre[ x, 0 ]  + Td*p[ x + Fd, -1 ] + 4 ) >> 3

- Otherwise,

predSamples[ x, y ] = predSamplesPre[ x, y ]

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).