| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F459 |
| --- | --- |

| Title: | Parsing friendly intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien Xianglin Wang Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: +1-858-845-1795 Email: wchien@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents a coding method for intra prediction mode. In the current HEVC Test Model, different codeword binerizations are defined based on the intra prediction modes of neighboring partitions for the luma intra prediction mode coding. It also uses different codeword binarization for chroma intra prediction mode based on the luma intra prediction mode of the current partition. This method unifies different binarizations by utilizing constant number of candidate modes. The simulation results show average performance gain of 0.3% and 0.2% in Intra only configurations.

# Introduction

In the current HEVC Test Model (HM 3.0), the parsing of luma intra prediction mode (intra_pred_mode) depends on intra_pred_mode of the neighboring blocks A and B, as shown in Figure 1. In the current Working Draft [2], candidate mode list (candModeList) and number of candidate modes (NumMPMC) are derived from the intra_pred_mode of the neighboring blocks A and B (candIntraPredModeA and candIntraPredModeB) as follows:

- If both candIntraPredModeN are not available, then the value 2 is assigned to candModeList[ 0 ] and NumMPMC and is set equal to 1

- Otherwise, if only one candIntraPredModeN is available or if both candIntraPredModeN are the same, then this candIntraPredModeN is assigned to candModeList[ 0 ] and NumMPMCand is set equal to 1

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ] and NumMPMC and is set equal to 2.

The parsing throughput is affected due to necessity of prior knowledge of  intra prediction modes of  neighboring blocks A and B (candIntraPredModeA and candIntraPredModeB). Furthermore, it requires extra memory storage during the parsing process to store intra_pred_mode (6 bits) for every 4x4 block.

B

A

Figure 1 intra_pred_mode for left and top blocks

On the other hand, the parsing process of the chroma intra prediction mode relies on the decoded luma prediction intra mode. In the current design, the chroma intra prediction modes contain horizontal prediction, vertical prediction, DC prediction, Planar prediction, luma signal prediction, and reuse of the luma intra prediction mode. When luma intra prediction mode is either horizontal prediction, vertical prediction, or DC prediction, the duplication is removed by excluding the associated prediction mode from the codeword.

# Proposed Algorithm

In this proposal, the number of candidate modes (NumMPMC) is a constant, i.e. NumMPMC does not change whether candIntraPredModeA and candIntraPredModeB are different or not.  Also, the number of  available chroma intra prediction modes is also constant.  With the proposed algorithm, the parsing process can be performed without accessing the neighboring intra prediction modes. Note that Planar mode is given as mode 34.

3.1 Luma intra prediction mode

In luma intra prediction, the proposed algorithm uses four most probable candidate modes (MPMC), i.e. NumMPMC=4.  The derivation of MPMC is as follows:

- If both candIntraPredModeN are not available, then the value 2 is assigned to candModeList[ 0 ],

the value 0 is assigned to candModeList[ 1], the value 1 is assigned to candModeList[ 2 ] and the value 34 is assigned to candModeList[ 3 ],

- Otherwise, if both candIntraPredModeN are the same, then this candIntraPredModeN is assigned to candModeList[ 0 ]. candModeList[ 1 ], candModeList[ 2 ] and candModeList[ 3 ] are derived by applying the following procedure with Table 3.1, 3.2, and 3.3.

  - PredModeMinus1_M[candModeList[ 0 ]] is assigned to candModeList[ 2], PredModePlus1_M[candModeList[ 0 ]] is assigned to   candModeList[ 3], where M represents intraPredModeNum.

  - If candModeList[ 0 ] is equal to 34, the value 2 is assigned to candModeList[ 1 ]

  - Otherwise, the value 34 is assigned to candModeList[ 1 ]

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ]. candModeList[ 2 ] and candModeList[ 3 ] is derived by applying the following procedure.

  - If intraPredModeNum is equal to 4, candModeList[ 2 ] and candModeList[ 3 ]  is derived from Table 3.4 by using candModeList[ 0 ] and candModeList[ 1 ] as inputs.

  - Otherwise, if candModeList[ 0 ] is equal to 34, PredModeMinus1_M[candModeList[ 1 ]] is assigned to candModeList[ 2], PredModePlus1_M[candModeList[ 1 ]] is assigned to candModeList[ 3], where M represents intraPredModeNum.

  - Otherwise, if candModeList[ 1 ] is equal to 34, PredModeMinus1_M[candModeList[ 0 ]] is assigned to candModeList[ 2], PredModePlus1_M[candModeList[ 0 ]] is assigned to candModeList[ 3], where M represents intraPredModeNum.

  - Otherwise, 34 is assigned to candModeList[ 3] and candModeList[ 1 ] is derived by applying the following procedure, where M represents intraPredModeNum.

  - If candModeList[ 1 ] is not equal to PredModeMinus1_M[candModeList[ 0 ]],  PredModeMinus1_M[candModeList[ 0 ]] is assigned to candModeList[ 2].

  - Otherwise, if candModeList[ 0 ] is not equal to PredModeMinus1_M[candModeList[ 1 ]],  PredModeMinus1_M[candModeList[ 1 ]] is assigned to candModeList[ 2].

  - Otherwise,  2 is assigned to candModeList[ 2],

  - Table 3.1 Candidate mode mapping when intraPredModeNum is equal to 4

| intraPredMode | 0 | 1 | 2 |
| --- | --- | --- | --- |
| PreModeMinus1_4 | 1 | 0 | 0 |
| PreModePlus1_4 | 2 | 2 | 1 |

Table 3.2 Candidate mode mapping when intraPredModeNum is equal to 18

| intraPredMode_index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredModeMinus1_18 | 11 | 16 | 0 | 14 | 10 | 12 | 13 | 15 | 9 | 6 | 3 | 4 | 0 | 5 | 7 | 1 | 8 |
| PredModePlus1_18 | 12 | 15 | 1 | 10 | 11 | 13 | 9 | 14 | 16 | 8 | 4 | 0 | 5 | 6 | 3 | 7 | 1 |

Table 3.3 Candidate mode mapping when intraPredModeNum is equal to 35

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredModeMinus1_35 | 21 | 30 | 0 | 26 | 19 | 23 | 25 | 28 | 32 | 6 | 18 | 21 | 22 | 24 | 27 | 29 | 31 | 33 |
| PredModePlus1_35 | 22 | 29 | 1 | 18 | 20 | 24 | 9 | 27 | 31 | 33 | 19 | 20 | 23 | 25 | 26 | 28 | 30 | 32 |

| intraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredModeMinus1_35 | 3 | 10 | 4 | 11 | 0 | 12 | 5 | 13 | 14 | 7 | 15 | 1 | 16 | 8 | 17 | 9 | 0 |
| PredModePlus1_35 | 10 | 4 | 11 | 0 | 12 | 5 | 13 | 6 | 3 | 14 | 7 | 15 | 1 | 16 | 8 | 17 | 1 |

Table 3.4 Candidate mode mapping when intraPredModeNum is equal to 4

| candModeList[ 0 ] | 0 | 0 | 0 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| candModeList[ 1 ] | 1 | 2 | 34 | 2 | 34 | 34 |
| candModeList[ 2 ] | 2 | 1 | 1 | 0 | 0 | 0 |
| candModeList[ 3 ] | 34 | 34 | 2 | 34 | 2 | 1 |

Note that Tables 3.1, 3.2 and 3.3 represents the neighboring luma intra prediction modes for a given MPMC and Planar mode is given as mode 34.

The coding of the syntax element rem_intra_luma_pred_mode is also modified. While calculating rem_intra_luma_pred_mode, the Planar mode is treated as mode 0 and the other modes are treated as intra_pred_mode +1. For CABAC, each bin is coded with a context. For VLC, two sets of tables for the coding rem_intra_luma_pred_mode are merged into one set of tables, since the number of the candidate modes is now a constant.

3.2 Chroma intra prediction mode

In the proposed method, there are six available chroma intra prediction modes (IntraPredModeC) that can be signaled. Two modes, luma signal prediction and reuse of the luma intra prediction mode remain unchanged. For the other four modes, two choices are added, which are the adjacent intra prediction modes (Table 3.2 and Table 3.3) and the perpendicularly adjacent intra prediction modes (Table 3.4 and Table 3.5).  The other four modes are derived as follow:

-   If intraPredModeNum is equal to 4, 0 is assigned to intra_chroma_pred_mode[0], 1 is assigned to   intra_chroma_pred_mode[1], 2 is assigned to intra_chroma_pred_mode[2] and 34 is assigned to   intra_chroma_pred_mode[3].

-   If intra_pred_mode is equal to 2, 0 is assigned to intra_chroma_pred_mode[0], 1 is assigned to   intra_chroma_pred_mode[1], 6 is assigned to intra_chroma_pred_mode[2] and 34 is assigned to   intra_chroma_pred_mode[3].

-  Otherwise, if intra_pred_mode is equal to 34, 0 is assigned to intra_chroma_pred_mode[0], 1 is assigned to intra_chroma_pred_mode[1], 2 is assigned to intra_chroma_pred_mode[2] and 6 is assigned to   intra_chroma_pred_mode[3].

- Otherwise, if intraPredModeNum is equal to 18 or 35, intra_chroma_pred_mode [ 0 ]. intra_chroma_pred_mode [ 1 ] and intra_chroma_pred_mode [ 2 ] are derived by applying the following procedure with Table 3.2, 3.3, and 3.5.

- PredModeMinus1_M[intra_pred_mode] is assigned to intra_chroma_pred_mode[0], PredModePlus1_M[intra_pred_mode] is assigned to intra_chroma_pred_mode[1], PredMode_P[intra_pred_mode] is assigned to intra_chroma_pred_mode[2] and 34 is assigned to   intra_chroma_pred_mode[3], where M represents intraPredModeNum

Table 3.5 Perpendicular mode mapping

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredMode_P | 1 | 0 | 0 | 6 | 8 | 7 | 3 | 5 | 4 | 3 | 8 | 16 | 15 | 14 | 13 | 12 | 11 | 10 |

| intraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredMode_P | 33 | 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 0 |

# Experimental results

The proposed method is implemented on HM 3.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM3.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate. Table 3.1 summarizes the experimental results. The results shows there is 0.3% and 0.2% BD-Rate reduction in Intra and Intra low complexity configurations without encoding and decoding time changes.

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | 0.0 | 0.1 | -0.2 | 0.0 | -0.1 |
| Class B | -0.3 | -0.3 | -0.2 | -0.2 | -0.4 | -0.3 |
| Class C | -0.3 | -0.5 | -0.5 | -0.3 | -0.5 | -0.5 |
| Class D | -0.4 | -0.4 | -0.5 | -0.3 | -0.4 | -0.6 |
| Class E | -0.4 | -0.1 | -0.1 | -0.2 | -0.5 | -0.4 |
| All | -0.33 | -0.3 | -0.3 | -0.22 | -0.4 | -0.4 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3.1, BD-rate saving of the proposed algorithm relative to HM3.0

# Conclusions

This contribution proposed a coding method for intra prediction mode to address the issues in the parsing process. During the parsing process, the proposed method does not rely on the neighboring information, so extra memory storage is not needed. The simulation results show the average performance gain of 0.3% and 0.2% in Intra only configurations.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1]  D F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011.

[2]	T. Wiegand, B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “WD3: Working Draft 3 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-E603, Geneva, April 2011.