### 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| mpm_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if (mpm_idx[ x0 ][ y0 ]>3) |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag ) |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### 7.4.7 Prediction unit semantics

mvp_idx_l0[ x0 ][ y0 ] specifies the motion vector predictor index of list 0 where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When mvp_idx_l0[ x0 ][ y0 ] is not present, it shall be inferred to be equal to 0.

mvp_idx_l1[ x0 ][ y0 ] has the same semantics as mvp_idx_l0, with l0 and list 0 replaced by l1 and list 1, respectively.

mvp_idx_lc[ x0 ][ y0 ] has the same semantics as mvp_idx_l0, with l0 and list 0 replaced by lc and list combination, respectively.

mpm_idx[ x0 ][ y0 ] and rem_intra_luma_pred_mode[ x0 ][ y0 ] specify the intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When mpm_idx[ x0 ][ y0 ] is less than 4, the intra prediction mode is inferred from a neighbouring intra-predicted prediction unit according to subclause 8.3.1.

The variable IntraLumaModeBins specifies the maximum number of bins indicating the rem_intra_luma_pred_mode[ x0 ][ y0 ]. IntraLumaModeBins depends on the size of the prediction unit and limits the number of available prediction modes as given by Table 7-.

Table 7-11 – Number of bins and number of mode for rem_intra_luma_pred_mode associated to PuSize

| PuSize | IntraLumaModeBins | Number of Modes |
| --- | --- | --- |
| PU_4x4 | 4 | 18 |
| PU_8x8 | 5 | 35 |
| PU_16x16 | 5 | 35 |
| PU_32x32 | 5 | 35 |
| PU_64x64 | 0 | 4 |

### Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2TrafoSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

Table 8-1 specifies the number of luma intra prediction modes intraPredModeNum depending on log2TrafoSize.

Table 8-1 – Specification of intraPredModeNum

| log2TrafoSize | intraPredModeNum |
| --- | --- |
| 2 | 18 |
| 3 | 35 |
| 4 | 35 |
| 5 | 35 |
| 6 | 4 |

Table 8-2 specifies the mapping table used for converting the number of intra prediction modes.

Table 8-2 – Specification of mapIntraPredMode5 and mapIntraPredMode9

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mapIntraPredMode5[ value ] | 0 | 1 | 2 | 3 | 4 | 0 | 2 | 1 | 1 | 2 | 2 | 0 | 0 | 2 | 2 | 1 | 1 |
| mapIntraPredMode9[ value ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| mapIntraPredMode5[ value ] | 2 | 3 | 4 | 4 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 2 | 2 |
| mapIntraPredMode9[ value ] | 2 | 3 | 4 | 4 | 0 | 0 | 5 | 5 | 6 | 3 | 7 | 7 | 1 | 1 | 8 | 8 | 2 |

IntraPredMode[ xB ][ yB ] labelled 0, 1, 2, .., 33 represents directions of predictions as illustrated in Figure 8-1.

Figure 8-1 – Intra prediction mode directions (informative)

IntraPredMode[ xB ][ yB ] is derived as follows.

–	The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

–	For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to 2

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- If intraPredModeN is greater than or equal to intraPredModeNum

- If intraPredModeNum is equal to 5 then candIntraPredModeN is set equal to mapIntraPredMode5[ intraPredModeN ]

- If intraPredModeNum is equal to 3 then candIntraPredModeN is set equal to mapIntraPredMode3[ intraPredModeN ]

- Otherwise candIntraPredModeN is set equal to mapIntraPredMode9[ intraPredModeN ].

- Otherwise, candIntraPredModeN is set equal to intraPredModeN

- The candModeList[x] and NumMPMCand are derived as follows:

- NumMPMCand is set equal to 4

- If both candIntraPredModeN are not available, then the value 2 is assigned to candModeList[ 0 ] the value 0 is assigned to candModeList[ 1], the value 1 is assigned to candModeList[ 2 ] and the value 34 is assigned to candModeList[ 3 ],

- Otherwise, if both candIntraPredModeN are the same, then this candIntraPredModeN is assigned to candModeList[ 0 ]. candModeList[ 1 ], candModeList[ 2 ] and candModeList[ 3 ] are derived by applying the following procedure with Table 8-3, 8-4, and 8-5.

  - PredModeMinus1_M[candModeList[ 0 ]] is assigned to candModeList[ 2], PredModePlus1_M[candModeList[ 0 ]] is assigned to   candModeList[ 3], where M represents intraPredModeNum.

  - If candModeList[ 0 ] is equal to 34, the value 2 is assigned to candModeList[ 1 ]

  - Otherwise, the value 34 is assigned to candModeList[ 1 ]

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ]. candModeList[ 2 ] and candModeList[ 3 ] is derived by applying the following procedure.

  - If intraPredModeNum is equal to 4, candModeList[ 2 ] and candModeList[ 3 ]  is derived from Table 8-6 by using candModeList[ 0 ] and candModeList[ 1 ] as inputs.

  - Otherwise, if candModeList[ 0 ] is equal to 34, PredModeMinus1_M[candModeList[ 1 ]] is assigned to candModeList[ 2], PredModePlus1_M[candModeList[ 1 ]] is assigned to candModeList[ 3], where M represents intraPredModeNum.

  - Otherwise, if candModeList[ 1 ] is equal to 34, PredModeMinus1_M[candModeList[ 0 ]] is assigned to candModeList[ 2], PredModePlus1_M[candModeList[ 0 ]] is assigned to candModeList[ 3], where M represents intraPredModeNum.

  - Otherwise, 34 is assigned to candModeList[ 3] and candModeList[ 1 ] is derived by applying the following procedure, where M represents intraPredModeNum.

  - If candModeList[ 1 ] is not equal to PredModeMinus1_M[candModeList[ 0 ]],  PredModeMinus1_M[candModeList[ 0 ]] is assigned to candModeList[ 2].

  - Otherwise, if candModeList[ 0 ] is not equal to PredModeMinus1_M[candModeList[ 1 ]],  PredModeMinus1_M[candModeList[ 1 ]] is assigned to candModeList[ 2].

  - Otherwise,  2 is assigned to candModeList[ 2],

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure.

- If prev_intra_pred_flag[ xB ][ yB ] is less than 4, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx[ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following procedure

  - IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

  - The available modes in candModeList [x] are sorted in increasing order and are assigned to candMPMAllList[x], with x = 0…4

- for (cIdx = 0; cIdx < NumMPMCand; cIdx++)

if (IntraPredMode[ xB ][ yB ]>= candMPMAllList [cIdx]) IntraPredMode[ xB ][ yB ]++

  - Table 8-3 Candidate mode mapping when intraPredModeNum is equal to 4

| intraPredMode | 0 | 1 | 2 |
| --- | --- | --- | --- |
| PreModeMinus1_4 | 1 | 0 | 0 |
| PreModePlus1_4 | 2 | 2 | 1 |

Table 8-4 Candidate mode mapping when intraPredModeNum is equal to 18

| intraPredMode_index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredModeMinus1_18 | 11 | 16 | 0 | 14 | 10 | 12 | 13 | 15 | 9 | 6 | 3 | 4 | 0 | 5 | 7 | 1 | 8 |
| PredModePlus1_18 | 12 | 15 | 1 | 10 | 11 | 13 | 9 | 14 | 16 | 8 | 4 | 0 | 5 | 6 | 3 | 7 | 1 |

Table 8-5 Candidate mode mapping when intraPredModeNum is equal to 35

| intraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredModeMinus1_35 | 21 | 30 | 0 | 26 | 19 | 23 | 25 | 28 | 32 | 6 | 18 | 21 | 22 | 24 | 27 | 29 | 31 | 33 |
| PredModePlus1_35 | 22 | 29 | 1 | 18 | 20 | 24 | 9 | 27 | 31 | 33 | 19 | 20 | 23 | 25 | 26 | 28 | 30 | 32 |

| intraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredModeMinus1_35 | 3 | 10 | 4 | 11 | 0 | 12 | 5 | 13 | 14 | 7 | 15 | 1 | 16 | 8 | 17 | 9 | 0 |
| PredModePlus1_35 | 10 | 4 | 11 | 0 | 12 | 5 | 13 | 6 | 3 | 14 | 7 | 15 | 1 | 16 | 8 | 17 | 1 |

Table 8-6 Candidate mode mapping when intraPredModeNum is equal to 4

| candModeList[ 0 ] | 0 | 0 | 0 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| candModeList[ 1 ] | 1 | 2 | 34 | 2 | 34 | 34 |
| candModeList[ 2 ] | 2 | 1 | 1 | 0 | 0 | 0 |
| candModeList[ 3 ] | 34 | 34 | 2 | 34 | 2 | 1 |

    - Derivation process for chroma intra prediction mode

[Ed.: (WJ) this subclause may be moved to the semantics of intra_chroma_pred_mode syntax]

Input to this process is a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

The chroma intra prediction mode IntraPredModeC is derived with intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] as inputs as follows :

- 35 is assigned to intra_chroma_pred_mode[4], IntraPredMode[ xB ][ yB ] is assigned to intra_chroma_pred_mode[5]

- If intraPredModeNum is equal to 4, 0 is assigned to intra_chroma_pred_mode[0], 1 is assigned to   intra_chroma_pred_mode[1], 2 is assigned to intra_chroma_pred_mode[2] and 34 is assigned to   intra_chroma_pred_mode[3].

- Otherwise, IntraPredModeC is derived as specifed in Table 8-7 with intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] as inputs if intraPredModeNum is not equal to 4.

Table 8-7 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ]

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ], X |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2 | 34 | X (X≠2 and X≠34) |  |  |
| 0 | 0 | 0 | PredModeMinus1_M[X] |  |  |
| 1 | 1 | 1 | PredModePlus1_M[X] |  |  |
| 2 | 6 | 2 | PredMode_P[X] |  |  |
| 3 | 34 | 6 | 34 |  |  |

Table 8-8 Perpendicular mode mapping

| IntraPredMode[ xB ][ yB ], X | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredMode_P | 1 | 0 | 0 | 6 | 8 | 7 | 3 | 5 | 4 | 3 | 8 | 16 | 15 | 14 | 13 | 12 | 11 | 10 |

| IntraPredMode[ xB ][ yB ], X | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PredMode_P | 33 | 32 | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 0 |

#### Initialisation process for intraModeTable

Outputs of this process are initial values of the variable array intraModeTable.

The variable array intraModeTable is initialized as specified in Table 9-4.

Table 9-4 – Specification of intraModeTable[k][codeNum]

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - k | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 | - 16 |
| - 0 | - 0 | - 1 | - 4 | - 2 | - 3 | - 6 | - 8 | - 5 | - 7 | - 9 | - 10 | - 12 | - 11 | - 13 |  |  |  |
| - 1 | - 0 | - 1 | - 2 | - 16 | - 5 | - 6 | - 4 | - 8 | - 3 | - 7 | - 9 | - 19 | - 13 | - 17 | - 10 | - 27 | - 12 |

|  | - codeNum |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - k | - 17 | - 18 | - 19 | - 20 | - 21 | - 22 | - 23 | - 24 | - 25 | - 26 | - 27 | - 28 | - 29 | - 30 | - 31 | - 32 | - 33 |
| - 1 | - 14 | - 28 | - 18 | - 15 | - 26 | - 20 | - 11 | - 25 | - 23 | - 24 | - 29 | - 21 | - 30 | - 22 |  |  |  |

#### 9.2.4.6 Parsing process for rem_intra_luma_pred_mode

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element rem_intra_luma_pred_mode in subclause .

Inputs to this process are bits from slice data, a variable puSize specifying the size of the current prediction unit, NumMPMCand and a variable array intraModeTable.

Outputs of this process are the syntax element rem_intra_luma_pred_mode and value-updated intraModeTable.

The value of rem_intra_luma_pred_mode is derived as follows.

–	Based on the input puSize, the value of a variable intraPredModeNum indicating the number of intra prediction modes for the given size of prediction unit is obtained according to Table 7-.

–	If intraPredModeNum is equal to 4,. No further step is carried out. Othewise,

–	The values of variable k and vlcNum are obtained as follows

  - If intraPredModeNum is equal to 18

    - set vlcNum equal to 14 and k equal to 0.

  - If intraPredModeNum is equal to 35

    - set vlcNum equal to 31 and k equal to 1.

  - The parsing process described in subclause Error: Reference source not found is invoked with vlcNum as input and the variable codeNum as output.

  - The value of rem_intra_luma_pred_mode is set equal to intraModeTable[k][codeNum].

  - The value of a variable counterNum is set to 0. The variable array intraModeTable[k] is updated by invoking process in subclause Error: Reference source not found with intraModeTable[k], codeNum and counterNum as inputs.

#### 9.3.2.8 Binarization process for mpm_idx

Input to this process is a request for the syntax element mpm_idx.

Output of this process is the binarization of the syntax element.

The binarization for mpm_idx is given by Table 9-546.

Table 9-46 – Binarization for rem_intra_luma_pred_mode

| Value of rem_intra_luma_pred_mode | Bin string |
| --- | --- |
| 0 | 00 |
| 1 | 01 |
| 2 | 100 |
| 3 | 101 |
| 4 | 11 |

#### 9.3.2.9 Binarization process for rem_intra_luma_pred_mode

Input to this process is a request for the syntax element rem_intra_luma_pred_mode and cNumBins.

Output of this process is the binarization of the syntax element.

The binarization for rem_intra_luma_pred_mode is given by Table 9-547.

Table 9-5 – Binarization for rem_intra_luma_pred_mode

| intraPredModeNum | Value of rem_intra_luma_pred_mode | Bin string |
| --- | --- | --- |
| 18 | 0 | 000 |
| 18 | 1 | 001 |
| 18 | 2-14 | FL, cMax = cNumBins, cMin= 4 |
| 35 | 0 | 0000 |
| 35 | 1-30 | FL, cMax = cNumBins, cMin=2 |