| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F462 |
| --- | --- |

| Title: | On reference picture marking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Huawei Technologies 400 Crossing Blvd, 2nd Floor Bridgewater, NJ 08807, USA | Tel: Email: | +1 908 541 3518 yekui.wang@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document proposes some changes to the reference picture marking process. Specifically, it is proposed that, at any access unit, only reference pictures with an identical or greater value of temporal_id may be marked as “unused for reference”. When “non-existing” pictures are involved, it is assumed that “non-existing” pictures have the greatest possible value of temporal_id, i.e., 7.

# Introduction

Temporal scalability is supported in HEVC WD3, wherein the field temporal_id (3 bits) is present in NAL unit header of the VCL NAL units (with NAL unit type 1, 4, or 5). In a temporally scalable bitstream, some high temporal layers (of greater temporal_id values) can be discarded to generate a bitstream subset with a lower frame rate, and gaps in frame_num may be present in the bitstream subset. In this case, the current HEVC decoding process generates short-term “non-existing” pictures having the missing frame_num values. Such “non-existing” pictures are handled in a similar way as normal short-term reference pictures in the DPB management process. And the reference picture list construction process excludes “non-exiting” pictures from being included in a reference picture list.

However, currently, it is possible that after decoding of a coded picture of a particular value of temporal_id, reference pictures with smaller values of temporal_id may be marked as “unused for reference” and get removed from the DPB. This creates the following problem. When the temporal layer containing such a coded picture and all higher temporal layers are discarded, decoding of another coded picture of a smaller value of temporal_id may become incorrect due to that the reference picture index value of one or more reference pictures used for inter prediction is different compared to when decoding the entire bitstream. Such an incorrect decoding may seriously degrade the quality of that picture itself and also many future pictures that directly or indirectly use that picture for inter prediction reference.

Therefore, it is beneficial to restrict the reference picture marking process such that only reference pictures with identical or greater values of temporal_id may be marked as “unused for reference” after decoding of any particular access unit.

This document proposes text changes for such a restriction.

# Proposed changes

The changes, in relative to HEVC WD3d8, provided below are proposed. The changes to sublcauses 8.2.3.3 and 8.2.3.4.1 are phrased equivalently to a phrase not mentioning the mark as “non-existing” but assuming that “non-existing” pictures have the greatest possible value of temporal_id, i.e., 7. If the processes for generating and handling of “non-existing” pictures are removed from the specification, e.g., as proposed in JCTVC-F460, then the mentioning of the mark as “non-existing” can be simply removed from the changes to sublcauses 8.2.3.3 and 8.2.3.4.1 (without assuming that “non-existing” pictures have temporal_id equal to 7).

# 8.2.3.3  Sliding window decoded reference picture marking process

This process is invoked when adaptive_ref_pic_marking_mode_flag is equal to 0.

Depending on the properties of the current picture as specified below, the following applies.

- Let numShortTerm be the total number of reference pictures that are marked as "used for short-term reference". Let numLongTerm be the total number of reference pictures that are marked as "used for long-term reference".

- When numShortTerm + numLongTerm is equal to Max( max_num_ref_frames, 1 ), the condition that numShortTerm is greater than 0 shall be fulfilled. The short-term reference picture that has the smallest value of FrameNumWrap among those short-term reference pictures that have temporal_id not less than that of the current picture or that are marked as “non-existing” is marked as "unused for reference".

# 8.2.3.4.1  Marking process of a short-term reference picture as "unused for reference"

This process is invoked when memory_management_control_operation is equal to 1.

Let picNumX be specified by

picNumX = CurrPicNum − ( difference_of_pic_nums_minus1 + 1 ). 	(8-13)

The value of picNumX is used to mark the corresponding short-term reference picture as "unused for reference". The short-term reference picture having PicNum equal to picNumX shall either have temporal_id not less than that of the current picture or be marked as “non-existing”.

# 8.2.3.4.2  Marking process of a long-term reference picture as "unused for reference"

This process is invoked when memory_management_control_operation is equal to 2.

The value of LongTermPicNum is used to mark the corresponding long-term reference picture as "unused for reference". The long-term reference picture identified by LongTermPicNum shall have temporal_id not less than that of the current picture.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).