| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F465 M20895 |
| --- | --- |

| Title: | Experiments on tools in Working Daft (WD) and HEVC Test Mode (HM-3.0) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Il-Koo Kim, Elena Alshina, Jianle Chen, Tammy Lee, Woo-Jin Han, JeongHoon Park (Samsung) Vivienne Sze, Madhukar Budagavi, Minhua Zhou (Texas Instruments Inc.) | Email: Email: | ilkoo.kim@samsung.com sze@ti.com |
| Source: | Samsung Electronics Co., Ltd. and Texas Instruments |  |  |

_____________________________

# Abstract

This contribution presents results of tests applied to several tools in the current WD and HM (3.0) to verify whether the included tool produced the intended effects. Each set of results have been cross-checked. Based on the results, this contribution recommends removal of the following tools as they do not show clear benefits in terms of coding efficiency, complexity and even for the design consistency: Center position of TMVP candidates; Chroma codeword switch; Direct coding of Intra DC coefficient in CAVLC mode; Parallel deblocking filter decision; Partial merge; and Planar prediction for chroma; Adaptive switching on/off combined coding for CAVLC. Various methods to simplify ALF-related tool were also investigated including: Region-based ALF only; Single pass ALF and One filter only for ALF.  It is also recommend in this contribution that a tool experiments similar to TE12 for HM1.0 establishment stage be done before current WD moves to committee draft (CD).

Abstract	1

1	Introduction	2

2	Partial merge	2

3	Planar prediction for chroma	3

4	Center position of TMVP candidates	4

5	Parallel deblocking filter decision	5

6	Chroma codeword switch	7

7	Direct coding of Intra DC coefficient in CAVLC mode	8

8	Region-based ALF only	9

9	Single pass ALF	10

10	One filter only for ALF	10

11  Adaptive switching on/off combined coding for CAVLC	11

12	Conclusions	12

13	Patent rights declaration(s)	12

# Introduction

In this contribution, test results of tools which are in the current working draft (WD) and test mode (HM3.0) are given. WD and HM are continuously evolving since HM1.0 is established. In each meeting, several tools are integrated with individual performance verification. However, one tool’s performance may be conflicted with other tools which are adopted in the same meeting concurrently or in successive meetings. Therefore, it is required to verify existing and already adopted technology in WD and HM based on the current integration whether it still gives values to the current WD and HM even after adoption. Furthermore, this kind of sanity check should be performed at the every stage of standardization in order to minimize effects on following adoptions. For this sake, 10 tools were evaluated in current WD and HM. Each set of results has been cross-checked (two sets of results are included with this contribution). The detailed results and suggestions are presented in following sections.

# Partial merge

Currently for partition size of 2NxN and Nx2N in CU larger than 8x8, the first PU is forced to merge. Partial merge was proposed as a method to decrease encoding time since motion estimation does not have to be performed on first PU for partition size 2NxN and Nx2N for CU>8x8. The coding loss of partial merge can be seen in Table 1.

However, there are several concerns with this approach. First, the current CU may have totally different motion from its spatial and temporal neighbors; the forced merge could cause visual quality degradation and coding artifacts. Second, for pipelined hardware architecture in which an encoder may choose to estimate the motion vectors for CUs in parallel, this forced merge mode causes design burden due to its irregularity. Finally, an encoder may choose to support CU size only down to 16x16, the partial CU merge mode essentially disables 2NxN, Nx2N motion vector partitions if the partial merge mode cannot be implemented on the encoder side due to architecture constraints.

For MTU matching with decoupled entropy encoding (i.e. bitstream slicing), there is a need that PUs of merge/skip mode can be coded as regular inter prediction mode if the availability of their neighboring PUs is impacted by insertion of slice header. The partial CU merge essentially disables such a flexibility and forces encoder to give up 2NxN, Nx2N prediction mode in which the first PU has to be merged.

It is interesting to note that similar encoding time savings can be achieved by disabling NxN PU for inter CU, with less coding efficiency loss compared to partial merge as shown in Table 2.

Table 1 Results for disabling partial merge (anchor has partial merge enabled)

|  | Random access HE |  |  | Low delay (B) HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | -1.1 | -1.0 |  |  |  |
| Class B | -0.4 | -0.5 | -0.5 | -0.2 | -0.3 | -0.6 |
| Class C | -0.6 | -0.6 | -0.8 | -0.4 | -0.4 | -0.6 |
| Class D | -0.5 | -0.5 | -0.5 | -0.4 | -0.4 | 0.2 |
| Class E |  |  |  | -0.1 | 0.2 | -0.1 |
| Overall | -0.5 | -0.7 | -0.7 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 116% |  |  | 114% |  |  |
| Dec Time[%] | 104% |  |  | 95% |  |  |

|  | Random access LC |  |  | Low delay (B) LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.7 | -0.7 | -0.7 |  |  |  |
| Class B | -0.4 | -0.3 | -0.3 | -0.3 | -0.2 | -0.2 |
| Class C | -0.7 | -0.6 | -0.6 | -0.5 | -0.3 | -0.4 |
| Class D | -0.5 | -0.6 | -0.6 | -0.3 | -0.2 | -0.2 |
| Class E |  |  |  | -0.1 | 0.5 | 0.4 |
| Overall | -0.6 | -0.6 | -0.5 | -0.3 | -0.1 | -0.1 |
| Enc Time[%] | 116% |  |  | 122% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |

Table 2 Results for disabling partial merge and NxN PU for inter CU (anchor has partial merge enabled)

|  | Random access HE |  |  | Low delay (B) HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5 | -0.8 | -0.9 |  |  |  |
| Class B | -0.3 | -0.5 | -0.5 | -0.1 | -0.2 | -0.6 |
| Class C | -0.5 | -0.4 | -0.5 | -0.2 | 0.0 | -0.2 |
| Class D | -0.2 | -0.2 | -0.1 | 0.0 | 0.1 | 0.6 |
| Class E |  |  |  | -0.1 | -0.4 | -0.2 |
| Overall | -0.4 | -0.5 | -0.5 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

|  | Random access LC |  |  | Low delay (B) LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | -0.5 | -0.6 |  |  |  |
| Class B | -0.4 | -0.3 | -0.3 | -0.2 | -0.1 | -0.1 |
| Class C | -0.4 | -0.2 | -0.2 | -0.2 | 0.0 | -0.1 |
| Class D | -0.1 | -0.1 | -0.1 | 0.1 | 0.3 | -0.1 |
| Class E |  |  |  | -0.2 | 0.2 | 0.3 |
| Overall | -0.4 | -0.3 | -0.3 | -0.1 | 0.1 | 0.0 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Planar prediction for chroma

Table 3 shows the chroma intra prediction mode used in HM3.0. One additional mode (planar mode) is added comparing to HM2.0. This test shows the result of using only five modes as in chroma intra prediction in HM2.0. In this test, if luma intra mode is planar mode and chroma mode is DM mode, then chroma uses planar intra prediction mode. The experimental results (Table 3) show that no performance degradation is found by using only five chroma intra modes with 2% encoding time reduction.

Table 3 chroma intra prediction mode used in HM3.0

| Pseudo mode index | Chroma intra mode |
| --- | --- |
| 0 | Vertical |
| 1 | Horizontal |
| 2 | DC |
| 3 | LM (chroma_fromLuma) |
| 4 | DM (Luma intra mode) |
| 34 | Planar |

Table 4 Results of removal of planar prediction for chroma

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | 0.0 |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class C | 0.1 | 0.0 | 0.0 | 0.1 | 0.2 | 0.1 |
| Class D | 0.1 | 0.0 | 0.0 | 0.1 | 0.3 | 0.3 |
| Class E | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.2 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Center position of TMVP candidates

For AMVP, a set of candidates are used to construct the AMVP list as shown in Figure 1. The AMVP list construction uses five spatial neighboring MVPs and two co-located temporal MVPs to produce a final MVP list of up to 3 MVP candidates.

Temporal MVPs are expensive since they require fetching from a large (e.g. external) memory where the data on the previous frame stored. The two co-located temporal MVPs used are the center position (CR) and H position which is at the bottom-right position right outside of the co-located PU. Reducing the number of candidates reduces the number of availability checks which helps to reduce the complexity of the AMVP list construction. Furthermore, reducing the number of TMVP candidates reduces memory reads from external memory. In this test, the CR co-located temporal MVP is removed (i.e. only H is used for the temporal MVP). This has negligible impact on coding efficiency as shown in Table 5.

Current PU

A0

E

B0

C

D

Co-located PU

CR

1

2

H

Figure 1.  Illustration of the HM-3.0 AMVP list construction.

Table 5 Results for disabling center position TMVP candidate

|  | Random access HE |  |  | Low delay (B) HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.1 | 0.1 |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | 0.0 | 0.1 | 0.1 | -0.1 | 0.1 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | -0.2 | 0.6 |
| Class E |  |  |  | 0.0 | 0.2 | -0.2 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 97% |  |  |

|  | Random access LC |  |  | Low delay (B) LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.1 | -0.1 |  |  |  |
| Class B | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 |
| Class D | -0.1 | -0.1 | -0.1 | 0.0 | -0.1 | -0.5 |
| Class E |  |  |  | -0.1 | -0.4 | 0.5 |
| Overall | 0.0 | -0.1 | -0.1 | -0.1 | -0.1 | 0.0 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 109% |  |  | 103% |  |  |

# Parallel deblocking filter decision

Parallel deblocking filter decision involves computing the boundary strength of the filter based on the un-deblocked pixels. However, even though the boundary strength decision is done in parallel, the filtering is still done sequentially.

Fig. 2 shows the data flow of HM2.0 vs. HM3.0.  Note that in AVC, the vertical edge filtered pixels can be stored in the same memory as the original pixel (i.e. in place computation.) However, for parallel deblocking in HM3.0, the vertical edge filtered pixels cannot use the same memory as the original pixels since the original pixels are required for calculating the boundary strength and on/off decision for the horizontal filter. Horizontal edge filtering requires reading from two different memories, one for decision (unfiltered pixels) and one for the actual pixel filtering (vertical filtered pixels). This results in an increase the amount of storage and memory bandwidth. Disabling parallel deblocking filter decision has negligible impact on coding efficiency as shown in Table 6.

Fig.2 Comparison of deblocking architecture for HM-2.0 and parallel deblocking filter decision (HM3.0).  Horizontal edge decision & filtering required reads from two separate memories for parallel deblocking filter decision. Blue values can be stored in one memory, green value must be stored in a separate memory.

Table 6 Results for disabling parallel deblocking filter

|  | All intra HE |  |  | Random access HE |  |  | Low delay (B) HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | -0.2 | 0.2 |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | 0.2 |
| Class E | 0.0 | 0.0 | 0.0 |  |  |  | 0.0 | 0.2 | 0.2 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  | 97% |  |  |

|  | All intra LC |  |  | Random access LC |  |  | Low delay (B) LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.2 |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.3 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | -0.1 |
| Class E | 0.0 | 0.0 | 0.0 |  |  |  | 0.1 | -0.2 | -0.7 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  | 97% |  |  |

# Chroma codeword switch

Table 7 chroma intra prediction mode codeword in HM3.0

| intra_chroma_pred_mode | Intra_luma_pred_mode |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | others |
| 0 (vertical) | n/a | 111 | 110 |
| 1(horizontal) | 111 | n/a | 1111 |
| 2 (DC/Planar) | 110 | 110 | 1110 |
| 3 (LM) | 10 | 10 | 10 |
| 4 (same as luma) | 0 | 0 | 0 |

Table 7 shows the codeword table of chroma intra mode in HM3.0. If luma mode is neither DC nor Planar mode and chroma mode is 2, one additional flag is signaled to indicate which one is used between DC and Planar mode. In chroma codeword switching scheme, the longest two codewords marked by highlighted texture are switched compared to normal unary binarization (in comparison of Table 8), which needs additional operation to switch between them.

In this experiment, the codeword generated with normal unary binarization is tested. As shown in Table 8, the highlighted codewords are switched back. The experimental results show that the normal unary binarization method does not cause any performance degradation for CABAC entropy coding. And performance of CAVLC entropy coding does not change at all since the same codeword length is utilized.

Table 8 chroma intra prediction mode codeword with unary binarization

| intra_chroma_pred_mode | Intra_luma_pred_mode |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | others |
| 0 (vertical) | n/a | 110 | 110 |
| 1(horizontal) | 110 | n/a | 1110 |
| 2 (DC/Planar) | 111 | 111 | 1111 |
| 3 (LM) | 10 | 10 | 10 |
| 4 (same as luma) | 0 | 0 | 0 |

Table 9 Results of chroma codeword switching

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | 0.0 |
| Class B | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | 0.1 |
| Class D | 0.0 | 0.1 | 0.1 |
| Class E | 0.0 | 0.0 | 0.1 |
| All | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Direct coding of Intra DC coefficient in CAVLC mode

In the current HM3.0, when the entropy coding mode is selected as CAVLC, the DC coefficient value of Intra CU is coded directly only if both upper and left PUs are not available. In this test, the efficiency of direct coding method of Intra DC coefficient in CAVLC mode is verified in both common test condition and slice mode test condition.

Table 10 shows the result by disabling direct coding of intra DC coefficient in CAVLC. Since the condition such that both upper and left PUs are not available happens only one time at the beginning of every picture coding under the default common test condition, almost no performance difference is observed.

Table 10 Results under common test condition

To increase the number of cases in which direct coding DC coding is applied, it is also evaluated under the slice mode test condition. Such cases are marked by red circle in Figure 3. By removing of direct coding of DC coefficient, Table 11 shows the results on slice mode in which the slice is set to 1500 bytes per one slice.

Figure 3 Direct DC coding cases in slice mode test condition

Table 11 Results under slice mode test condition

# Region-based ALF only

Both block-based and region-based ALF are used in HM3.0. For block-based ALF, a variance metric is calculated for every 4x4 block and the result is used to select one of up to 16 filters. For region-based ALF, the frame is divided into 16 regions each with a different filter (the same filter is used in a given region so that filter switch only occurs across different regions). Switching between block-based and region based can occur at a slice level. In this section, test results are provided where only region-based ALF is used. Three test cases: All Intra High Efficiency, Random access High Efficiency, Low delay (B) High Efficiency; were studied under common test conditions [1]. Test results are shown in Table 12. Only region based ALF results in a coding loss of 0.3%, 0.4% 0.4% for AI-HE, RA-HE and LD-HE

Table 12 Test results: Only region based ALF vs both block based and region based ALF

|  | All intra HE |  |  | Random access HE |  |  | Low delay (B) HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.5 | 0.0 | 0.0 | 0.6 | 0.2 | 0.3 |  |  |  |
| Class B | 0.2 | 0.0 | 0.0 | 0.6 | 0.0 | 0.1 | 0.4 | 0.4 | 0.2 |
| Class C | 0.2 | 0.0 | 0.0 | 0.3 | 0.1 | 0.0 | 0.4 | 0.0 | -0.1 |
| Class D | 0.2 | -0.1 | -0.1 | 0.1 | -0.1 | -0.2 | 0.1 | -0.2 | 0.3 |
| Class E | 0.4 | -0.2 | -0.2 |  |  |  | 0.5 | -0.3 | -0.3 |
| Overall | 0.3 | 0.0 | 0.0 | 0.4 | 0.0 | 0.1 | 0.4 | 0.0 | 0.1 |
| Enc Time[%] | 96% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 95% |  |  | 97% |  |  | 97% |  |  |

.

# Single pass ALF

Default number of passes used for ALF in HM3.0 is 16. In this section, test results are provided when encoder search ALF process uses only one pass. Compare to default settings, the number of passes was reduced to be one by following change of configuration file:

ALFEncodePassReduction        : 1

Three test cases: All Intra High Efficiency, Random access High Efficiency, Low delay (B) High Efficiency; were studied under common test conditions [1]. Test results are shown in Table 13Error: Reference source not found.

Table 13 Test results: single pass ALF vs 16 passes ALF

|  | All intra HE |  |  | Random access HE |  |  | Low delay (B) HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | -0.1 | 0.1 | 0.1 |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.1 | 0.4 | 0.3 | 0.4 |
| Class C | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.3 | -0.1 | 0.0 |
| Class D | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | -0.1 | 0.3 | 0.0 | 0.2 |
| Class E | 0.0 | -0.1 | -0.1 |  |  |  | 0.5 | -0.1 | 0.1 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.4 | 0.1 | 0.2 |
| Enc Time[%] | 92% |  |  | 103% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  | 99% |  |  |

Noticeable performance degradation was found for Low Delay (B) HE test case only and average across 3 test cases performance drop is 0.1%/0.0%/0.1% (Y/U/V). For All Intra HE test cases, this change leads to 8% encoding time reduction. An average encoding time reduction is 2%, but encoding time is very rough measure for complexity.

# One filter only for ALF

In HM3.0, two different ALF can be switched between block-based ALF (ALR_BA) and region-based ALF (ALF_RA). Each ALF method can use maximum 16 filters. Block-based ALF utilizes block-based pixel classification to get 16 distinctive classes. Region-based ALF separate a picture to 16 equal size regions. One filter is generated for each class or region. Class and region can be merged and number of filter can be reduced. In this test, the effect of number of filter in ALF is verified. Instead of 16 filters, only one filter is utilized for both block-based ALF and region-based ALF.

In order to set the number of filter as 1, NO_VAR_BIN macro is set as 1 instead of 16 in HM3.0 software and source code was modified accordingly to work properly with this macro setting. The result is summarized in Table 14Error: Reference source not found. Average loss for all high efficiency configurations is 0.8% with 98% and 97% encoding and decoding time compared to HM3.0.

Table 14 Result of one filter usage for ALF

|  | All intra HE |  |  | Random access HE |  |  | Low delay (B) HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.7 | 0.0 | 0.0 | 0.9 | 0.0 | 0.4 |  |  |  |
| Class B | 0.5 | -0.2 | -0.1 | 1.3 | -0.2 | -0.1 | 1.3 | 0.1 | -0.2 |
| Class C | 0.6 | -0.3 | -0.3 | 0.6 | -0.3 | -0.4 | 0.8 | -0.5 | -0.5 |
| Class D | 0.2 | -0.2 | -0.2 | 0.2 | -0.4 | -0.4 | 0.2 | 0.0 | -0.6 |
| Class E | 0.6 | -0.4 | -0.4 |  |  |  | 1.5 | -0.1 | -0.5 |
| Overall | 0.5 | -0.2 | -0.2 | 0.8 | -0.2 | -0.1 | 0.9 | -0.1 | -0.4 |
| Enc Time[%] | 97% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 95% |  |  | 98% |  |  | 97% |  |  |

# Adaptive switching on/off combined coding for CAVLC

In HM3.0, combined coding for inter prediction direction and reference frame index in CAVLC is adaptively switched on and off by checking neighboring (the left, the above and the above left) PUs information. Combined coding has two different schemes to code inter prediction direction and reference frame index: the lookup table which can cover the cases of the reference frame index less than 2 in both directions and the separate coding of inter prediction direction and reference frame index in other cases (i.e. exception cases) by excluding the lookup table combinations. Therefore, by switching on/off combined coding, the difference is mainly occurred by whether such cases (i.e the reference frame index less than 2 in both directions) are coded by lookup table or by separate coding in B slice.

The adaptive scheme for switching on/off combined coding checks whether all the surrounding (left, above and above left) PUs do not use the reference frame with the index equal or greater than 2. Only if such condition is satisfied, combined coding is automatically turned on for the current PU.

In this contribution, the adaptive scheme is disabled and the combined coding is turned off only if slice type is not B slice. Since turning off condition of the adaptive scheme is never occurred in the current default condition, it is tested with the following number of references:

NumOfReference		: 4           # Number of reference frames
NumOfReferenceB_L0	: 4           # Number of reference frames for L0 for B-slices
NumOfReferenceB_L1	: 4           # Number of reference frames for L1 for B-slices

The result is summarized in Table 15Error: Reference source not found. In average, no loss for low complexity profile is observed compared to HM3.0

Table 15 Disabling adaptive switching scheme

|  | Random access LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |
| All | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.1 | 0.1 | 0.1 |
| Class C | 0.0 | 0.2 | 0.0 |
| Class D | 0.0 | 0.1 | -0.2 |
| Class E | 0.0 | 0.3 | 0.0 |
| All | 0.0 | 0.2 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusions

In this contribution, test results are provided for tools in the current WD and HM. Based on the results of each test, removal of following tools are recommended, as they do not show clear benefits in terms of coding efficiency, complexity and design consistency.

- Partial merge

- Planar prediction for chroma

- Center position of TMVP candidates

- Parallel deblocking filter decision

- Chroma codeword switch

- Direct coding of Intra DC coefficient in CAVLC mode

- Adaptive switching on/off combined coding for CAVLC

In addition, it is suggested that results of ALF-related tools to simplify ALF are to be considered in the HEVC standardization.

- Region-based ALF only

- Single pass ALF

- One filter only for ALF

Finally, it is proposed to have TE like tool experiments similar to TE12 in HM1.0 establishment stage before current WD moves to committee draft (CD). This process will help to double-check whether each tool still has positive effects after integration and no conflicts with other tools, and it would be beneficial to the HEVC standard.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. and Texas Instruments may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).