| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F473 WG11 Number: m20903 |
| --- | --- |

| Title: | CE1: Cross-check report of experiment A.2 and A.10 by Panasonic |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi | Tel: Email: | +81 6 6900 9689 sugio.toshiyasu@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result of CE1 experiment A.2 and A.10 in JCTVC-E701. The experimental results perfectly match with the one provided by Sony and MediaTek in the rate-distortion performance.

- Experimental Condition

Table 1 shows summary of the experiment A.2 and A.10 for category A in CE1 [1]. MVSR position and TMVP position are described in Figure 1 and 2. The software to be tested was provided by LG and its performance relative to the HM3.0 software was checked under the common test condition described in JCTVC-E700 [2] and table 2 macro setting provided by LG. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

Table 1: Summary of the experiments A.2 and A.10 for category A.

| Experiment | MVSR (position) | Prediction (TMVP position) | Tester | Checker |
| --- | --- | --- | --- | --- |
| Anchor | HM3.0 (TL) | HM3.0 (H or C0) | - | - |
| A.2 | C0 | HM3.0 | Sony | Panasonic |
| A.10 | C0 | C3 | MediaTek | Panasonic |

Figure 1: Description of the positions for MDSR.

Figure 2: Description of the positions for TMVP.

Table 2: Micro setting

| Experiment | MDSR | TMVP | Configuration |
| --- | --- | --- | --- |
| Anchor | HM3.0 (TL) | HM3.0 (H or C0) | #define LG_DISABLE_TMVP_H_AMVP 0 #define LG_DISABLE_TMVP_H_MRG 0 #define MTK_MV_COMPRESS_POS 0 #define MTK_TMVP_SELECT_POS 1 |
| A.2 | C0 | HM3.0 (H or C0) | #define LG_DISABLE_TMVP_H_AMVP 0 #define LG_DISABLE_TMVP_H_MRG 0 #define MTK_MV_COMPRESS_POS 3 #define MTK_TMVP_SELECT_POS 1 |
| A.10 | C0 | C3 | #define LG_DISABLE_TMVP_H_AMVP 1 #define LG_DISABLE_TMVP_H_MRG 1 #define MTK_MV_COMPRESS_POS 3 #define MTK_TMVP_SELECT_POS 2 |

- Results

R-D performance and execution time of each experimental result are summarized in Table 3 to 4. Detailed results are included in the attached excel sheet. It was confirmed that these results perfectly match with the one provided by Sony and MediaTek.

Table 3: Experimental results of A.2

Table 4: Experimental results of A.10

- Conclusion

The results of CE1 experiment A.2 and A.10 were confirmed.

- References

- Joel Jung et al., “Description of Core Experiment CE1: Motion Data Storage Reduction”, JCTVC-E701, Geneva, March 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, Geneva, March 2011.