| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F474 |
| --- | --- |

| Title: | CE9: Description of experiments ROB01 and ROB02 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information/ Proposal |  |  |
| Author(s) or Contact(s): | Guillaume LAROCHE Patrice ONNO Tangi POIRIER Christophe GISQUET Canon Research Centre France Rue de la Touche Lambert 35510 Cesson-Sevigne, FRANCE | Email: | guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr christophe.gisquet@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution reports the results of ROB01 and ROB02 experiments for section 4.5 of CE 9 as described in JCTVC-E709. Both experiments deal with the parsing robustness related to the use of the temporal predictor in Inter and Merge modes. The ROB01 experiment fully solves this parsing issue by determining the number of predictors in the set according to the availability of the spatial predictor positions. The ROB02 experiment uses the same scheme to fix parsing issue. The difference comes from the use of a specific derivation process which produces non-redundant predictors. ROB02 experiment obtains 0.5% coding loss compared to the CE9 anchor with 95% decoding runtime variation.

# Introduction

In HM3.0, AMVP and Merge candidates consist not only of spatial MVP candidates but also of temporal MVP candidates from previous reference pictures. Before coding the index of the best MVP candidate, redundant MVPs are removed from the candidates set. This might cause parsing errors when previous reference pictures are lost. In JCTVC-E219 [1] we presented two tools to solve the parsing issue related to the temporal motion prediction of Inter and Merge modes. The first tool (Tool 1) consists in forcing the availability of the temporal candidate for the Inter and Merge modes even if this predictor doesn’t exist. Thanks to this tool, the parsing issue is fully solved. The second tool (Tool 2) aims at improving the coding efficiency of the first tool. This second tool consists in replacing the duplicate candidates by candidates with different (non-redundant) values.

Moreover during the 5th JCTVC meeting, it was pointed out in JCTVC-E118 [3] that the AMVP and Merge candidate index coding implies a parsing throughput due to the current (HM3.0) candidates derivation. Indeed, the index coding depends on the number of candidates in the set. Consequently, all the predictors set needs to be derived to read the index of the best candidate. The solution proposed in this contribution solves the throughput issue of the predictor index decoding for both Inter and Merge modes. Indeed, only the availability of spatial predictors needs to be checked to decode the candidate index. The spatial neighboring availability is easy to compute in advance. This was pointed out in JCTVC-F341 [4] related to a solution which solves the parsing throughput issue.

Finally, the proposed solution reduces the memory complexity and the computational complexity for parsing applications. Indeed, with the proposed solution, the motion vector field doesn’t need to be computed and stored as currently needed in the HM3.0 design.  Moreover, each slice can be parsed independently. These two points are interesting features that were supported in the previous standards.

# Technical description

## Tool 1: Determination of the number of candidates based on spatial candidates

The first tool entirely solves the parsing issue for all frames. Indeed, with Tool 1 the number of candidates in the set depends only on the data which comes from the current slices and not from the data of another slice. Combined with HM3.0 motion vector derivation process, this tool consists in testing the availability of the spatial candidates and it considers that the temporal candidate is always available for the encoding and decoding candidates index for both Inter and Merge modes. However, for Merge mode, some spatial predictors can be removed with the Avoid Merge concept as the HM3.0. In the worst case, 5 spatial positions are checked for Inter mode and 4 for Merge modes.

For ROB01 experiment, only if the bottom right temporal predictor and the collocated center are not available the temporal predictor is changed by the zero value. The derivation process of predictor is the same as the HM3.0 anchor. So, the duplicate candidates are removed from the set but it is not taken into account for the index coding.

For the specific case where the temporal slice is an Intra slice, we introduce a flag in the bitstream to specify if the temporal predictor is used or not in the set of candidates. This flag is transmitted for each Inter slice and it is equal to “false” when the temporal slice is an Intra slice. This saves 0.1% bitrate for random access cases.

## Tool 2: Generation of non-redundant predictors set

Tool 2 improves the coding efficiency of the first tool. With Tool 2, the duplicate candidates are replaced by candidates with different values (non-redundant). This offers the possibility to efficiently use the signaling bits which have to be transmitted to solve the parsing issue and which may cause coding efficiency loss. Moreover, the combination of Tool 1 and Tool 2 can be used to reduce the complexity at decoder by restricting the number of predictors which need to be derived (cf. section 2.5). The following sub-sections describe the derivation process for Tool 2.

### Inter mode

For AMVP, the modifications are only related to the above and temporal predictor. The derivation of the left predictor is not affected by Tool 2 derivation. Figure 1 shows the spatial and temporal positions checked in this proposed derivation process.

#### Left

No change compared to HM3.0

#### Above

If the above predictor exists, as defined by Tool 1, the same spatial above positions as the HM3.0 anchor are checked. At the end of this process, if the above predictor is a duplicate predictor compared to the left predictor, the above predictor is replaced by the following predictors until a non-duplicate value is found:

- Temporal motion vector at the same position as the above predictor (T Above).

- Above predictor without ¼-pel accuracy.

- Above predictor without sub-pel accuracy.

- Above predictor plus a delta on horizontal component of the motion vector predictor.

With the last derivation, the value of the above candidate is definitely a non-duplicate compared to the Left predictor.

Figure 1 Spatial and temporal positions checked by Tool 2 derivation for AMVP.

#### Temporal

With tool 1 the temporal predictor is always in the set. Consequently the same temporal derivation as HM3.0 is applied. It means that if the bottom right temporal position (H candidate) exists it is tested. If its value is a duplicate value compared to the left or the above candidate, the temporal predictor is changed by the value of the center collocated if it exists. Otherwise the temporal predictor is equal to the zero value. If it is still a duplicate, the following predictors are tested until a non-duplicate value is found:

- Temporal below left

- Temporal above right

- Temporal above left

- Temporal predictor without ¼-pel accuracy.

- Temporal predictor without sub-pel accuracy.

- Temporal predictor plus a delta on horizontal component of the motion vector predictor.

- Temporal predictor minus a delta on horizontal component of the motion vector predictor.

If the predictor reaches the last predictor, the temporal predictor is definitely a non duplicate candidate.

## Merge and Merge Skip modes

For Merge and Merge Skip modes the modifications are related to the four last predictors (the first predictor is not impacted by Tool 2).  shows the spatial and temporal positions checked in this proposed Merge derivation process.

### Left

No change compared to HM3.0 design.

### Above

If the above predictor is a duplicate, the following predictors could be tested until a non-duplicate value is found:

- Temporal above

- Above predictor without ¼-pel accuracy.

- Above predictor without sub-pel accuracy.

- Above predictor plus a delta on horizontal component of the motion vector predictor.

### Temporal

For the temporal predictor exactly the same derivation as AMVP is used (cf. section 2.2.1.3)

### Corner Above Right

If the corner above right is a duplicate predictor, the following predictors are tested until a non-duplicate value is found:

- Temporal above right

- Corner above right without ¼-pel accuracy.

- Corner above right without sub-pel accuracy.

- Corner above right plus a delta on horizontal component of the motion vector predictor.

- Corner above right minus a delta on horizontal component of the motion vector predictor.

- Corner above right plus a delta on vertical component of the motion vector predictor.

### Corner Below Left

If the corner below left is a duplicate predictor, the following predictors are tested until a non-duplicate value is found:

- Temporal below Left

- Corner below Left without ¼-pel accuracy.

- Corner below Left without sub-pel accuracy.

- Corner below Left plus a delta on horizontal component of the motion vector predictor.

- Corner below Left minus a delta on horizontal component of the motion vector predictor.

- Corner below Left plus a delta on vertical component of the motion vector predictor.

- Corner below Left minus a delta on vertical component of the motion vector predictor.

Some temporal positions are checked twice in the worst case. This should reduce the memory access to this predictor in fast implementation.

Figure 2  Spatial and temporal positions checked by Tool 2 derivation for Merge and Merge Skip modes.

## Avoid Merge concept for Tool 1 and Tool 2

The ‘avoid Merge’ concept was introduced in Merge candidate derivation for non 2Nx2N partitions to avoid imitating 2Nx2N partitioning. This scheme can be summarized as the following:

- For a second partition 2NxN, all candidates equal to the motion of the first PU (the left PU) are removed from the set.

- For a second partition Nx2N, all candidates equal to the motion of the first PU (the above PU) are removed from the set.

- For a fourth partition NxN, if the three other PUs have the same motion, the candidates which have the same motion as these three other PUs are removed from the set.

During CE9 discussions, it was pointed that our implementation of JCTVC-E219 [1] doesn’t include the ‘avoid merge’ concept. So, for the HM3.0, we have adapted the ‘avoid imitating 2Nx2N’ concept for Merge modes as follows:

Tool 1:

-The Left predictor of the Merge modes is removed from the set for the second 2NxN PU.

-The Right predictor of the Merge modes is removed from the set for the second Nx2N PU.

Tool 2:

For a second PU, the value of a non-duplicate candidate is determined by taking into account the value of the first PU. In that case, no candidate should have the same value as the first PU.

## Fast derivation process at decoder side for Tool 2

In our first implementation in HM2.0 presented in JCTVC-E219 [1], the use of Tool 2 significantly increases the decoding time. For HM3.0, we first reduced it by significantly decreasing the amount of possible candidate. But the superior number of predictors used in the derivation increases the decoding time by 1%.

The use of the combination of Tool 1 and Tool 2 can reduce the decoding complexity. Indeed, the candidate index for Inter and Merge Modes doesn’t depend on the derivation process of the set. Consequently at decoder side, the index of predictor can be known before applying the predictor set derivation. In our Tool 2, a candidate is considered to be a duplicate candidate compared to the previous candidates already inserted in the list of candidates. So, when the index is equal to i, the decoder can derive only the ith   predictors in the set. For ROB02, we implemented this functionality (only the ith   predictors of the set are derived instead of all predictors). This gives a significant reduction with 95% decoding runtime compared to HM3.0. Please note that all predictors of the HM3.0 Anchor need to be derived before selecting the right predictor.

Moreover, this modification can be used at encoder side for fast encoding. Indeed, with the combination of Tool 1 and Tool 2 the encoder could derive a restricted number of candidates in the set (at least one as in the previous standards which use only one motion vector predictor). In the same way, the encoder can limit the amount of operations at decoder side (the number of predictors derived) by selecting the best predictor among the first candidates which need a lower amount of operations. This is a new functionality for encoder compared to the HM3.0 anchor.

## Encoding time complexity reduction

The combination of Tool 1 and Tool 2 (corresponding to ROB02 experiment) increases the average number of predictors in the predictors set for both Inter and Merge mode. The encoding time increase induced by ROB02 comes from this higher number of predictors in the set because these additional predictors are evaluated at encoder side. By studying it, we found that a large part of the complexity increase comes from the RD cost evaluation of Merge mode 2Nx2N (and not from the Merge Skip mode). So, for ROB02, we disabled the RD cost evaluation of non-duplicate candidate obtained with Tool 2 for Merge Mode 2Nx2N (and not for Merge Skip mode). This leads to a reduction of 5% for the encoding time with 0.1% BDR increase.

# Results

The implementation was conducted in the HM3.0 with the inclusion of bug #146 as described in JCTVC-E709 [2]. Simulation results are based on sequences and common test configurations specified in the document JCTVC-E700 [5].

## ROB01 (Tool 1 only)

The following table gives the results of the ROB01 experiment. On average, ROB01 obtains 1.4% loss compared to the HM3.0 Anchor (1.2% without Class E). The impact of ROB01 is lower on RA cases with 1% loss only.  ROB01 has no significant impact on encoding and decoding time complexity.

Table 1 Simulation results for ROB01 experiment.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.8 | 0.7 | 0.5 | 0.9 | 0.9 | 0.5 |
| Class B | 1.0 | 0.7 | 0.5 | 1.1 | 0.8 | 0.7 |
| Class C | 1.1 | 1.0 | 0.9 | 1.1 | 0.9 | 1.0 |
| Class D | 1.1 | 0.9 | 0.9 | 1.1 | 0.6 | 0.7 |
| Class E |  |  |  |  |  |  |
| Overall | 1.0 | 0.8 | 0.7 | 1.1 | 0.8 | 0.7 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.4 | 1.1 | 0.8 | 1.5 | 1.0 | 1.1 |
| Class C | 1.3 | 1.0 | 1.0 | 1.2 | 0.8 | 0.8 |
| Class D | 1.3 | 1.1 | 1.0 | 1.3 | 0.7 | 0.6 |
| Class E | 3.5 | 3.5 | 2.8 | 3.8 | 2.8 | 3.5 |
| Overall | 1.7 | 1.5 | 1.3 | 1.8 | 1.2 | 1.3 |
| Overall wo CL E | 1.3 | 1.1 | 0.9 | 1.3 | 0.8 | 0.8 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

## ROB02 (combination of Tool 1 and Tool 2)

The following table summarizes the results of ROB02. On average, ROB02 obtains 0.5% loss compared to the HM3.0 anchor and 0.4% without the Class E. On average, the encoding variation is equal to 103%. This comes from a higher number of predictors in the set. And the decoding time variation is on average equal to 95%. This decoding time reduction comes from the fast derivation process at decoder side described in section 2.5.

Table 2 Simulation results for ROB02 experiment.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4 | 0.3 | 0.5 | 0.2 | 0.4 | 0.2 |
| Class B | 0.6 | 0.5 | 0.4 | 0.5 | 0.6 | 0.5 |
| Class C | 0.7 | 0.8 | 0.8 | 0.4 | 0.5 | 0.5 |
| Class D | 0.6 | 0.7 | 0.6 | 0.3 | 0.3 | 0.3 |
| Class E |  |  |  |  |  |  |
| Overall | 0.6 | 0.6 | 0.6 | 0.4 | 0.4 | 0.4 |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 96% |  |  | 94% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | 0.1 | -0.4 | 0.1 | 0.2 | 0.5 |
| Class C | 0.5 | 0.4 | 0.2 | 0.2 | 0.1 | 0.3 |
| Class D | 0.5 | 0.1 | 1.0 | 0.4 | -0.2 | 0.0 |
| Class E | 1.9 | 2.2 | 1.4 | 1.4 | 2.1 | 2.7 |
| Overall | 0.8 | 0.6 | 0.4 | 0.4 | 0.4 | 0.7 |
| Overall wo CL E | 0.5 | 0.2 | 0.2 | 0.2 | 0.1 | 0.3 |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 95% |  |  | 94% |  |  |

## Parsing Check

In order to be sure that the ROB01 and ROB02 experiments solve entirely the parsing issue, we have simulated losses at decoder side (as in JCTVC-E219). In this implementation, for the decoding of each frame of the sequence, all its reference frames are re-initialized. It means that all data for all CUs of the previous frame are initialized (CU partitioning, mode, reference frames indexes, directions, motion vectors, etc.). Please note that the slice header is also lost, except the POC number. Indeed, the same parsing issue when the POC of the reference frame is lost occurs in HEVC as in H.264/AVC. In an error resilient implementation, this issue can be solved easily. This functionality can be enabled/disabled by a macro in our CE9 software and easily tested.

For both experiments (ROB01 and ROB02) and each test case (RAHE, RALC, LDHE, LDLC), the decoder did not crash which demonstrates that our proposed solutions fully solve the parsing issue of Inter and Merge modes.

## Simplified ROB02

The core experiment 9 addresses the simplification of the AMVP motion vector predictors derivation. It was pointed during CE9 discussions that ROB02 increases the complexity of the motion vector derivation process by increasing the number of scaling operations. So, we simplified the derivation process of Tool 2 in order to limit the amount of scaling operations and the number of predictors derived in the worst case.

For AMVP derivation process, we limited the amount of scaling operations to one for each candidate. It means that if a tested predictor is scaled for one candidate position and if it is a duplicate candidate, the following predictors in the list won’t be scaled. Yet, for the non-duplicate temporal predictors, the motion vector which has the same POC difference as the current vector is selected.

For the Merge derivation process, only one scaling per direction is used for the temporal candidate as for the AMVP process. For other candidates, the non-redundant temporal predictors are checked as for the AMVP process. Table 3 shows in the worst-case thef numbers of scaling operations for the HM3.0, ROB02 experiment and simplified ROB02 experiment. For Merge modes, the maximum number of scaling operations for the simplification of ROB02 is the same as for the HM3.0 anchors. For AVMP, the maximum number of scaling operations is limited to 6 which is a reduction compared to the HM3.0 which has a maximum of 10 scaling operations.

Table 3 Maximum numbers of scaling operations per mode for HM3.0, ROB02, Simplified ROB02.

|  | Merge modes | AMVP (L0 & L1) |
| --- | --- | --- |
| HM3.0 | 2 | 10 |
| ROB02 | 16 | 24 |
| Simplified ROB02 | 2 | 6 |

In the derivation process of Tool 2 when no predictor position gives a non-redundant candidate, the quarter-pel accuracy is removed and so is the sub-pel accuracy if needed. In the simplified ROB02, we removed that last process.

The following table summarizes the results for ROB02 with the described simplifications. Small gains of 0.1% bitrate savings were obtained on RAHE, RALC and LDHE compared to ROB02.

Table 4 Simulation results for Simplified ROB02 experiment.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4 | 0.4 | 0.4 | 0.3 | 0.3 | 0.0 |
| Class B | 0.7 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Class C | 0.7 | 0.7 | 0.7 | 0.4 | 0.5 | 0.6 |
| Class D | 0.7 | 0.6 | 0.6 | 0.5 | 0.2 | 0.3 |
| Class E |  |  |  |  |  |  |
| Overall | 0.6 | 0.5 | 0.5 | 0.4 | 0.4 | 0.4 |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 98% |  |  | 95% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | -0.2 | -0.4 | 0.0 | 0.0 | 0.0 |
| Class C | 0.4 | 0.1 | 0.1 | 0.0 | -0.1 | 0.0 |
| Class D | 0.5 | -0.1 | 0.8 | 0.3 | 0.0 | -0.2 |
| Class E | 1.9 | 2.1 | 1.5 | 1.4 | 2.1 | 2.8 |
| Overall | 0.7 | 0.3 | 0.4 | 0.4 | 0.3 | 0.5 |
| Overall wo CL E | 0.5 | -0.1 | 0.1 | 0.1 | -0.1 | 0.0 |
| Enc Time[%] | 103% |  |  | 105% |  |  |
| Dec Time[%] | 98% |  |  | 94% |  |  |

# Acknowledgements

We would like to acknowledge Microsoft for their cross checking. And we would like to thank Minhua Zhou from TI and Yoshinori Suzuki from NTT DOCOMO for their help on “Avoid Merge” implementation and their additional experiment proposals.

# Conclusion

This contribution provides the description and results of ROB01 and ROB02 experiments of CE9. Both experiments fully solve the parsing issue of temporal predictor. Compared to the first contribution JCTVC-E219 [1], some implementation improvements were integrated to reduce the complexity and some adaptations were proposed to match with the current HM3.0 design. The experiment ROB01 gives 1.4% of coding loss without impacting the complexity. The best compromise is obtained with ROB02 which gives 0.5% coding loss with small impact on encoding time and a significant decrease of decoding time (95%). Moreover, simplifications of the derivation process were investigated. These simplifications enable to reduce the number of scaling operations compared to the current anchor and give gains compared to the initial ROB02.

In conclusion, the proposed ROB02 offers the following advantages compared to the current HM design:

- Fully solves the parsing robustness issue

- Fully solves the parsing throughput issue

- Good compromise between coding efficiency/complexity/robustness

- Decoding time reduction

- Computational and memory complexity reduction for parsing applications. (No motion vectors value computed or stored).

According to the ROB02 results and its related advantages, we propose to adopt the related modification in the HM design.

# Annex: additional experiments

During CE9 discussion since the Geneva meeting, it was pointed that it would be interesting to evaluate the efficiency of Tool 2 in the HM3.0. The aim of this experiment is to understand if Tool 2 improves only the coding efficiency of Tool 1 or if it also improves the coding efficiency of the current Anchor. For this experiment, we disabled the Tool 1 in ROB02. It means that the temporal candidate isn’t always available but its availability depends on its existence and on its mode. So, this experiment doesn’t solve the parsing issue.

The following table gives the results of this experiment. On average, this experience obtains the same coding efficiency as the ROB02 experiment with 0.5% BDR loss. So, Tool 2 improves the coding efficiency when it is combined with Tool 1 but decreases the coding efficiency of the HM3.0 anchor.

Table 5 Simulation results for “Tool2 in HM3.0 context” experiment.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.4 | 0.5 | 0.4 | 0.1 | 0.4 | 0.1 |
| Class B | 0.5 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 |
| Class C | 0.5 | 0.5 | 0.5 | 0.3 | 0.3 | 0.4 |
| Class D | 0.5 | 0.6 | 0.4 | 0.3 | 0.2 | 0.4 |
| Class E |  |  |  |  |  |  |
| Overall | 0.5 | 0.5 | 0.4 | 0.3 | 0.3 | 0.3 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.6 | 0.3 | 0.1 | 0.3 | 0.3 | 0.3 |
| Class C | 0.5 | 0.3 | 0.4 | 0.2 | 0.1 | 0.0 |
| Class D | 0.4 | 0.0 | 0.7 | 0.3 | 0.1 | 0.2 |
| Class E | 1.8 | 1.8 | 1.8 | 1.5 | 2.0 | 2.2 |
| Overall | 0.8 | 0.5 | 0.6 | 0.5 | 0.5 | 0.6 |
| Overall wo CL E | 0.5 | 0.2 | 0.4 | 0.3 | 0.2 | 0.2 |
| Enc Time[%] | 102% |  |  | 103% |  |  |

# References

[1] G. Laroche, C. Gisquet, P. Onno, E. François, N. Ouedraogo, J. Ricard, “Robust solution for the AMVP parsing issue”, JCTVC-E219, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[2] Y.-W. Huang, B. Bross, M. Zhou, W.-J. Chien, I.-K. Kim, “Description of Core Experiment 9: MV Coding and Skip/Merge Operations”, JCTVC-E709, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[3] M. Zhou, “A study on HM2.0 bitstream parsing and error resiliency issue”, JCTVC-E118, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[5] T. Hellman, Y. Yu, “Decoder Performance Restrictions due to Merge/MVP Index Parsing”, JCTVC-F341, 6th Meeting: Torino, IT, 14-22 July, 2011.

[5] F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

## Syntax modification

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| use_collocated | u(1) |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } else |  |
| if( entropy_coding_mode_flag && slice_type != I) | u(3) |
| cabac_init_idc | u(2) |
| } |  |
|  | ue(v) |

### Slice header semantics

use_collocated: specifies if the collocated motion vector predictors are used for the decoding of the current slice.

### Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag ) |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

[Ed: (BB) NumMergeCand needs to be defined. Decoding processes of neighboring blocks have to be finished to derive NumMergeCand. Same applies for NumMVPCand (TK)]

NumMVPCand() is defined by just checking the availability of the 3 candidates for the Inter mode.

NumMergeCand is defined by just checking the availability of the 5 candidates for the Merge mode.

## Decoding process modification

##### Derivation process for luma motion vectors for merge mode

Figure 8-3 – Spatial  and temporal neighbours that can be used as merging candidates (informative) illustrates the position of the spatial neighbours (SA, SB, SC, SD) and temporal predictors (TA, TB, TC and TD) relative to the current prediction unit.

This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N and the prediction list utilization flags predFlagL0N and predFlagL1N with N being replaced by A, B, C or D.

- The derivation process of reference indices for temporal merging candidate in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), nPSW, nPSH as the inputs and the output is directly assigned to refIdxLX.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col || availableFlagL1Col	(9-1)

predFlagLXCol = availableFlagLXCol	(9-2)

- The merging candidate list, mergeCandList, is constructed of which elements are given by the following order:

  - Left candidate

    - SA (spatial, left), if availableFlagA is equal to 1

  - Above candidate:

    - SB (spatial, above), if availableFlagB is equal to 1

    - TB (temporal, above), if availableFlagTB is equal to 1

  - Temporal candidate

    - TCol, if availableFlagTCol is equal to 1

  - Corner 1 candidate

    - SC (spatial above right), if availableFlagSC is equal to 1

    - TC (temporal, above right), if availableFlagTC is equal to 1

  - Corner 2 candidate

    - SD (spatial, below left), if availableFlagSD is equal to 1

    - TD (temporal, below left), if availableFlagTD is equal to 1

During the construction of the mergeCandList (from step 2 to step 5 above), if one candidate as the same motion vectors as one element in the list, this candidate is modified with the following method until it is different:

- mvLXN[0] = (mvLXN[0]>>1)<<1 and  mvLXN[1] = (mvLXN[1]>>1)<<1;

- mvLXN[0] = (mvLXN[0]>>2)<<2 and mvLXN[1] = (mvLXN[1]>>2)<<2;

- mvLX[N0] = mvLXN[0]  +  8;

- mvLXN[1] = mvLXN[1]  -  8;

- mvLXN[0] = mvLXN[0]  +  8;

- mvLXN[1] = mvLXN[1]  -  8;

- If several merging candidates have the same motion vectors and the same reference indices, the merging candidates are removed from the list except the merging candidate which has the smallest order in the mergeCandList.

- If the number of elements NumMergeCand within the mergeCandList is equal to 1, mergeIdx is set equal to 0, otherwise, mergeIdx is set equal to merge_idx[ xP][ yP ].

- The following assignments are made with N being the candidate at position mergeIdx in the merging candidate list mergeCandList ( N = mergeCandList[ mergeIdx ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ] 	(9-3)

mvLX[ 1 ] = mvLXN[ 1 ]	(9-4)

refIdxLX = refIdxLXN	(9-5)

predFlagLX = predFlagLXN	(9-6)

- If all availability flags availableFlagN (with N being replaced by SA, TA, SB, TB, TCol, SC, TC,  SD or TD) are equal to 0, mergeIdx is set equal to 0 and the variables mvLX, refIdxLX and predFlagLX (with X being replaced by 0 or 1) are inferred as follows.

If slice_type is equal to P, the following applies.

mvLX[ 0 ] = 0  (9-7)

mvLX[ 1 ] = 0	(9-8)

refIdxL0 = 0		(9-9)

refIdxL1 = -1	(9-10)

predFlagL0 = 1	(9-11)

prefFlagL1 = 0	(9-12)

Otherwise ( slice_type is equal to B ), the following applies.

mvLX[ 0 ] = 0	(9-13)

mvLX[ 1 ] = 0	(9-14)

refIdxL0 = 0		(9-15)

refIdxL1 = 0		(9-16)

predFlagL0 = 1	(9-17)

prefFlagL1 = 1	(9-18)

##### Derivation process for motion vector predictor candidates (Inter)

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A,or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-4 – Spatial motion vector neighbours

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0 .. 1, specifies sample locations with xAk = xP – 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- When the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and RefPicOrderCnt( currPic, refIdxLY[ xAk ][ yAk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] and ListA is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, the following applies.

- If RefPicOrderCnt( currPic , refIdxA, ListA ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXA is set equal to mvLXA

- Otherwise, mvLXA is derived as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-19)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-20)

mvLXA = ClipMv( ( DistScaleFactor * mvLXA + 128 ) >> 8 )	(8-21)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxA, ListA ) )	(8-22)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-23)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0 MinPuSize , xB2 = xP - MinPuSize and yBk = yP – 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- When the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX and mvLX[ xBk ][ yBk ] is not identical to mvLXA, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- When availableFlagLXB is equal to 0, for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ](with Y = !X) is equal to 1 and RefPicOrderCnt( currPic , refIdxLY[ xBk ][ yBk ], LY) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ] and ListB is set equal to LY.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- If availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is equal to RefPicOrderCnt( currPic, refIdxLX, LX ), mvLXB is set equal to mvLXB.

- Otherwise if availableFlagLXB is equal to 1 and RefPicOrderCnt( currPic , refIdxB, ListB ) is not equal to RefPicOrderCnt( currPic, refIdxLX, LX ),, mvLXB is derived as specified below.

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-24)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-25)

mvLXB = ClipMv( ( DistScaleFactor * mvLXB + 128 ) >> 8 )	(8-26)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic , refIdxB, ListB ) )	(8-27)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-28)

- When availableFlagLXB is equal to 1 and mvLXB is identical to mvLXA, availableFlagLXB is set equal to 0.

5.	When mvLXB is equal to mvLXA and availableFlagLXA is equal to 1, mvLXB is modified according to the following order until the motion vector mvLXB is different from mvLXA

  - mvLXB[0] = (mvLXA[0]>>1)<<1 and  mvLXB[1] = (mvLXA[1]>>1)<<1;

  - mvLXB[0] = (mvLXA[0]>>2)<<2 and mvLXB[1] = (mvLXA[1]>>2)<<2;

  - mvLXB[0] = mvLXA[0]  +  8;

        - Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXTemp,

- the availability flag availableFlagLXTemp.

The function RefPicOrderCnt( pic, refidx, LX ) is specified by the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of pic with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-exisiting.”

Depending on the values of slice_type and collocated_from_l0_flag, the variable tempPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable tempPic specifies the picture that contains the co-located partition as specified by RefPicList1[ 0 ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable tempPic specifies the picture that contains the co-located partition as specified by RefPicList0[ 0 ].

Variable tempPu and its position ( xPTemp, yPTempCol ) are derived in the following ordered steps:

- Right-bottom luma position of the current prediction unit is defined by

xPRb = xP + nPSW	(8-29)

yPRb = yP + nPSH	(8-30)

- The variable tempPu is set as the prediction unit covering the modified position given by ( ( xPRb >> 4 ) << 4, ( yPRb >> 4 ) << 4 ) inside the tempPic.

- If tempPu is coded in an intra prediction mode or tempPu is unavailable, the following applies.

  - Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1	(8-31)

yPCtr = ( yP + ( nPSH >> 1 ) – 1	(8-32)

  - The variable tempPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the tempPic.

- ( xPTemp, yPTemp ) is set equal to the top-left luma sample of the tempPu relative to the top-left luma sample of the tempPic.

- If tempPu is coded in an intra prediction mode or tempPu is unavailable, the following applies.

  - Below Left  position of the current prediction unit is defined by

xPBl = ( xP - nPSW  )

yPBl = ( yP +  nPSH )

The variable tempPu is set as the prediction unit covering the modified position given by ( ( xPBl >> 4 ) << 4, ( yPBl>> 4 ) << 4 ) inside the tempPic.

- If tempPu is coded in an intra prediction mode or tempPu is unavailable, the following applies.

  - Above right  position of the current prediction unit is defined by

xPAr = ( xP + nPSW  )

yPAr = ( yP -  nPSH )

The variable tempPu is set as the prediction unit covering the modified position given by ( ( xPAr >> 4 ) << 4, ( yPAr >> 4 ) << 4 ) inside the tempPic.

- If tempPu is coded in an intra prediction mode or tempPu is unavailable, the following applies.

  - Above  left position of the current prediction unit is defined by

xPAl = ( xP  - nPSW)

yPAl = ( yP -  nPSH )

The variable tempPu is set as the prediction unit covering the modified position given by ( ( xPAl >> 4 ) << 4, ( yPAl >> 4 ) << 4 ) inside the tempPic.

The variables mvLXTemp and availableFlagLXTemp are derived as follows.

- If tempPu is coded in an intra prediction mode or tempPu is unavailable, both components of mvLXTemp are set equal to 0 and availableFlagLXTemp is set equal to 0.

- Otherwise (tempPu is not coded in an intra prediction mode and tempPu is available), the variables mvTemp and refIdxTemp are derived as follows,

  - If PredFlagL0[ xPTemp ][ yPTemp ] is equal to 0, the motion vector mvTemp and the reference index refIdxTemp are set equal to MvL1[ xPTemp ][ yPTemp ] and RefIdxL1[ xPTemp ][ yPTemp ], respectively.

  - Otherwise (PredFlagL0[ xPTemp ][ yPTemp ] is equal to 1), the following applies.

  - If PredFlagL1[ xPTemp ][ yPTemp ] is equal to 0, the motion vector mvTemp and the reference index refIdxTemp are set equal to MvL0[ xPTemp ][ yPTemp ] and RefIdxL0[ xPTemp ][ yPTemp ], respectively.

  - Otherwise (PredFlagL1[ xPTemp ][ yPTemp ] is equal to 1), the following applies.

    - The following asignments are made with X being 0 or 1.

      - RefIdxTempLX = RefIdxLX[ xPTemp ][ yPTemp ]

      - If PicOrderCnt( TempPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( tempPic, RefIdxTempLX, LX ) is greater than PicOrderCnt( currPic ) or PicOrderCnt( TempPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( colPic, RefIdxTempLX, LX ) is less than PicOrderCnt( currPic ), the variable MvXCross is derived as follows.

        - MvXCross = 1

      - Otherwise (PicOrderCnt( tempPic ) is less than PicOrderCnt( currPic ) and RefPicOrderCnt( tempPic, RefIdxTempLX, LX ) is less than or equal to PicOrderCnt( currPic ) or PicOrderCnt( tempPic ) is greater than PicOrderCnt( currPic ) and RefPicOrderCnt( tempPic, RefIdxTempLX, LX ) is greater than or equal to PicOrderCnt( currPic )), the variable MvXCross is derived as follows.

        - MvXCross = 0

    - If one of the following conditions is true, the motion vector mvTemp, the reference index refIdxTemp and ListTemp are set equal to MvL1[ xPTemp ][ yPTemp ], RefIdxTempL1 and L1, respectively.

      - Mv0Cross is equal to 0 and Mv1Cross is equal to 1

      - Mv0Cross is equal to Mv1Cross and reference index list is equal to L1

    - Otherwise, the motion vector mvTemp, the reference index refIdxTemp and ListTemp are set equal to MvL0[ xPTemp ][ yPTemp ], RefIdxTempL0 and L0, respectively.

and the variable availableFlagLXTemp is set equal to 1 and the following applies.

  - If PicOrderCnt( tempPic ) – RefPicOrderCnt( tempPic, refIdxTemp, ListTemp ) is equal to PicOrderCnt( currPic ) - RefPicOrderCnt( currPic, refIdxLX, LX ),

mvLXTemp = mvTemp	(8-33)

  - Otherwise, mvLXTemp is derived as scaled version of the motion vector mvTemp as specified below

tx = ( 16384 + Abs( td / 2 ) ) / td	(8-34)

DistScaleFactor = Clip3( -1024, 1023, ( tb * tx + 32 ) >> 6 )	(8-35)

mvLXTemp = ClipMv( ( DistScaleFactor * mvTemp + 128 ) >> 8 )	(8-36)

[Ed. (WJ): software has a clip function for scaled motion vector. Do we need this? AVC does not have it]

where td and tb are derived as

td = Clip3( -128, 127, PicOrderCnt( tempPic ) – RefPicOrderCnt( tempPic, refIdxTemp, ListTemp ) )	(8-37)

tb = Clip3( -128, 127, PicOrderCnt( currPic ) – RefPicOrderCnt( currPic, refIdxLX, LX ) )	(8-38)

- When mvLXTemp is equal to mvLXB and availableFlagLXB is equal to 1, mvLXTemp is modified according to the following order until the motion vector mvLXTemp is different from mvLXB

  - mvLXTemp[0] = (mvLXB[0]>>1)<<1 and  mvLXTemp[1] = (mvLXB[1]>>1)<<1;

  - mvLXTemp[0] = (mvLXB[0]>>2)<<2 and mvLXTemp[1] = (mvLXB[1]>>2)<<2;

  - mvLTemp[0] = mvLXB[0]  +  8;

  - mvLXTemp [1] = mvLXB[1]  -  8;